STANDARD MICROSYSTEMS CORPORATION

NDIS driver

For Ethernet LAN Adapters

GENERAL INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide provides the following instructions:

1.  Overview

2.  Configuration

3.  PROTOCOL.INI Parameters

4.  Error Messages

1.  OVERVIEW

------------

The NDIS driver is designed to work with all SMC Ultra and EtherCard

Families and the older WD Ethernet LAN Adapters. It conforms to the

Network Driver Interface Specification (NDIS) version 2.0.1 defined

by Microsoft/3Com. The driver has been tested with Microsoft LAN

Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN Program,

DEC Pathworks and other NDIS compatible network operating systems and

protocol stacks.

SUPPLIED FILES:

NDIS.DOC           Installation Guide (this File)

SMC8000.OS2         OS/2 NDIS MAC driver

SMC8000.DOS         DOS NDIS MAC driver

PROTOCOL.INI       PROTOCOL.INI file sample

SMCDOSMC.NIF       NIF file for Micro Channel adapters

using DOS and IBM LAN Server

SMCOS2MC.NIF       NIF file for Micro Channel adapters

using OS/2 and IBM LAN Server

SMCDOSAT.NIF       NIF file for AT adapters using DOS

and IBM LAN Server

SMCOS2AT.NIF       NIF file for AT adapters using OS/2

and IBM LAN Server

SMCDOSJP.NIF       NIF file for Western Digital EtherCard

PLUS adapters using DOS and IBM LAN

Server

SMCOS2JP.NIF       NIF file for Western Digital EtherCard

PLUS adapters using OS/2 and IBM LAN

Server

SMC.MSG            OS/2 Driver Message File

Install the adapter card in your PC using the instructions in the

Installation Guide included with the adapter.

2.  Configuration

------------------

The SMC NDIS drivers may be used to install one or two adapter

cards. When installing a LAN software, select either one or two SMC

Adapters. When the LAN setup exits, examine the CONFIG.SYS

file and the PROTOCOL.INI file.

1. The CONFIG.SYS file should have only one DEVICE= line that

contains the SMC8000.XXX driver for DOS/OS2.

For example:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMC8000\SMC8000.DOS

or

DEVICE=C:\LSP\SMC8000.DOS

or

DEVICE=C:\IBMCOM\MACS\SMC8000.OS2

2. The PROTOCOL.INI file should specify one or two SMC8000$ entries.

a. For the first adapter

[SMC8000_NIF]

DriverName = SMC8000$

IOBase = 0x280

For the second adapter

[SMC80002_NIF]

DriverName = SMC8000$

IOBase = 0x300

Make sure the IOBase value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include an IOBase

statement with the DriverName.

b. For Micro Channel adapters, for the first adapter

[SMC8000_NIF]

DriverName = SMC8000$

SlotNumber = 3

For the second adapter

[SMC8000_NIF]

DriverName = SMC8000$

SlotNumber = 4

Make sure the SlotNumber value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include a SlotNumber

statement with the DriverName.

c. A sample entry for an older WD Ethernet adapter which

is not software configurable

[SMC8000_NIF]

DriverName = SMC8000$

IOBase = 0x300

RAMAddress = 0xCC00

IRQ = 5

3. Modify adapter values so that the two adapters do not use

the same values for I/O Base Address, RAM Address and IRQ.

Refer to the adapter's Installation Guide for information

about these parameters.

3.  PROTOCOL.INI PARAMETERS

---------------------------

Several parameters are typically specified in the SMC8000 section of

the PROTOCOL.INI file. The only parameter that must be specified

is the 'DriverName'. All others have default values supplied

by the driver and can be left out of the PROTOCOL.INI file if the

installed adapter matches the default configuration. Software-

configurable and Micro Channel adapters ignore the values of IRQ,

IOBase and RAMAddress and instead use the values in the POS

registers set up when the card was installed.

Below is a list of parameters and their definitions commonly found

in the PROTOCOL.INI file.  Both default values and possible value

ranges are provided for each parameter.

IOBase       Specifies the base I/O address used by the adapter.

The default is 0x280. This parameter is required for

all ISA adapters.

SlotNumber   Specifies the number of the slot containing the

adapter card in a Micro Channel machine.  This

parameter is a decimal number in the range of 1 - 10

and is required for all Micro Channel adapters.

NetAddress   Overrides the factory set network address of the

network adapter.  The value of this parameter is a

hexadecimal string of 12 digits between double

quatation marks.  The address must be unique among

all network addresses on the network.

CLOAK        will enable cloaking. THIS MUST BE ADDED to the

protocol.ini in order for cloaking to work.

ALLOWRIPL    enables the driver to be RIPL loaded in an OS/2

environment.

The following parameters are required for WD adapters which are not

software configurable

IRQ     Specifies the interrupt used by the adapter.  The

default is 3,

RAMAddress   Specifies the Shared RAM segment address used by the

adapter.  The default is 0xD000.

4. ERROR MESSAGES

--------------

Could not find an adapter [at slot ___] [at I/O base address ___].

The driver could not find an adapter at the specified

location.

Cannot access adapter RAM properly.

The memory test for adapter memory failed.

Adapter hardware failure detected (self-test).

There was an unidentified failure during adapter

initialization.

Adapter hardware failure detected during initialization.

The hardware did not respond correctly to initialization

commands.

Other system RAM already occupies adapter RAM address space.

There is a ram conflict with the adapter fram.

Other system ROM already occupies adapter RAM address space.

There is a rom conflict with adapter ram.

Adapter unable to insert into network.

The adapter was unable to re-initialize and re-open the

adapter.

RAM base address chosen for driver does not match adapter

configuration.

The ram base address must be specified by EZSTART.

The definition in the configuration file must be removed.

IRQ chosen for driver does not match adapter configuration.

The irq must be specified by EZSTART.

The definition in the configuration file must be removed.

RAM size chosen for driver does not match adapter configuration.

The ram size is stored on the adapter and is fixed in size.

The definition in the configuration file must be removed.

Adapter IRQ higher than 7 not valid in 8-bit slot.

Adapter RAM base address on illegal boundary.

Adapter ram must start on 200h boundaries.

I/O base address chosen for driver not valid.

The adapter i/o base must be between 200h-ec0h on a 20h

boundary.

RAM base address chosen for driver not valid (out of range).

The ram base must be between a000h-c000h

IRQ chosen for driver not valid (out of range).

IRQ values are between 2-15.

RAM size chosen for driver not valid (out of range).

Ram size must be either 200h (8k) or 400h (16k).

Adapter configuration information corrupted; run software setup

program.

The onboard EEROM contents have been corrupted. Please

run EZSTART to verify and correct.

EtherCard Ultra, EtherCard PLUS, TokenCard, and SuperDisk are

trademarks, and SMC and Standard Microsystems are registered

trademarks of Standard Microsystems Corporation. Other Trademarks

mentioned herein belong to their respective companies.#

#

#