HP 4020I Sure Store CD-ROM

Windows-NT

The 4020I Cd writter manufactured by HP WILL NOT Function Properly Under Windows-NT with the Advansys Host Adapter Card.

Windows NT can not operate the Drive itself since it is not a standard Device. It is a combination of a Type 5 Device(cd) & a Type 4 Device( worm). The drive returns a mixture of the two types of information back when executing the cd rom driver.

The following is a statement of what is possible with the 4020I CD under Windows-NT.

- With 1.20 and older Microcode on the drive, The drive will not properly read any CD's

- With Microcode Version 1.25 the drive will read a standard Single session CD and will only read the last session of a multisession CD.

- Will write a CD using EZ-CD provided by HP.

- These are the only tests that have been done with the 1.25 Microcode. The fact that the Multisession CD's can not be read is still a problem when all sessions placed on the disk need to be available for reading. Loss of previous sessions on CD's as you progress through sessions is a definate outcome.

- The file WINNT.EXE is our current Windows NT drivers. This file needs to be executed from a hard disk in the following manner.

WINNT A: <CR>

This will cause the file to extract all files onto a floppy placed in the A:                      		drive. This will create files for  Windows-NT  Operating System

- Run Windows NT SETUP from the Windows NT MAIN program Group.

- Click OPTIONS from pull down Menus

- Choose ADD/REMOVE SCSI Adatpers

- click ADD

- Select other From Menu

- Insert the floppy from above in drive A: and type A:\

- Select the Proper Driver

- Exit Windows NT setup and Reboot your system for the driver to take effect.

- Support for any additional programs is up to the authors of said programs as we have done no testing of any product on this device.

- We have had reports that the newest Firmware 1.26 for the drive operates fine but that some software programs require a program called WNASPI32.DLL and ASPI32.SYS.  WNASPI32.DLL should be in your WINNT\SYSTEM32 directory.  ASPI32.SYS should be in your WINNT\SYSTEM32\DRIVERS directory.   Please be advised that this is a program that should be provided by the vendor of the software, as it is not provided by Microsoft or us.

Tech Support