Documentation for VgaSpeed VGA Graphics Benchmark Program

By:       J. Rockford Cogar

119 Oklahoma Avenue

Oak Ridge, TN 37830

Use:      VgaSpeed is FreeWare. This program may be used any way you wish,

as long as you do not charge any money for the program itself.

I. Introduction.

The program VgaSpeed is a VGA Graphics Benchmark Program. What VgaSpeed

measures is how fast a given computer system can generate VGA graphics. There

are several important hardware characteristics that determine how fast a

computer can do graphics operations:

1. CPU Speed

2. Expansion Bus Speed

3. Number of Video Wait States

4. Speed of Internal VGA Card Operations

An example of a fast graphics machine would include the following: a 33 mhz

80486 CPU, a 10 mhz bus speed, very few video wait states and a 'fast' VGA

card.

Because graphics operations on VGA cards are mostly 8 bit I/O operations, 16

bit bus connectors on VGA cards do not appear to greatly improve the speed of

graphics.

The results of VgaSpeed are given as relative to the graphics performance of

an IBM PS/2 Model 60 VGA system as a VS (VgaSpeed) number.

A VS of 2.00, would show that the current system has the ability to do VGA

graphics twice as fast as an IBM PS/2 Model 60 VGA. The fastest 80486 system

that has been measured gave a VS of about 2.70. Typical 80386SX VGA systems

yield VS numbers of about 1.40. It is around VS numbers equal to 1.40 that

Microsoft Windows 3.0 starts to look fairly good.

II. The Tests.

VgaSpeed is made up of four different graphics tests: Write Mode Zero Dot

Test, Write Mode Two Dot Test, Bit Map Move Test and Bit Map Text Test.

The graphics code in VgaSpeed was written in assembly language from

documentation in the IBM EGA and VGA Technical Reference Guides. Every effort

was made to make the code as fast as possible. After all, the goal is too

find out how fast a given system can do VGA graphics, not how fast the the

system can run a bunch of code through the CPU.

VgaSpeed was written using Borland Turbo Assembler 2.0 and Borland Turbo

'C++' 1.0.

By default, all tests are executed while the BIOS video mode was '16'. This

is the same as EGA 640 pixels horizontal by 350 pixels vertical 16 color

mode. This allows VgaSpeed to be run on EGA video systems.

When VgaSpeed is run in VGA 640 x 480, 16 color mode (by using the 'v'

command line switch) the VS number will be almost exactly the same as a test

of EGA  640 x  360, 16  color mode.  The graphics  testing takes  about 40

seconds on a fast 80486 system to 140 seconds on an 8mhz AT system.

1. Write Mode Zero Dot Test.

This test draws 817,452 dots (pixels) on the graphics screen using

hardware write mode zero. Hardware write mode zero is the write mode used

by most graphics programs. It appears that Microsoft Windows 3.0 uses

write mode for all in all EGA and VGA type driver programs.

2. Write Mode Two Dot Test.

This test draws 1,634,904 dots (pixels) on the graphics screen using

hardware write mode two. In general, the process of writing a pixel to the

graphics screen in write mode two is twice as fast as one written using

write mode zero. Hardware write mode two is the mode used by most very

high performance graphics programs. For example, all of the EGA/VGA

graphics based data acquistion software from Tennelec/Nucleus Inc. (Oak

Ridge TN), use hardware write mode two. Some of the faster commercial CAD

packages also appear to use hardware write mode two.

3. Bit Map Move Test.

This test does 504 bitmap block moves. The size of the bitmap is 30,102

pixels (a 173 by 174 rectangle). The pattern leftover from the write mode

two dot test is copied from video RAM to system RAM, then from system RAM

to a different location in video RAM. This process is repeated 252 times.

The hardware write mode here is 'zero'. It is likely that most all

programs that use bit map moves to/from video RAM to/from system RAM use

hardware write mode zero to do so.

Because this operation involves memory block moves (using the: REP MOVSW

operation), the number of memory wait states of the source and target

memory can be a major factor in the speed of this test. In addition,

computers with expansion bus speeds greater than the standard 8 mhz, do

well on this test.

4. Bit Map Text Test.

This test shows how fast text can be displayed on a graphics mode screen.

The Bit Map Text Test writes a 39 character string in an 8 pixel tall font

to the graphics screen 2048 times. In addition, a 40 character string is

written to the screen in an 14 pixel tall font 2048 times. Both fonts are

8 pixels wide. All this means that 14,286,848 pixels get written on the

screen in this test! The hardware write mode used in this here is 'two'.

II. System Requirements.

1. A color VGA or EGA video system with at least 128KB of video RAM.

2. DOS version 2.0 or greater.

3. 64KB of free RAM.