Date: 3rd Mar. 1996.

Copy the following files to the distribution diskette.

EPSA2.SYS      -> EPSA2 WINDOWS NT DRIVER.

OEMSETUP.INF   -> Required by Windows NT setup

TXTSETUP.OEM   ->  ,,

DISK1          ->  ,,

Run Winnt Setup, Choose options and select SCSI Adapter Installation.

Choose 'Add' and select 'Others' from the adapter menu.

Now, WINNT prompts you for the path where the information resides,

Type the target (A: or the directory) and proceed from Now on as per

instructions.

After completion, an entry 'EPSA2X' will be created in the registry.

Adding command line switches to EPSA2 driver:

1. The command line switches are similar to the DOS  and are listed as

follows:

'/rx' - x ranges from 0 to 10

'/wx' - x ranges from 0 to 3

'/de' - disables EPP checking

'/di' - disables interrupt operation

'/fe' - forces EPP.

2. To add parameters to EPSA2X, run REGEDT32 and choose EPSA2X.

Create a subkey 'Parameters'. Choose 'Parameters' key and create a subkey

'Device0' and choose it. Under 'Device0' create a value 'DriverParameter'

whose type is REG_SZ. The values can be the command line string

for eg.

" /de /r8 /w0"

Note: After installing EPSA2 driver, the system should be rebooted.

This driver has been successfully tested with

1) SCSI Harddisks (Quantum and Conner)

2) CD-ROM

3) Archive Qic and Data tape drives