Sun PCNFS Installation

-----------------------

NE2000 Compatible EtherPair/EtherCoax Family LAN Adapter Drivers

For Use in Sun PCNFS Environment

1. INSTALLATION INSTRUCTIONS

a. Add Device=c:\dos\ansi.sys command at CONFIG.SYS, Reboot PC.

b. Insert the PCNFS Installation  diskette into the diskette drive (A: or B:)

and type: install. Press Enter.

c. Enter the destination "Drive letter:" (usually C:). Press Enter.

d. Select "Ethernet" and press Enter.

e. Select the "NDIS" option of "Ethernet Adapter" menu

f. Follow all the prompts that appear.

g. Specify the PC name, IP address, server name, and server

server IP address (Contact System Administrator).

h. Follow all the prompts to finish the installation.

i. The system create \LANMAN and \NFS directory on the destination drive.

j. After you are finished, copy the ETHNE.DOS file from the

subdirectory \MSLANMAN.DOS\DRIVERS\ETHNE\ACCNE on your

driver program diskette to the \LANMAN (C: or D:)

directory on your destination driver.

k. Using a text editor, edit the CONFIG.SYS. Add this

line above the NFS_NDIS driver specification:

DEVICE=C:\LANMAN\ETHNE.DOS

l. Edit the PROTOCOL.INI file at \LANMAN directory.

1. Replace [your-mac-module] with [ETHNE]

2. Replace drivername = YOURMAC$ with drivername = ETHNE$

Specify the correct IRQ and IOBASE value (May be omitted)

for this adapter depending on whether the adapter's set

IRQ = 3

IOBASE = 0x300

...

3. Replace bindings = YOUR-MAC-MODULE with bindings = ETHNE$

m. Reboot the computer.

n. Exec NFSCONF.EXE to configuration system (i.e Mount a virtual driver E:)

Exec TELNET.EXE to do terminal emulation.

Sample configuration files:

2. CONFIG.SYS

shell=c:\command.com /P /E:526

dos=high, UMB

rem ----pcnfs install

Device=c:\dos\ansi.sys

DEVICE=C:\NFS\PCNFS.SYS  /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\LANMAN\PROTMAN.SYS /i:C:\LANMAN --> specify PROTOCOL.INI directory.

device=C:\LANMAN\ETHNE.DOS  ----------->  NDIS driver name

DEVICE=C:\LANMAN\NFS-NDIS.SYS

FILES=50

BUFFERS=50

LASTDRIVE=v

STACKS=9,256

3. PROTOCOL.INI at \LANMAN directory

[protocol manager]

drivername = PROTMAN$

[ETHNE]

drivername = ethne$

irq = 3

iobase = 0x300

[NFS-NDIS]

drivername = NFSLINK1

bindings	 = ETHNE

4. AUTOEXEC.BAT

...

SET TZ=PST8

SET path=C:\;C:\NFS;C:\LANMAN

SET NFSDRIVE=D

C:lanman\netbind	 ----> Add this line to Binding NDIS driver

PRT *

NFSRUN