PC   ERROR  CODE

x00, xx00, xxx00 = no error

adsc	address status change

ai 	analog input

ao	analog output

apt	address pass through

artic	real time interface coprocessor

bi	binary input

bo 	binary output

ce	convert enable

cmos 	complementary metal oxide semiconductor

co	command output,convert out

cpt	command pass through

crc	cyclic redundancy check

cts	clear to send

dac	dual asynchronous communication

dcl	device clear

dec 	device clear

det 	device execute trigger

di	data in

dma	direct memory access

dsea	display station emulation adapter

dsr	data set ready

do	data out

ecc	error checking & correction

eoi	end of identify

eos	end of string

eot	end of transmission

err	error

esdi	enhanced small device interface

fifo	first in first out

get 	group execute trigger

gpib 	general purpose interface bus

h/v	horizontal/vertical

icl	interface clear

ieee	institute of electrical & electronic engineer

ipl	initial program load

irq	interrupt request

is	interrupt status

isa	industry standard architecture

lan	local area network

lok	lockout

lokc	lockout changed

lut	look up table

mca	micro channel architecture

midi	musical instrument digital interface

nec	nippon electronic co (drive controller)

pgc	professional graphic controller

rag/cag	row address generator/column address generator

rem	remote

remc	remote changed

sdc	selected device clear

sdlc	synchronous data link control

sip	single inline package

spmr	serial poll mode register

sr	shift register

srq	service request

srqi	service request in

stb	strobe

vca	voice control adapter

================

01xx	SYSTEM BOARD

101	interrupt failure

102	bios rom checksum error (pc,xt),timer (at,mca)

103	basic rom checksum error (pc,xt), timer interrupt (at,mca)

104	interrupt controller (pc,xt);protected mode (at,mca)

105	timer (pc,xt), keyboard controller (mca)

106	system board

107	system board adapter card or math coproc, hot nmi test (mca)

108	system board, timer bus (mca)

109	dma test, memory select (mca)

110	system board memory (isa); system obard parity check (mca)

111	adapter memory (isa);memory adapter parity check (mca)

112*	adapter, watchdog time-out (mca)

113*	adapter, dma arbitration time-out (mca)

* = if seen during advanced diag, replace device being tested when error

code appeared on screen.

115	80386 protect mode

121	unexpected hardware interrupt

131	cassette wrap test (pc)

132	dma extended registers

133	dma verify logic

134	dma arbitration logic

151	real-time clock (or cmos ram)

152	system board (isa); real time clock or cmos (mca)

160	system board id no recognized (mca)

161	system option (dead battery) (cmos chip lost power)

162	system options (run setup) (cmos checksum or crc error)

163	time & date (run setup) (clock not updating)

164	memory size (run setup);(cmos does not match system)

165	adapter id mismatch (mca)

166	adapter time-out;card busy (mca)

167	system clock not updating (mca)

199	user - indicated device list not correct

02xx	MEMORY (SEE ALSO 110,111,164)

201	memory error

202	memory address line 0-15

203	memory address line 16-23;line 16-31 (mca)204 relocated memory (psw2)

205	error 1st 128k (ps2 isa);cmos (ps2 mca)

207 	rom failure

211	system board memory;system board 64k (mca)

215	memory address error;64k on daughter/sip 2 (70)

216	system board memory;64k on daughter/sip 1 (70)

221	rom to ram copy (mca)

225	wrong speed memory on system board (mca)

03xx	KEYBOARD

301	keyboard no respond

302	user-indicated error from keyboard test (pc,xt)

302	keyboard locked (at, ps2 model 25,30)

303	keyboard/system board interface

304	keyboard or system unit error; keyboard clock (mca)

305	keyboard fuse on system obard (50,60,80);++5v error (70)

341	keyboard

342	keyboard cable

343	enhancement card or cable

365	keyboard (replace it)

366	interface cable (replace cable)

367	enhancement card or cable (replace)

04xx	MONOCHROME DISPLAY

401	memory, horizontal sync frequency or vertical sync test

408	user-indicated display attributes

416	user-indicated character set

424	user-indicated 80x25 mode

432	monochrome card parallel port test

05XX	COLOR/GRAPHIC DISPLAY

501	memory, horizontal sync frequency or vertical sync test

508	user-indicated display attributes

516	user-indicated character set

524	user-indicated 80x25 mode

532	user-indicated 40x25 mode

540	user-indicated 320x200 graphic mode

548	user-indicated 640x200 graphic mode

556	light pen test

564	user-indicated screen paging test

06xx	DISKETTE DRIVES &/OR ADAPTER

601	disk/adpater test fail, drive or controller (mca)

602	disk test (pc,xt); disk boot record (mca)

603	disk size error

606	disk verify function

607	write protect disk

608	bad command;disk status returned

610	disk initialization (pc,xt)

611	timeout disk status returned

612	bad nec; disk status returned

613	bad dma disk status returned

614	dma boundary error

621	bad seek; disk status returned

622	bad crc;disk status returned

623	record not found; disk status returned

624	bad address mark; disk status returned

625	bad nec seek; disk status returned

626	disk data compare error

627	disk change line error

628	disk removed

630	drive a  index stuck high

631	drive a index stuck low

632	drive a track 0 stuck off

633	drive a track 0 stuck on

640	drive b index stuck high

641	drive b index stuck low

642	drive b track 0 stuck off

643	drive b track 0 stuck on

650	drive speed

651	format fail

652	verify fail

653	read fail

654	write fail

655	controller

656	drive

657	write protect stuck protect

658	change line stuck changed

659	write protect stuck unprotected

660	change line stuck unchanged

07xx	MATH COPROCESSOR

702	exception errors test

703	rounding test

704	arithmetic test1

705	arithmetic test2

706	arithmetic test3

707	combination test

708	integer store test

709	equivalent expressions

710	exceptions

711	save state

712	protected mode test

713	voltage/temperature sensitivity test

09xx	PARALLEL PRINTER ADAPTER

901	data register latch

902	control register latch

903	register address decode

904	address decode

910	status line wrap connector

911	status line bit 8 wrap

912	status line bit 7 wrap

913	status line bit 6 wrap

914	status line bit 5 wrap

915	status line bit 4 wrap

916	interrupt wrap

917	unexpected interrupt

92x	featyre register

10xx	ALTERNATE PRINTER ADAPTER

10xx	adapter test fail

1002	jumpers (model 25,30)

11xx	COMMUNICATIONS DEVICES ASYNCHRONOUS COMMUNICATIONS ADAPTER

SYSTEM BOARD ASYNCHRONOUS PORT (MCA) 16550 INTERNAL MODEM (PS2)

1101	adapter test fail

1102	card selected feedback

1103	port 102 register test

1106	serial option

1107	communication cable or system board (mca)

1108	irq 3

1109	irq 4

1110	modem status register not clear 16550 chip register

1111	ring indicated 16550 control line internal wrap test

1112	trailing edge ring indicate 16550 control line external wrap test

1113	receive & delta receive line signal detect 16550 transmit

1114	receive line signal detect 16550 receive

1115	delta receive line signal detect 16550 transmit & receive data unequal

1116	line control register; all bits cannot be set 16550 interrupt function

1117	line control register;all bits cannot be reset 16550 baud rate test

1118	transmit holding &/or shift register stuck on 16550 interrupt-driven

receive external data wrap test

1119	data ready stuck on 16550 fifo

1120	interrupt enable register; all bits cannot be set

1121	interrupt enable register;all bits cannot be reset

1122	interrupt pending stuck on

1123	interrupt id register stuck on

1124	modem control register; all bits cannot be set

1125	modem control register;all bits cannot be reset

1126	modem status register; all bits cannot be set

1127	modem status register; all bits cannot be reset

1128	interrupt id

1129	cannot force overrun error

1130	no modem status interrupt

1131	invalid interrupt pending

1132	no data ready

1133	no data available interrupt

1134	no transmit holding interrrupt

1135	no interrupt

1136	no received line status interrupt

1137	no receive data available

1138	transmit holding register not empty

1139	no modem status interrupt

1140	transmit holding register not empty

1141	no interrupts

1142	no irq4 interrupts

1143	no irq3 interrupts

1144	no data transferred

1145	maximum baud rate

1146	minimum baud rate

1148	timeout error

1149	invalid data returned

1150	modem status register error

1151	no dsr and delta dsr

1152	no dsr

1153	no delta dsr

1154	modem status register not clear

1155	no cts and delta cts

1156	no cts

1157	no delta cts

12xx	ALTERNATE COMMUNICATIONS DEVICES ASYNCHRONOUS COMMUNICATIONS ADAPTER

(ISA) DUAL ASYNCHRONOUS COMMUNICATIONS (DAC) ADAPTER (MCA) 16550

INTERNAL MODEM

12xx	same as 1100-1157 isa systems, except for ps2 codes listed below

1202		jumpers (model 25,30)

1202 or 06	any serial device but most likely dual async adapter

1208 or 09	any serial device but most likely dual async adpater

1212		dual async adpater or system board

1218 or 19	dual async adpater or system board

1227		dual async adpater or system board

1233 or 34	dual async adpater or system board

13xx	GAME CONTROL ADAPTER

1301	adapter fail

1302	joystick test

14xx	COLOR/GRAPHICS PRINTER

1401	printer test fail

1402	not ready;out of paper

1403	no paper;interrupt fail

1404	matrix printer test fail;system board time-out

1405	parallel adapter

1406	presence test

15xx	SDLC (SNCHRONOUS DATE LINE CONTROL) COMMUNICATION ADAPTER

1501	adapter test fail

1510	8255 port b

1511	8255 port a

1512	8255 port c

1513	8253 timer #1 did not reach terminal count

1514	8253 timer #1 output stuck on

1515	8253 timer #0 did not reach terminal count

1516	8253 timer #0 output stuck on

1517	8253 timer #2 did not reach terminal count

1518	8253 timer #2 output stuck on

1519	8273 port b error

1520	8273 port a error

1521	8273 command/read time-out

1522	interrupt level 4 (timer & modem change)

1523	ring indicator stuck on

1524	received clock stuck

1525	transmit clock stuck on

1526	test indicate stuck on

1527	ring indicate not on

1528	receive clock not on

1529	transmit clock not on

1530	test indicate not on

1531	data set ready not on

1532	carrier detect not on

1533	clear-to-send not on

1534	data set ready stuck on

1535	carrier detect stuck on

1536	clear-to-send stuck on

1537	level 3 (transmit/receive) interrupt

1538	receive interrupt results error

1539	wrap data miscompare error

1540	dma channel 1 transmit error

1541	dma channel 1 receive error

1542	8273 error checking or status reporting error

1547	level 4 stray interrupt

1548	level 3 stray interrupt

1549	interrupt presentation sequence time out

16xx	DSEA (DISPLAY STATION EMULATION ADAPTER) IN CASE OF 16XX ERROR

TRY REMOVING NON-IBM ADAPTER & THEN REPEAT THE TEST

1604 or 08	dsea or system twin-axial network problem

1624 or 34	dsea

1644 or 52	dsea

1654 or 58	dsea

1662		interrupt level switches set wrong or defective dsea

1668		same as 1662

1669 or 74	if with earlier version of diagnostics disk, replace with

version 3.0 or later

1674		station address switches set wrong or defective dsea

1684 or 88	feature not installed, device address switches set wrong

or dsea

17xx	FIXED (HARD) DISK/ADAPTER

1701	drive not ready (pc,xt); fixed disk/adapter test (at,ps2)

1702	tiem-out (pc,xt);fixed disk/adapter (at,ps2)

1703	drive (PC,XT,PS2)

1704	controller (pc,xt);adapter or drive error (at,ps2)

1705	no record found

1706	write fault

1707	track 0 error

1708	head select error

1709	bad edd (at)

1710	read buffer overrun

1711	bad address mark

1712	bad address mark (pc,xt);error of undetermined cause (at)

1713	data compare error

1714	drive not ready

1730-1732	adapter

1750	drive verify

1751	drive read

1752	drive write

1753	random read test

1754	drive seek test

1755	controller

1756	controller ecc test

1757	controller head select

1780	hard disk drive c fatal;time out

1781	hard disk drive d fatal;time out

1782	hard disk controller (no ipl from hardfile)

1790	drive c nonfatal error (can attempt to run ipl from drive)

1791	drive d non fatal error (can attempt to run ipl from drive)

18xx	EXPANSION UNIT (PC,XT ONLY)

1801	expansion unit post error

1810	enable/disable

1811	extender card wrap test failure while disabled

1812	high order address lines fail while disabled

1813	wait state fail while disabled

1814 	enable/disable could not be set on

1815	wait state fail while enable

1816	extender card wrap test fail while enabled

1817	high order address lines fail while enabled

1818	disable not functioning

1819	wait request switch not set correctly

1820	receiver card wrap test or an adapter card in expansion unit

1821	receiver high order address lines

20xx	BSC (BISYNCHRONOUS COMMUNICATIONS) ADAPTER

2001	adapter test fail

2010	8255 port a

2011	8255 port b

2012	8255 port c

2013	8253 timer #1 did not reach terminal count

2014	8253 timer #1 output stuck on

2015 	8253 timer #2 did not reach terminal count

2016	8253 timer #2 output stuck on

2017	8251 data set ready fail to come on

2018	8251 clear to send no sensed

2019	8251 data set ready stuck on

2020	8251 clear to send stuck on

2021	8251 hardware reset

2022	8251 software reset command

2023	8251 software error reset command

2024	8251 transmit ready did not come on

2025	8251 receive ready did not come on

2026	8251 could not force overrun error status

2027	interrupt no timer interrupt

2028	interrupt transmit replace card or planar

2029	interrupt transmit replace card only

2030	interrupt transmit replace card or planar

2031	interrupt transmit replace card only

2033	ring indicate stuck on

2034	receive clock stuck on

2035	transmit clock stuck on

2036	test indicate stuck on

2037	ring indicate not on

2038	receive clock not on

2039	transmit clock not on

2040	test indicate not on

2041	data set ready stuck on

2042	carrier detect not on

2043	clear to send not on

2044	data set ready stuck on

2045	carrier detect stuck on

2046	clear to send stuck on

2047	unexpected transmit interrupt

2048	unexpected receive interrupt

2049	transmit data did not equal receive data

2050	8251 detected overrun error

2051	lost data set ready during data wrap

2052	receive time out during data wrap

21xx	ALTERNATE BISYNCHRONOUS COMMUNICATIONS ADAPTER

21xx	same as 2000-2052

22xx	CLUSTER ADAPTER

22xx	adapter test fail

23xx	PLASMA MONITOR ADAPTER

23xx	adapter test fail

24xx	ENHANCED GRAPHICS ADAPTER SYSTEM BOARD VIDEO (MCA)

2401	adapter test fail

2402	monitor if colors change, otherwise system board

2408	user-indicated display attributes

2409	monitor

2410	system board

2416	user indicated character set

2424	user indicated 80x25 mode

2432	user indicated 40x25 mode

2440	user indicated 320x200 graphic mode

2448	user indicated 640x200 graphic mode

2456	light pen test

2464	user indicated screen paging test

25xx	ALTERNATE ENHANCED GRAPHICS ADAPTER

25xx	adapter test fail

26xx	PC/370-M ADAPTER

2601-75		memory card

2677-80		processor card

2681		memory card

2682 & 2694  	processor card

2695		memory card

2697		processor card

27xx	PC/3277 EMULATION ADAPTER

27xx	emulator test fail

28xx	3278/79 EMULATOR 3270 CONNECTION ADAPTER

28xx	adapter test fail

29xx	COLOR/GRAPHICS PRINTER

29xx	printer test fail

30xx	LAN (LOCAL AREA NETWORK) ADAPTER

3001	adapter test fail (rom)

3002	ram

3003	digital loopback

3005	4v or 12v

3006	interrupt conflict

3007	analog

3008	reset command

3015	refer to pc network service manual

3020	replace adapter with jumper w8 enabled

3040	lf translator cable

3041	refer to pc network service manual

30xx	primary pc network adapter

3001	adapter test fail

3002	rom

3003	id

3004	ram

3005	hic

3006	{+/-sign} 12V dc

3007	digital loopback

3008	host detected hic fail

3009	sync fail and no go bit

3010	hic test ok an no go bit

3011	go bit and no cmd 41

3012	card not present

3013	digital fall through

3015	analog

3041	hot carrier on other card

3042	hot carrier on this card

31xx	ALTERNATE LAN ADAPTER

31xx		SAME AS 3000-3041

3115 OR 40	LF translator cable

32xx	PC DISPLAY ADAPTER

32xx	adapter test fail

33xx	COMPACT PRINTER (PC,XT ONLY)

33xx	printer test fail

35xx	ENHANCED DISPLAY STATION EMULATION ADAPTER

3504	adapter connect to twin axial cable during off line test

3508	work station address in use by another work station, or diagnostic

disk from another pc was used

3509	diagnostic prg fail,retry on new disk

3540	work station address invalid;not configured at controller;

twin axial cable fail or not connect;or diagnostic disk from another

pc was used

3588	feature not install or device i/o address switches set wrong

3599	diagnostic prg fail retry on new disk

36xx	IEEE 488 ADAPTER

3601	adapter test fail (base address & read register incorrect following

initialization)

3602	write to spmr

3603	write to adr or ieee 400 adapter addressing problems

3610	adapter cannot be programmed to listen

3611	adapter cannot be programmed to talk

3612	adapter cannot take control with ifc

3613	adapter cannot  goto standby

3614	adapter cannot take control asynchronously

3615	adapter cannot take control asynchronously

3616	adapter cannot pass control

3617	adapter cannot be addressed to listen

3618	adapter cannot be unaddressed to listen

3619	adapter cannot be addressed to talk

3620	adapter cannot be unaddressed to talk

3621	adapter cannot be addressed to listen with extended addressing

3622	adapter cannot be unaddressed to listen with extended addressing

3623	adapter cannot be addressed to talk with extended addressing

3624	dapter cannot be unaddressed to talk with extended addressing

3625	adapter cannot write to self

3626	adapter cannot generate handshake error

3627	adapter cannot detect dcl message

3628	adapter cannot detect sdc message

3629	adapter cannot detect end with eoi

3630	adapter cannot detect eot with eoi

3631	adapter cannot detect end with 0-bit eos

3632	adapter cannot detect end with 7-bit eos

3633	adapter cannot detect get

3634	mode 3 addressing not functioning

3635	adapter cannot recongize undefined command

3636	adapter cannot detect rem,remc,lok or lokc

3637	adapter cannot clear rem or lok

3638	adapter cannot detect srq

3639	adapter cannot conduct serial poll

3640	adapter cannot conduct parallel poll

3650	adapter cannot dma to 7210

3651	data error on dma to 7210

3652	adapter cannot dma from 7210

3653	data error on dma from 7210

3658	uninvoked interrupt received

3659	adapter cannot interrupt on adsc

3660	adapter cannot interrupt on adsc

3661	adapter cannot interrupt on co

3662	adapter cannot interrupt on do

3663	adapter cannot interrupt on di

3664	adapter cannot interrupt on err

3665	adapter cannot interrupt on dec

3666	adapter cannot interrupt on end

3667	adapter cannot interrupt on det

3668	adapter cannot interrupt on apt

3669	adapter cannot interrupt on cpt

3670	adapter cannot interrupt on remc

3671	adapter cannot interrupt lokc

3672	adapter cannot interrupt srqi

3673	adapter cannot interrupt on terminal count on dma to 7210

3674	adapter cannot interrupt on terminal count on dma from 7210

3675	spurious dma terminal count interrupt

3697	illegal dma configuration setting detected

3698	illegal interrupt level configuration setting detected

38xx	DATA ACQUISITION ADAPTER

3801	adapter test fail

3810	timer read test

3811	timer interrupt test

3812	delay BI 13 test

3813	rate BI 13 test

3814	BO 14, isirq test

3815	BO 0, count-in test

3816	BI STB count out test

3817	BO 0, BO CTS test

3818	BO 1,BI 0 test

3819	BO 2, BI 1 test

3820	BO 3, BI 2 test

3821	BO 4, BI 3 test

3822	BO 5, BI 4 test

3823	BO 6, BI 5 test

3824	BO 7, BI 6 test

3825	BO 8, BI 7 test

3826	BO 9, BI 8 test

3827	BO 10,BI 9 test

3828	BO 11,BI 10 test

3829	BO 12,BI 11 test

3830	BO 13,BI 12 test

3831	BO 15,BI 13 test

3832	BO STB, BO GATE test

3833	BI CTS, BI HOLD test

3834	AI CO, BI 15 test

3835	counter interrupt test

3836	counter read test

3837	AO 0 ranges test

3838	AO 1 ranges test

3839	AI 0 values test

3840	AI 1 values test

3841	AI 2 values test

3842	AI 3 values test

3843	analog input interrupt test

3844	AI 23 address or value test

39xx	PROFESSIONAL GRAPHICS CONTROLLER ADAPTER

3901	adapter test fail

3902	rom1 self test

3903	rom2 self test

3904	ram self test

3905	cold start cycle power

3906	data error in communication ram

3907	address error in communication ram

3908	bad data detect while read/write to 6845 like register

3909 bad data detect in lower hex-E0 bytes while read/write 6845-like register

3910	PGC display bank output latches

3911	basic clock

3912	command control error

3913	vertical sync scanner

3914	horizontal sync scanner

3915	intech

3916	LUT address error

3917	LUT red RAM chip error

3918	LUT green RAM chip error

3919	LUT blue RAM chip error

3920	LUT data latch error

3921	horizontal display

3922	vertical display

3923	light pen

3924	unexpected error

3925	emulator addressing error

3926	emulator data latch

3927	base for error codes 3928-3930 (emulator ram)

3928-3930	emulator ram

3931	emulator h/v display problem

3932	emulator cursor position

3933	emulator attribute display problem

3934	emulator cursor display

3935	fundamental emulation ram problem

3936	emulation character set problem

3937	emulation graphic display

3938	emulation character display problem

3939	emulation bank select error

3940	display ram u2

3941	display ram u4

3942	display ram u6

3943	display ram u8

3944	display ram u10

3945	display ram u1

3946	display ram u3

3947	display ram u5

3948	display ram u7

3949	display ram u9

3950	display ram u12

3951	display ram u14

3952	display ram u16

3953	display ram u18

3954	display ram u20

3955	display ram u11

3956	display ram u13

3957	display ram u15

3958	display ram u17

3959	display ram u19

3960	display ram u22

3961	display ram u24

3962	display ram u26

3963	display ram u28

3964	display ram u30

3965	display ram u21

3966	display ram u23

3967	display ram u25

3968	display ram u27

3969 	display ram u29

3970	display ram u32

3971	display ram u34

3972 	display ram u36

3973	display ram u38

3974	display ram u40

3975	display ram u31

3976	display ram u33

3977	display ram u34

3978	display ram u37

3979	display ram u39

3980	pgc ram timing

3981	pgc read/write latch

3982	sr bus output latches

3983	addressing error (vertical column of memory;u2 at top)

3984	addressing error (vertical column of memory;u4 at top)

3985	addressing error (vertical columnn of memory;u6 at top)

3986	addressing error (vertical column of memory;u8 at top)

3987	addressing error (vertical column of memory;u10 at top)

3988	base for error codes 3989-3991 (horizontal bank latch error)

3989-3991	horizontal bank latch errors

3992	rag/cag pgc

3993	multiple write mode, nibble mask errors

3994	row nibble (display ram)

3995	pgc addressing

44xx	5278 DISPLAY ATTACHMENT UNIT & 5279 DISPLAY

44xx	test fail

45xx	IEEE INTERFACE ADAPTER (IEEE 488)

45xx	adapter test fail

46xx	ARTIC MULTIPORT/2 INTERFACE ADAPTER

4611	adapter

4612/13	memory module

4630	adapter

4640/41	memory module

4650	interface cable

48xx	INTERNAL MODEM

48xx	modem test fail

49xx	ALTERNATE INTERNAL MODEM

49xx	modem test fail

56xx	FINANCIAL COMMUNICATION SYSTEM

56xx	system test fail

70xx	CHIP SET (PHOENIX BIOS ONLY)

7000	cmos fail

7001	shadow ram fail (rom not shadowed to ram)

7002	cmos configuration data error

71xx	VOICE COMMUNICATION ADAPTER

7101	adapter test fail

7102	instruction or external data memory

7103	pc to vca interrupt

7104	internal data memory

7105	dma

7106	internal register

7107	interactive shared memory

7108	vca to pc interrupt

7109	dc wrap

7111	external analog wrap & tone output

7114	phone attachment test

73xx	3.5 INCH DISK DRIVE

7301	disk drive/adapter test fail

7306	disk change line error

7307	write protect disk

7308	bad command

7310	track zero error

7311	time out

7312	bad NEC

7313	bad dma

7314	dma boundary error

7315	bad index

7316	speed error

7321	bad seek

7322	bad crc

7323	record not found

7324	bad address mark

7325	bad nec seek

74xx	8514/A DISPLAY ADAPTER/A

74xx	adapter test fail

7426	monitor

744x-7x	8514 memory module

76xx	PAGE PRINTER

7601	adapter test fail

7602	adapter card

7603	printer

7604	printer cable

84xx	PS/2 SPEECH ADAPTER

84xx	adapter test fail

85xx	2MB MEMORY ADAPTER

85xx	adapter fail

850x-1x	80286 expanded memory adapter/a (model 50)

852x	80286 expanded memory adapter/a, memory module (model 50)

86xx	PS/2 POINTING DEVICE (MOUSE)

8601	pointing device;mouse time out (mca)

8602	pointing device;mouse interface (mca)

8603	system board; mouse interrupt (mca)

8604	pointing device or system board

89xx	MIDI ADAPTER

89xx	adapter test fail

100xx	MULTIPROTOCOL COMMUNICATIONS ADAPTER

10002 or 06	any serial device but most likely multiprotocol adapter

10007	multiprotocol adapter or communication cable

1008/09	any serial device, but most likely multiprotocol adapter

10012,10018-19,10042,10056	multiprotocol adapter or system board

101xx	modem & communication adapter/a; system board

10102	card selected feedback

10103	port 102 register test

10106	serial option

10108	irq 3

10109	irq 4

10110	16450	chip register

10111	16450	control line internal wrap test

10113	transmit

10114	receive

10115	transmit & receive data not equal

10116	interrupt function

10117	baud rate test

10118	interrupt driven receive external data wrap test

10125	reset result code

10126	general result code

10127	S registers write/read

10128	echo on/off

10129	enalbe/disable result codes

10130	enable #/word result codes

10133	connect results for 300 baud not received

10134	connect results for 1200 baud not received

10135	local analog loopback 300 baud test

10136	local analog loopback 1200 baud test

10137	no response to escape/reset sequence

10138	S register 13 incorrect parity or # of data bits

10139	S register 15 incorrect bit rate

104xx	ESDI fixed disk or adapter

10450	write/read test

10451	read verify test

10452	seek test

10453	wrong device type indicated

10454	controller fail sector buffer test

10455	controller

10456	controller diagnostic command

10461	format error

10462	controller head select

10463	write/read sector error

10464	drive primary map unreadable

10465	controller ecc 8 bits

10466	controller ecc 9 bits

10467	soft seek error

10468	hard seek error

10469	soft seek error count exceeded

10470	controller attachment diagnostic error

10471	controller wrap mode interface

10472	controller wrap mode drive select

10473	error during esdi read verify test

10480	drive c, ESDI adapter or system board

10481	drive d seek fail,esdi adpater or system board

10482	esdi fixed disk adpater

10483	esdi fixed disk adapter;controller reset;drive select 0

10484	controller head select 3 selected bad

10485	controller head select 2 selected bad

10486	controller head select 1 selected bad

10487	controller head select 0 selected bad

10488	controlled rg-cmd complete 2

10489	controlled wg-cmd complete 1

10490	drive c format;read fail;controller

10491	drive d format;read fail

10499	controller

107xx	5.25 inch external disk drive or adapter;drive or adapter test fail

112xx	SCSI ADAPTER

112xx	adapter test fail

129xx	PROCESSOR CARD FOR MODEL 70  TYPE 3

12901	processor portion of processor board

12902	cache portion of processor board

149xx	PLASMA DISPLAY & ADAPTER

14901 or 14902	system board or plasma display

14922	system board or display adapter

14932	display adapter

165xx 6157 STREAMING TAPE DRIVE OR TAPE ATTACHMENT ADAPTER

165xx	adapter test fail

16520	streaming tape drive

16540	tape attachment adapter

166xx	primary token-ring network pc adapter; adapter test fail

167xx	alternate token ring network pc adapter;adapter test fail

194xx	ADAPTER MEMORY MODULE

194xx	adapter test fail

210xx	SCSI FIXED DISK & CONTROLLER

210xx	disk or controller test fail

215xx	SCSI CD-ROM SYSTEM

215xx cd rom system test fail