DEC  1994

PC-NFS Driver Installation

for

D-Link DE-650 PCMCIA Ethernet Adapter

=====================================

*********************************** NOTE ***********************************

Each DE-650 series adapter comes with a driver diskette of its own.  If you

have more then one model of DE-650 adapter, make sure that you use the driver

diskette that was included with the DE-650 adapter you are using.

****************************************************************************

The D-Link PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS on your

PC with the D-Link DE-650 PCMCIA Ethernet Adapter.

The following files will be used in this installation:

Files		Contents

------------    --------------------------------------------------------

PDNFS.SYS	PC-NFS Driver Program.

DE65NPD.SYS	Packet driver program for DE-650 PCMCIA card.

DE65NPD.COM	Packet driver program for DE-650 PCMCIA card.

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 5 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \IBMPC\NFS directory of your harddisk:

PDNFS.SYS

DE65NPD.SYS

DE65NPD.COM

A:\IBMPC\INSTALL\*.*

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PCMCIA initial program    <--- Please see note below.

DEVICE=C:\NFS\DE65NPD.SYS <-d> <-w> <sw_int> <irq> <io_addr>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

Note:

Before running packet driver, run PCMCIA initial program on

A:\IBMPC\INSTALL directory. Please see as following:

*** PC installed Socket service, Card service, and Client driver ***

Skip this step

OR

*** PC installed Socket service or Databook Cardtalk ***

*** PC's PCMCIA controller is Intel 82365SL ***

*** PC is Toshiba T3300SL ***

DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

For detail, please refer to \IBMPC\INSTALL\README.DOC

[ Parameters for INSTALL utility ]

DCD16.EXE

DCD16.EXE is an integration version of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

For more detail information, please refer to \IBMPC\INSTALL\README.DOC.

device=DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Parameter description:

/iop   :I/O base

/irq   :IRQ value

/slt   :Slot number

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default value is 0. You must set 0 when you use slot 1.

Set slt:1 when slot 2 is used,etc.

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate other computers

through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 5 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your harddisk:

PDNFS.SYS

DE65NPD.SYS

DE65NPD.COM

A:\IBMPC\INSTALL\*.*

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

PCMCIA initial program    <------------------ (please see note below)

DE65NPD <-d> <-w> <sw_int> <irq> <io_addr>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Note:

Before run DE65NPD, run PCMCIA initial program on A:\IBMPC\INSTALL

directory.

Please see as following:

*** PC installed Socket service ,Card service, and Client driver ***

Skip this step

OR

*** PC installed Socket service or Databook Cardtalk ***

*** PC's PCMCIA controller is Intel 82365SL ***

*** PC is Toshiba T3300SL ***

device=DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Please refer to \IBMPC\INSTALL\README.doc for detail information.

[ Parameters for INSTALL utility ]

DCD16.EXE

DCD16.EXE is an integration version of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

For more detail information, please refer to \IBMPC\INSTALL\README.DOC.

device=DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Parameter description:

/iop   :I/O base

/irq   :IRQ value

/slt   :Slot number

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable irq value.

The default value is 0x5.

P2 :The selectable slot number.

The default value is 0. You must set 0 when you use slot 1.

Set slt:1 when slot 2 is used,etc.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate other computers

through the TCP/IP protocol.

Notes:

[-d]		Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

[-w]		The -w parameter is for use with the Windows environment only.

<sw_int>	Is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<irq>		Is the hardware interrupt (IRQ) line set on your network

interface card. The default is 5 if this parameter is not

specified.

<io_addr>	Is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.