STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v4.0 NDIS v3.1 Driver

For Ethernet LAN Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Supported network cards

o  An overview

o  Supplied files

o  Driver installation

- Primary installation (network card not presently installed)

- Network card already installed or older driver previously loaded

SUPPORTED NETWORK CARDS

o  EtherEZ Ethernet network cards

(SMC8416 board types)

o  Elite16 Ethernet network cards

(SMC8013 board types)

o  Ultra16 Ethernet network cards

(SMC8216 board type)

o  Elite network cards

(SMC8003 board type)

o  PLUS Ethernet network cards

(SMC/WD8003 board type)

o  EtherCard PLUS Micro Channel network cards

(SMC/WD8003xx/A or 8013xx/A board type)

OVERVIEW

This file explains how to install the Windows NT driver for x86-based

platforms.

SUPPLIED FILES

General files

o  NT_40.DOC        This file

o  RELEASE.DOC      Release notes for this version of the driver

Files to install the driver on Windows NT

o  SMC8000N.SY_      The driver

o  SMC8000N.HL_      The driver's associated help file

o  OEMSETUP.INF      The driver's setup file

DRIVER INSTALLATION

o Primary installation (network card not presently installed)

If you've already installed another card or loaded another driver,

refer to the procedure titled 'Network card already installed or

older driver previously loaded'.

NOTE:  If you experience any problems during driver installation,

you may choose to run EZStart and use its 'Automatic' feature

to configure the EtherEZ PnP SMC8416 network card. Refer to

the README.DOC file on the SuperDisk, or for further detail,

refer to the EZSTART.DOC file found within EZStart. EZStart

will not run in a Windows NT DOS window. You must boot from

DOS to setup and test card.

1.  Complete the Windows NT installation without an SMC network card

installed in your PC. Have the SMC SuperDisk and Windows NT

installation files (on CD or hard drive) available. When prompted

with a window asking about 'Participating on a network', change

the default to 'Do not connect this computer to a network at

this time'. Click 'Next' and follow the prompts to reboot.

2.  Install the SMC network card. Start Windows NT.

3.  Double-click the 'My Computer', 'Control Panel', and 'Network'

icons. The 'Network Configuration' window appears, prompting you

to install Windows NT networking. Select 'Yes'. Windows NT

invokes the 'Network Setup Wizard'. Click on 'Next'.

4.  When prompted to 'Search for a Network Adapter', click on 'Select

from List'; do not select 'Start Search'.

5.  When prompted to 'Select a Network Adapter', click on 'Have Disk'.

Insert the SuperDisk when prompted and click 'OK'.

6.  You are presented with the 'Select OEM Option' window. Highlight

'SMC8416 EtherEZ' (or appropriate) and click 'OK'.

7.  When prompted to start searching for a network adapter, click

on 'Next'.

8.  Select network protocols when prompted and click 'Next'.

NOTE:  To complete a TCP/IP installation, you will need to know

your IP Address and Subnet Mask.

9.  Click 'Next' and click 'Next' again when prompted to 'Install

Network Components'. When prompted for Windows NT files, type

the path to your Windows NT files on CD-ROM (e.g., D:\WINNT), or

change the path to those files on your hard drive, and click 'OK'.

10. You will be prompted with the 'Adapter Card Setup' window.

Verify the correct network card has been detected. Click on 'OK'

to accept the default settings. In most cases Windows NT will

detect the correct  card settings and you will not have to change

anything.

11. Click on 'Next' and click 'Next' again when prompted to 'Start

the Network'.

12. Enter the 'Workgroup' or 'Domain' names (optional) when prompted

and click on 'Next'.

13. Select 'Finish' when prompted and click on 'Yes' when prompted to

reboot. Remove the SuperDisk.

14. FOR FURTHER CUSTOMIZATION

You can modify card settings via the 'Network' applet. Install

additional protocols and network components by selecting the

appropriate folder (e.g., Customizing Protocols, Services,

Identification, or Bindings).

o Network card already installed or older driver previously loaded

Follow this procedure to replace Windows NT driver files with the

latest SMC driver files. By removing driver files, you will lose

network settings. Make sure you have a record of settings such as IP

address, workgroup name, etc. Check with your network administrator

for complete information.

NOTE:  If you experience any problems during driver installation,

you may choose to run EZStart and use its 'Automatic' feature

to configure the EtherEZ PnP SMC8416 network card. Refer to

the README.DOC file on the SuperDisk, or for further detail,

refer to the EZSTART.DOC file found within EZStart. EZStart

will not run in a Windows NT DOS window. You must boot from

DOS to setup and test the card.

1.  Install the SMC network card (if not already installed), attach

network cable, and boot Windows NT.

2.  Double-click the 'My Computer', 'Control Panel', and 'Network'

icons. From the 'Network' window, select the 'Adapter' tab.

3.  Highlight the network driver you wish to remove and click

'Remove'. (You may choose to record network card settings for

future reference. If so, highlight the network card and click on

'Properties'.) Click 'Yes'.

4.  Click 'Close' and select 'Yes' when prompted to restart.

5.  Double-click the 'My Computer', 'Control Panel', and 'Network'

icons and select the 'Adapters' tab.

6.  Do not select any of the network cards listed. Select 'Add'.

7.  You are presented with the 'Select Network Adapter' window. Click

on 'Have Disk'.

8.  You are presented with the 'Insert Disk' window. Specify the path

to the root directory of the SuperDisk (e.g., A:>) and click 'OK'.

9.  You are presented with the 'Select OEM Option' window. Highlight

'SMC8416 EtherEZ' (or appropriate) and click 'OK'.

10. You will be prompted with the 'Adapter Card Setup' window.

Verify the correct network card has been detected. Click on 'OK'

to accept the default settings. In most cases Windows NT will

detect the correct  card settings and you will not have to change

anything.

11. You are presented with the 'Network' window where the SMC network

card is now listed as an installed adapter. At this point you can

select 'Close' to exit the 'Network' applet and follow the prompts

to restart Windows NT or, you may choose to perform the optional

procedures described below.

Optional:  Install additional protocols and network components by

selecting the appropriate folder (e.g., Customizing

Protocols, Services, Identification, or Bindings). Refer

to the Microsoft Windows NT installation guide for more

information. After completing your changes click on

'Close' to exit the 'Network' applet.

12. Select 'Yes' when prompted to reboot for the changes to take

effect.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation, 80 Arkay Drive,

Hauppauge, NY 11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, PLUS, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.