S  O  U  N  D  B  L  A  S  T  E  R

D e v e l o p e r   K i t

D o c u m e n t a t i o n

(From paper to file by NMR 1991.  Complete documentation)

(Sections 1 - 7 and 10 are not included, because they don't contain

any usefull info! - Erwin.)

=========================================================================

TABLE OF CONTENTS

HARDWARE TECHNICAL REFERENCE

Chapter 8      Programming the DSP

Chapter 9      DSP Interface for Voice

Chapter 11     DSP Commands

Chapter  8    PROGRAMMING THE DSP

Introduction .................................8-3

Resetting DSP ................................8-4

Writing to DSP ...............................8-5

Reading from DSP .............................8-5

Handling interrupt from DSP ..................8-6

8-1

==========================================================================

<page 8-2 left blank intentionally>

PROGRAMMING THE DSP

Introduction

The Sound Blaster has a DSP Chip (CT-DSP 1321) wich handles all the

voice I/O and MIDI operations.

The DSP is programmed through 4 specific I/O adresses wich are selectable

on the Sound Blaster Card (SBC).

These I/O adresses are:

a. RESET Port-output to 2x6H.

b. READ DATA Port-input from 2xAH.

c. WRITE COMMAND or DATA-output to 2xCH.

WRITE-BUFFER status-input from 2xCH.

d. DATA-AVAILABLE status-input from 2xEH.

"x" is the number of the jumper selectable from 1 to 6, wich specifies

the base adress of the SBC as 210H, 220H, 230H, 240H, 250H, 260H.

The DSP also provides a hardware interrupt for some of its functions.

This interrupts is selectable from 2, 3, 5 or 7.

Within the interrupt service routine, the DSP AVAILABLE Status Port

(2xEH) is read to acknowledge the interrupt.

8-3

==========================================================================

Resetting DSP

-------------

The DSP has to be reset once before it is programmed.  The reset causes

it to perform an initialization.

After the initialization, DSP returns a data byte 0AAH at the READ DATA

Port.  (Refer to the section on Reading Data from DSP)

Procedure to reset DSP:

a.  Write a "1" to RESET Port (2x6H) and wait for 3 micro-seconds.

b.  Write a "0" to RESET Port.

c.  Poll for a READY BYTE = 0AAH from the READ DATA PORT (2xAH).

Before reading 2xAH, it is advisable to check the DATA AVAILABLE

status at port 2xEH, i.e. MSB (most significant bit)

If the return value is not 0AAH after about 100 loops, or there is no

return value at all, it may be a reset error or an incorrect I/O adress

is being used.  The routine should abort the loop and declare an

error.  Typically, the DSP takes about 100 micro-seconds to initialize

itself.

8-4

==========================================================================

Writing to DSP

--------------

The DSP's WRITE COMMAND/DATA Port (2xCH) is used to write command/data

to the DSP.

Before a command/data is written to DSP, the most significant bit

(MSB) of WRITE BUFFER status Port (input 2xCH) must be checked to ensure

that the DSP is ready to receive the command or data.

The DSP is ready to receive command/data only when the MSB of the

WRITE BUFFER status is a "0".

Procedure to write to DSP:

a. Read the DSP's WRITE BUFFER status-Port 2xCH.

b. Repeat (a) until the MSB read is "0".

c. Write the data to the WRITE COMMAND/DATA Port.

Reading from DSP

----------------

Data is read from the DSP through the READ DATA Port (2xAH).

Before the data is read, the MSB of DATA AVAILABLE status Port (2xEH)

is checked to see that data is ready for reading.

When the MSB of DATA AVAILABLE status Port (2xEH) is checked

to see that data is ready for reading.

When the MSB of DATA AVAILABLE status is a "1", data can be read from

the READ DATA Port (2xAH).

Procedure to read from DSP:

a. Read the DSP's DATA AVAILABLE Status Port (2xEH).

b. Repeat (a) until the MSB is a "1".

c. Read the data from DSP's READ DATA Port (2xAH).

8-5

=========================================================================

Handling Interrupt from DSP

DSP provides hardware interrupt for the following functions:

a. DMA mode for DAC

b. DMA mode for ADC

c. Interrupt mode for MIDI input

An interrupt service routine has to be set up to process the interrupt.

Within the interrupt service routine, the interrupt is acknowledged by

reading the DATA AVAILABLE Status Port (2xEH) once.

For functions that DSP provides data to read (interrupt mode MIDI

input), the data is read from the READ DATA Port (2xAH).

(Note:the interrupt has to be acknowledged after the data is read).

At the end of the interrupt service routine, the command 20H (EOI,

End Of Interrupt) is output to the interrupt controller port 20H.

8-6

==========================================================================

Chapter  9    DSP INTERFACE FOR VOICE

Operation Mode....................................9-3

Direct Mode......................................9-3

DMA Mode.........................................9-4

Sampling Rate.....................................9-5

Direct Mode......................................9-5

DMA Mode.........................................9-5

Voice Data Size...................................9-7

Programming the DMAC..............................9-8

End of DMA Interrupt.............................9-11

Use of Reference Byte............................9-12

Summary..........................................9-13

9-1

==========================================================================

<page 9-2 left blank intentionally>

DSP Interface for Voice

In order to program the DSP for digitized sound input/output, this

section must be read in detail.  It can only be skipped when you

are calling the CT-VOICE.DRV for voice I/O.

This section assumes that you have certain knowledge of programming

the interrupt service routine and the DMA (DMA controller 8237).

Operation Mode

--------------

DSP provides functions for digitized voice input and output in 2 modes:

a. Direct mode

b. DMA mode

Direct mode

-----------

In direct mode, voice data is output to our input from DSP directly

by the application program.  All the data are handled in a 8-bit format.

9-3

==========================================================================

DMA mode

--------

Under the DMA mode, the Channel 1 of the DMAC on the CPU is programmed

to transfer the data between the memory and the DSP.  This frees the

application program for other tasks.

Besides the 8-bit data format, DSP supports 3 additional compressed

data format for voice output.  The digitized voice can be compressed by

software to save disk and memory space.  The DSP decompresses the

data during voice output.  This has a clear advantage in that the

CPU need not spend time decompressing the data and memory wasted to

store the decompressed data.

The compression schemes used are:

a. 4-bit DAC      (2 to 1 compression)

b. 2.6-bit DAC    (3 to 1 compression)

c. 2-bit DAC      (4 to 1 compression)

9-4

==========================================================================

Sampling Rate

-------------

Depending on the mode of operation-Direct or DMA, the sampling rate is

handled differently.

Direct mode

-----------

Sampling rate depends on the number of data read from or written to the

DSP at a constant interval within a second.

For example, to record voice at 8000HZ sampling rate, the program has

to perform 8000 ADC at equal interval within a second.

Usually, the Timer interrupt (INT 8) is used to faciliate this

implementation.  The frequency of the interrupt is reprogrammed, so

that, it interrupts at the number of time needed for that particular

sampling rate.  A new Timer interrupt service routine is used to read

the ADC data from or write the DAC data to the DSP.

There is no minimum sampling rate in this case and the maximum depends

on how fast the Timer interrupt can be programmed and the interrupt

service routine can service the interrupt.

DMA mode

--------

Under DMA mode, the sampling rate is controlled by the DSP.  The DSP

uses a one-byte variable, called TIME_CONSTANT, to generate the DMA

request for the DMA Controller on the system to transfer the data.

The TIME_CONSTANT variable is set by the DSP command's 40H.  It only

requires to be set once, unless sampling rate needs to be changed.

9-5

==========================================================================

The formula for the TIME_CONSTANT is as follow:

TIME_CONSTANT=256-1000000/sampling rate

Eg.

For a 8000Hz sampling rate,

TIME_CONSTANT        = 256-1000000/8000

= 131

Due to the difference in complexity of each scheme, the maximum sampling

rate for each scheme varies:

Input(ADC) :       8-bit only,      4 KHz to 12 KHz

Output(ADC):       8-bit,           4 KHz to 23 KHz

4-bit ADPCM,     4 KHz to 12 KHz

2.6-bit ADPCM,   4 KHz to 13 KHz

2-bit ADPCM,     4 KHz to 11 KHz

Note:  Setting to a sampling rate that is beyond the capability of that

mode will result in no increase in actual sampling rate.

9-6

==========================================================================

Voice Data Size

---------------

Theoretically, for the Direct and DMA mode, the size of voice data can

be transferred is limited by the size of memory buffer available on the

system for the voice data.

For Direct mode, since the data is output/input directly by the application

program through the DSP, the size of data to be transferred is handled

by the application program.

Under the DMA mode, the DATA_LENGTH is used as a 2-byte parameter for

any of the voice input/output command.  The LSB (least significant byte)

of the DATA_LENGTH is written to the DSP after the command and followed

by the MSB.

The DATA_LENGTH is defined as:

DATA_LENGTH=actual number of data - 1.

For example, to transfer 1 byte of data, DATA_LENGTH = 0.  To transfer

a full 64KB of data, DATA_LENGTH = 65535 (FFFFH).

Note:   1. Due to the characteristic of the DMAC, the actual size of data

to transfer at a time is limited only by the physical page

boundary, wich is 64KB at maximum.

2. The details on how to transfer data beyond 64KB is left to

a later section.

9-7

==========================================================================

Programming the DMAC

--------------------

The discussion here only provides brief instructions on programming the

DMAC (8237) for transferring the data between the memory and the DSP.

To have a better understanding of programming the DMAC (DMA Controller),

refer to its Data Sheet.

Two unique characteristics of the DMA have to be discussed.

a.  The DMAC can only transfer data within a physical memory page.

The system memory is divided into physical page of 64KB each.

For a 1 MB memory, it is divided into 16 pages as follows:

Page        Segment:Offset Adress

0                   0000:0000-0000:FFFF

1                   1000:0000-1000:FFFF

2                   2000:0000-2000:FFFF

:                           :

:                           :

:                           :

E                   E000:0000:E000:FFFF

F                   F000:0000:F000:FFFF

As an example, a set of voice data stored at the segment-offset adress

6C00:0000 to 6C00:5FFF is actually residing in 2 physical pages:

-    The ending part of physical page 6 from offset C000H onwards.

-    The beginning part of page 7 until offset 1FFFFH.

9-8

==========================================================================

The DMAC (DMA controller) cannot be programmed to transfer this set of

data in one go as they are not within the same physical page.

It must be handled as 2 blocks.

Therefore, a set of voice data has to be divided into sub-blocks at the

physical page boundary.  This limits the maximum number of data that can

be transferred each time to 64KB.

b.  The DMAC's data counter is always 1 byte less than the actual number

of bytes to transfer.

This is how the DATA_LENGTH for the DSP, discussed in previous

section, is being defined.

The parameters for programming the DMAC are:

a. Page Number     - The physical page adress of the data for DMA.

b. Base Adress     - A 2-byte value for the offset adress of the

data in the physical page.  The LSB comes

first, then the MSB.

c. Data Counter    - A 2-byte value for the DATA_LENGTH as defined

in a previous section.  LSB comes first also.

d. Transfer Mode   - A byte that indicates the direction of transfer

(read from or write to memory), direction of

memory adress change and other parameters.

The value to use are:

For voice input, (DAC)     -49H

For voice output, (ADC)    -45H

9-9

==========================================================================

The DMAC is programmed through a few I/O ports.  The procedure is listed

below:

a. out 0AH with 5; mask off channel 1

b. out 0CH with 0; clear byte pointer F/F to lower byte

c. out 0BH with 49H; set transfer mode to DAC, for ADC, it is 45H

d. out 02H with LSB of Base Adress

e. out 02H with MSB of Base Adress

f. out 83H with Page Number

g. out 03H with LSB of Data Counter

h. out 03H with MSB of Data Counter

i. out 0AH with 1; enable channel 1

Note:  The DMAC must be programmed before the DSP is programmed for

the DMA transfer.

9-10

==========================================================================

End of DMA Interrupt

--------------------

The DSP is designed to provide a hardware interrupt at the end of each

DMA transfer.  The actual interrupt number selectable from 2, 3, 5

and 7 on the card.  (Refer to the Installation Note for detail)

For voice data that resides in more than one physical page, it

must be divided into sub-blocks at the page boundary for transfer.

When the interrupt occurs after transferring each sub-blocks, the

interrupt service routine programs the DMAC and the DSP to transfer

next sub-block, until all the blocks are transferred.  This solves

the problem of a voice data that crosses the physical page boundary,

be it more than 64K or not.

The actions of the interrupt service routine are as follow:

a. Preserve machine status

b.  Acknowledge the DSP interrupt

c.  Goto (f), if no more blocks to transfer

d.  Program the DMAC for next block

e.  Program the DSP for next block

f.  Output EOI(End of Interrupt) to Interrupt Controller at I/O port

20H

g.  Restore machine status

h.  Execute an IRET

Note:  1.  The hardware interrupt is also enabled/disabled by the

Interrupt Register of the DMAC at I/O port (21H).

2.  Application program has to clear/set the corresponding

bit on the port for the interrupt number used.  MSB (bit 7)

for interrupt 7 and LSB (bit 0) for interrupt 0.

9-11

==========================================================================

Use Of Reference Byte

---------------------

The compression scheme used by DSP is known as ADPCM (Adaptive Delta

Pulse Code Modulation).  This scheme stores the difference between data

values instead of actual data.

The first byte of the compressed data is always a REFERENCE byte

wich is not the ADPCM code but the actual data byte value.  This

byte is used by the DSP as a reference during the decompression

of the data.

A separate set of ADPCM DAC commands are used to identify the REFERENCE

byte in a data block.  As the compressed data may be sent to the DSP

in a few sub-blocks, due to page crossing problem discussed earlier, and

only the first block contains the REFERENCE byte.

The different ADPCM DAC commands for each compression scheme are shown

below:  (Refer to each command's specification for more detail)

������������������������������������������������������Ŀ

� Compression  �      with         �      without      �

�   Scheme     � REFERENCE Byte    �  REFERENCE Byte   �

������������������������������������������������������Ĵ

�   4-bit      �      75H          �      74H          �

�              �                   �                   �

�  2.6-bit     �      77H          �      76H          �

�              �                   �                   �

�   2-bit      �      7BH          �      7AH          �

��������������������������������������������������������

9-12

==========================================================================

Summary

-------

The procedure to output voice (DAC) is listed below:

a. Progra the TIME_CONSTANT for the DSP, if necessary

b. Setup the interrupt service routine to handle the

End of DMA interrupt

c. Program the DMAC for the voice output, with 49H as the mode

register value

d. Unmask the Interrupt Mask Register at I/O port 21H

e. Program the DSP to output the first sub-block with a

REFERENCE byte, if the compressed data is used

f. At each interrupt service, output subsequent block until all

blocks are output

g. When all blocks are output, mask the Interrupt Mask Register

h. Restore the original interrupt service routine

9-13

==========================================================================

The procedure to input a voice (ADC) is listed below:

a. Program the TIME_CONSTANT for the DSP, if necessary

b. Setup the interrupt service routine to handle the End of DMA interrupt

c. Program the DMAC for the voice input, with 45H as the mode register

value

d. Unmask the Interrupt Mask Register at I/O port 21H

e. Program the DSP to input the first sub-block

f. At each interrupt service, input next sub-block until memory buffer

full

g. Mask the Interrupt Mask Register

h. Restore the original interrupt service routine

9-14

==========================================================================

=========================================================================

<page 10-4 left blank intentionally>

Chapter  11      DSP COMMMANDS

Command:  1xH...................................11-3

Command:  2xH...................................11-5

Command:  3xH...................................11-3

Command:  40H...................................11-5

Command:  7xH...................................11-3

Command:  DxH...................................11-5

DMA Control....................................11-10

Summary of DSP Commands........................11-12

Summary of DSP I/O Ports.......................11-13

11-1

==========================================================================

<page 11-2 left blank intentionally>

DSP Commands

Command: 1xH

Description: 8-bit DAC & 2-bit ADPCM DAC

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � ��       � DMA      � 2-BIT    � REFERENCE �

�       �          �          � MODE     � ADPCM BYTE�

�       �          �          �          �           �

�  0    � ��       � ��       � 8-BIT    � DIRECT    �

�       �          �          � UNPACKED � MODE      �

������������������������������������������������������

Commands & Functions

10H   : DIRECT 8-BIT DAC (single byte transfer)

14H   : DMA mode 8-BIT DAC

16H   : DMA mode 2-BIT ADPCM DAC

17H   : DMA mode 2-BIT ADPCM DAC with REFERENCE BYTE

10H   : DIRECT mode DAC

The interface program is responsible for controlling the

sampling rate.

(NB:DIRECT mode 2-bit ADPCM DAC is not available)

(1)  Send Command 10H

(2)  Send single byte of data

(3)  Wait for the correct timing and repeat (1) & (2) until

end of transfer

11-3

==========================================================================

14H   : DMA mode 8-BIT DAC

(1)  Load data for DAC to memory

(2)  Set up DMAC (8237) for DMA operation

(Refer to the section on Programming the DMAC)

(3)  Set the DSP TIME_CONSTANT to the desired sampling

rate

(Refer to Command 40H)

(4)  Send Command 14H

(5)  Send DATA_LENGTH (2 bytes, LSB first) where

DATA_LENGTH + 1 is the size of the data block to transfer

(6)  Transfer of the whole block (blocksize=DATA_LENGTH+1)

starts immediately after (5)

(7)  Break larger blocks that crosses 64K-memory page into

smaller blocks and send subsequent blocks via the

END_DMA interrupt service routine.

16H/17H: DMA mode 2-BIT ADPCM DAC

(1)  Load data for DAC to memory

(2)  Set up DMAC (8237) for DMA operation

(3)  Set the DSP TIME_CONSTANT to the desired sampling rate

(4)  Send Command 17H (indicating first DAC byte in memory

is a REFERENCE byte)

(5)  Send DATA_LENGTH (2 bytes, LSB first)

(6)  Transfer of the whole block(blocksize=DATA_LENGTH+1)

starts immediately after (5)

(7)  Break larger blocks that crosses the 64K-memory page into

smaller blocks and send subsequent blocks via the

END_DMA interrupt

(8)  Subsequent blocks should be send by the 16H command

since there is no REFERENCE byte in them

Remark:  Processing MIDI OUT and DAC together.

Only in DMA mode can the DAC be processed simultaneously

with the MIDI OUT

11-4

==========================================================================

Command: 2xH

Description: ADC INPUT

The ADC (Analog-Digital Convertor) input comes from a microphone amplifier.

A microphone can be plugged directly to the topmost socket on the

back of the card.

ADC is 8-bit either in DIRECT or DMA modes.

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � ��       � DMA      � ��       � ��        �

�       �          � MODE     �          �           �

�       �          �          �          �           �

�  0    � ��       � ��       � ��       � DIRECT    �

�       �          �          �          � MODE      �

������������������������������������������������������

Commands & Functions

20H : DIRECT ADC (Single Byte Data Transfer)

24H : DMA mode ADC

20H : DIRECT MODE ADC

(1)  Send Command 20H

(2)  Read single byte of ADC DATA from DSP

(3)  Wait for the correct timing and repeat (1) & (2) until

end of transfer

24H : DMA mode ADC

(1)  Send Command 24H

(2)  Send DATA_LENGTH (2 bytes, LSB first) to DSP

(3)  Break larger blocks that crosses the DMA page into sub-blocks

and request a new sub-block for DMA input at the

END_DMA interrupt service routine

(4)  Repeat (1), (2) & (3) until end of transfer

All data digitized are 8-bit, compressed to 4-bit, 2-bit or 2.6-bit

are done by the VOXKIT.EXE software.

11-5

==========================================================================

Command: 3xH

Description: Reading and Writing the MIDI Port

The MIDI port described here is a basic MIDI port that conforms to

the specifications of the International MIDI Association.

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � WRITE    � ��       � ��       � INTERRUPT �

�       �          �          �          � READ      �

�       �          �          �          �           �

�  0    � READ     � ��       � ��       � ��        �

�       �          �          �          �           �

������������������������������������������������������

There are 3 modes available:

30H : MIDI READ (polling mode)

31H : MIDI READ (interrupt mode)

38H : MIDI WRITE (polling mode)

30H : MIDI READ (polling mode)

Issue this command followed by reading a MIDI data byte from

READ DATA port (2xAH) of the DSP.  The program must check

bit 7 (=1) of the DATA AVAILABLE status port (2xEH) before

reading.  If no MIDI data is available, the program must

repeatedly poll the status bit until MIDI data is available.

31H : MIDI READ (interrupt mode)

This is the suggested mode of reading MIDI data.

Send Command 31H.  When a MIDI data is available, the DSP

will interrupt the CPU.  An Interrupt Service Routine must be

set up to read the MIDI data.

The MIDI data is read from the READ DATA port, then a read

on the status port (2xEH) will clear the Interrupt Request

IRQ (2, 3, 5 & 7).

Send Command 31H again to stop the process.

11-6

==========================================================================

Note: If MIDI data comes in too rapidly, the DSP can buffer

up to 64 bytes of MIDI data.

38H : MIDI WRITE

Issue Command 38H followed by one byte of the MIDI data to

write.  Program must check the WRITE-BUFFER STATUS PORT

(2xCH) for bit 7=0, before writing any command or data.

11-7

==========================================================================

Command: 40H

Description: SET THE TIME CONSTANT

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � ��       � ��       � ��       � ��        �

�       �          �          �          �           �

�       �          �          �          �           �

�  0    � ��       � ��       � ��       � ��        �

�       �          �          �          �           �

������������������������������������������������������

Sampling rate for DAC and ADC in DMA modes is set by setting the

TIME_CONSTANT of the DSP.

Command & Function

40H :  SET TIME CONSTANT

(1)  Send Command 40H

(2)  Send one byte TIME_CONSTANT

TIME_CONSTANT = 256-1000000/8000

= 131

Eg.

Required sampling rate of 8000Hz

TIME_CONSTANT = 256-1000000/8000

= 131

Lowest sampling rate possible :      3906.25 Hz

when TIME_CONSTANT = 0

11-8

==========================================================================

Command : 7xH

Description: 4-bit ADPCM DAC & 2.6-BIT ADPCM DAC

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � ��       � DMA      � 2.6-BIT  � REFERENCE �

�       �          � MODE     � ADPCM    � BYTE      �

�       �          �          �          �           �

�  0    � ��       � ��       � 4-BIT    � ��        �

�       �          �          � ADPCM    �           �

������������������������������������������������������

Commands & Functions

74H : DMA mode 4-bit ADPCM DAC

75H : DMA mode 4-bit ADPCM DAC with REFERENCE BYTE

76H : DMA mode 2.6-bit ADPCM DAC

77H : DMA mode 2.6-bit ADPCM DAC with REFERENCE BYTE

Remarks:

Procedure is similiar to Command 1xH.

11-9

==========================================================================

Command: DxH

Description: DMA & SPEAKER CONTROL

Commands & Functions

D0H : HALT DMA

D1H : TURN ON SPEAKER

D3H : TURN OFF SPEAKER

D4H : CONTINUE DMA

SPEAKER CONTROL

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � ��       � ��       � SPEAKER  � ��        �

�       �          �          � OFF      �           �

�       �          �          �          �           �

�  0    � ��       � ��       � SPEAKER  � ��        �

�       �          �          � ON       �           �

������������������������������������������������������

D1H : TURN ON SPEAKER

The speaker here refers to the connection of the DAC output

to the amplifier input.  When Speaker is ON, it means that the

DAC's output is sent to the amplifier for output to the speaker.

It does not affect the C/MS music or FM music.

D3H : TURN OFF SPEAKER

The DAC is disconnected from the speaker, no sound from the

DSP will be heard.

The ON/OFF SPEAKER commands could be used to mute the voice

output during a game.  Since most game action could be

dependent on the voice output.  The OFF SPEAKER command

will mute the voice, but the DSP will carry on the voice output

as usual without affecting the game action.

11-10

==========================================================================

DMA Control:

����������������������������������������������������Ŀ

�status � BIT-3    � BIT-2    � BIT-1    � BIT-0     �

����������������������������������������������������Ĵ

�  1    � ��       � CONTINUE � ��       � 0         �

�       �          � DMA      �          �           �

�       �          �          �          �           �

�  0    � ��       � HALT     � ��       � 0         �

�       �          � DMA      �          �           �

������������������������������������������������������

D0H : HALT DMA

HALT the DMA operation that could be going on between the DSP

and the CPU.  The DSP will cease to send out DMA request for

any DMA mode of operation.

D4H : CONTINUE DMA

Continue the halted DMA operation.

11-11

==========================================================================

Summary of DSP Commands

���������������������������������������������������������������Ŀ

�CODE �      COMMAND        � BIT3� BIT2� BIT1� BIT0� REMARKS   �

���������������������������������������������������������������Ĵ

� 0x  �    INVALID          �  X  �  X  �  X  �  X  �           �

���������������������������������������������������������������Ĵ

� 1x  �  8-BIT & 2-BIT DAC  �  -  � DMA � 2BIT� 2BIT� 10:DIRECT �

���������������������������������������������������������������Ĵ

� 2x  �     8-BIT ADC       �  -  � DMA �  X  �REF X� 20:DIRECT �

���������������������������������������������������������������Ĵ

� 3x  �      MIDI           �WR/RD�  X  �  X  �I/REA�           �

���������������������������������������������������������������Ĵ

� 4x  �  SET TIME CONSTANT  �  X  �  X  �  X  �  X  �           �

���������������������������������������������������������������Ĵ

� 5x  �    (Reserved)       �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� 6x  �    (Reserved)       �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� 7x  � 4-BIT & 2.6-BIT DAC �  -  � DMA �2.6BT� REF �           �

���������������������������������������������������������������Ĵ

� 8x  �    (Reserved)       �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� 9x  �      ----           �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� Ax  �      ----           �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� Bx  �      ----           �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� Cx  �      ----           �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� Dx  �DMA & SPEAKER CONTROL�  X  �R/DMA�OF/ON�ED/SC�           �

���������������������������������������������������������������Ĵ

� Ex  �    (Reserved)       �     �     �     �     �           �

���������������������������������������������������������������Ĵ

� Fx  �      ----           �     �     �     �     �           �

�����������������������������������������������������������������

11-12

==========================================================================

Summary of DSP I/O Ports

I/O        Description            I/O Adress

---        --------------         ----------

WRITE      DSP RESET              = OUT 2x6H,1; WAIT 3 uS; OUT 2x6H,0

WRITE      DATA/COMMAND           = 2xCH

READ       WRITE-BUFFER STATUS    = 2xCH/D7=0, buffer free

READ       READ DATA              = 2xAH

READ       DATA-AVAILABLE STATUS  = 2xEH/D7=1, data available

<EOF>