| Product | : | DSL 305EU (Binatone) |
| --- | --- | --- |
| Tested by | : | Lee Chee Soon |
| Hardware Version | : | V0.2 |
| Firmware Version | : | V21.19.2 |
| Test Dates | : | September 5, 2002 - September 11, 2002 |
| Test Type | : | Functional Test |
| Test Objectives | : | To test the functionality of the modem. |
| Review By | : |  |

1	Introduction

This test plan would specify the functionality of the ADSL modem and also provide future reference.

1.1	ADSL Modem Information

| Modem Name/Project | DSL 305E/EU |
| --- | --- |
| Chipset and Manufacturer | Conexant CX82310 |
| Firmware Version | 21.19.2 |
| Other Features | Flash ROM upgradeable |

1.2	Test Setup

| CPE Configuration |  |
| --- | --- |
| Manufacturer | Aztech Systems Limited |
| Model | DSL 305E/EU |
| Chipset | Conexant CX82310 |
| Line Driver/Receiver | - |
| Hardware Revision | V0.2 |
| Firmware/Driver Revision | V20.6.4 |
| Application S/W Revision | Web Management |
| ADSL Head-end Configuration |  |
| DSLAM Manufacturer | - |
| DSLAM Model | - |
| Number of Ports/Line Card | - |
| Line Card Revision | - |
| Line Card AFE Chipset | - |
| Line Card Firmware | - |
| Software Load Version | - |
| Wireline Simulator | - |
| Hardware Revision | - |

Notes:

- Please use the common ADSL test system for all tests.

1.3	System Configuration

This section specifies other device available on the test system and also the OS platform for conducting the test. This can be used for debugging purposes.

| # System Configuration |  |
| --- | --- |
| System Name | Fujitsu Siemens Computers |
| Processors | Pentium III – 500Mhz |
| BIOS Version | W6183F1 V1.4B3 042400 |
| System Memory | 128MB RDRAM |
| System Chipset | Intel (ICH 82801AA) |
| Hard drive | IBM 3.2GB |
| CD-ROM Drive | 12x CD-ROM |
| Video cards | On-board Graphics 810E Graphics Controller |
| Network Adapter | Linksys LNE100TX Fast Ethernet Accton EN1207F Series PCI Fast Ethernet |
| Sound | On-board Intel 82801AA AC’97 Audio Controller |
| PCI slot for NIC | 3 PCI slots |
| Windows 95 OSR2.1 | Not Tested |
| Windows 98 | Not Tested |
| Windows 98 SE | Tested |
| Windows ME | Tested |
| Windows 2000 RTM | Tested |
| Windows XP Pro | Tested |

Table 1.1: Target Systems Configuration

Network Setup

DSL305E/EU

DSLAM

Internet

\	\

Figure 1.1: Network Setup

1.4	Physical Layer and Hardware

| Requirement |  | Result | Description |
| --- | --- | --- | --- |
| 1.4.1 | Ethernet with Full Duplex | Pass *10/100Mbps auto selection | The modem must have at least one 10BaseT Full Duplex Ethernet port. |
| 1.4.2 | Auto-sensing Ethernet wiring | Not supported | The user may apply a crossed or a non-crossed Ethernet cable without notice. |
| 1.4.3 | Auto-sensing ADSL wiring | Not supported | The user may apply a RJ-11 cable using either the pair on pins 3&4 or the pair on pins 2&5 for the ADSL signal without notice. |
| 1.4.4 | Power On/Off Switch | Not Applicable | The modem must have a power on/off switch. |
| 1.4.5 | Reset-to-Factory Default button | Pass | With a simple method it must be possible to reset the modem configuration to its factory default setting. The preferred method is to apply a power cycle while holding a reset button for at least 5 seconds. |
| 1.4.6 | No Audible Noise | Pass | During normal operation, the level of audible noise level should be negligible and meet standards as for devices used in silent household environments. |

1.5	Firmware Requirements

| Requirement |  | Result | Description |
| --- | --- | --- | --- |
| 1.5.1 | Dynamic Host Configuration Protocol (DHCP) Server | Pass | - Ensure that the DHCP Server option in the router modem is enable. Other option such as NAT can be disable for the testing purpose at this state. - Set the TCP/IP Properties option in the Network applet under Control Panel windows with its <IP Address> tab set to “Obtain an IP address automatically” option. |
| 1.5.2 | Network Address Translation (NAT) / Network Address Port Translation (NAPT) | Pass |  |
| 1.5.3 | PAP and CHAP Authentication Test (RFC 2364 and RFC2516) | Pass * Selection is made in Web Interface | - Connect to PPP server using PAP and CHAP authentication method. - Make sure the PPP session is established and surf the Internet for a while. |
| 1.5.4 | PVC Support | Pass |  |
| 1.5.5 | Firmware updates | Pass | The modem firmware must be capable of being locally or remotely upgraded. |
| 1.5.6 | Default IP address | Pass | The modem and its embedded servers use 10.0.0.2 as IP address by factory default setting. The DHCP server is using the 10.0.0.X space by default. |
| 1.5.7 | OAMLOOPBACK F4/F5 with End-to-End or Segment-to-Segment | Pass *Tested on F5 End-to-End | Using Web Interface, under Diagnostic Test |
| 1.5.8 | IPSec Pass-Through | Unable to test | IPSec Pass-Through with more than one VPN client active on the LAN. It is acceptable that the VPN Clients each connect to a different VPN Server with a distinct WAN IP address. |
| 1.5.9 | Multiple IPSec Pass-Through | Unable to test | Multiple IPSec Client Pass-Through (more than one VPN client active) e.g. by applying dynamic port mapping of LAN port 500 to WAN ports 500, 501, 502, … |

1.6	Management Requirements

| Requirement |  | Result | Description |
| --- | --- | --- | --- |
| 1.6.1 | PPP User ID and Password | Pass | The PPP User ID and Password will saved to the flash. |
| 1.6.2 | Admin User ID and Password | Pass | The Admin User ID and Password must be clearly specified the access right. |
| 1.6.3 | Utility for collecting traffic statistics | Pass | Check whether the Status shows the traffic statistics on the WAN interface, Ethernet interface, and USB interface or not. |
| 1.6.4 | Remote Access Enable / Disable | Pass | The modem must allow the user to turn on/off all remote access. |

1.7	Router Features Functionality Test

This test is used to specify the basic router features functionality available on the device under test, under the connected environment.  To simplify the test environment and system setup for connectivity testing.  We focus on the testing of these features using the test system under different OS environments.

| Test Title |  | Win98SE | WinME | Win2000 | WinXP | Test Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| 1.7.1 | DNS Queries | Pass | Pass | Pass | Pass | - Make sure all the protocols are able to connect to Internet. - Use domain name to surf the net. E.g. http://www.yahoo.com/, http://www.microsoft.com/. |
| 1.7.2 | Multiple PVC | Pass | Pass | Pass | Pass | The modem must be able to configure for multiple PVC. |
| 1.7.3 | 10Mbps port connectivity test | Pass | Pass | Pass | Pass | Check the modem is able to transfer data using 10Mbps NIC. |
| 1.7.4 | 100Mbps port connectivity test | Pass | Pass | Pass | Pass | Check the modem is able to transfer data using 100Mbps NIC. |
| 1.7.5 | WAN connectivity test for DSL 305EU | Pass | Pass | Pass | Pass | Checks that all systems are able to surf the Internet when the modem is connected to a switch or hub. |
| 1.7.6 | USB port connectivity test for DSL 305EU | Fail Refer to * | Fail Refer to * | Fail Refer to * | Fail Refer to * | - Check that when the USB cable is connected to the USB port of the system, the USB Link led is on. - Checks that when system configured to work under same workgroup. The Link/Act led is on and blinking when accessing information across systems. |

* USB Link light blinks regardless connected or not connected to the system.

* Upcoming firmware changes will be able to solve this issue.

1.8	USB Driver Installation/Uninstallation Test for DSL 305EU

| Test Title |  | Win98SE | WinME | Win2000 | WinXP | Test Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| 1.8.1 | Plug and Play Installation | Pass | Pass | Pass | Pass | At OS startup, verify that the USB gateway modem is auto detected and able to identify the correct driver. The installation should be able to recover from normal boot. |
| 1.8.2 | Verify Control Panel | Pass | Pass | Pass | Pass | Under Add/Remove Programs, verify that whether the uninstallation program is available or not. |
| 1.8.3 | Verify Device Manager | Pass | Pass | Pass | Pass | Verify that the USB gateway modem is properly installed or not. |
| 1.8.4 | USB Port Independence | Pass | Pass | Pass | Pass | The USB gateway modem should be able to function when plug into one of the USB ports. |
| 1.8.5 | Device compatibility | Pass | Pass | Pass | Pass | The USB gateway modem must be able to work with other USB device connected. E.g. USB Hub / Keyboard / Camera. |
| 1.8.6 | Removing Drivers using the “Add/Remove Programs” | Pass | Pass | Pass | Pass | Remove the USB gateway modem via the “Add/Remove Programs”. Shut down, and manually unplug the USB gateway modem and power up the system. Make sure that the USB gateway modem does not appear in the Device Manager and all related file being removed. |
| 1.8.7 | Plug and Plug repetition test | Pass | Pass | Pass | Pass | Plug in and out the USB gateway modem for 20 times. Make sure that the USB gateway modem is able to detect again by the system. |
| 1.8.8 | USB port re-detection test | Pass | Pass | Pass | Pass | First, plug in the Ethernet cable and make a connection to the ISP, then unplug the Ethernet cable and plug in the USB cable. Make sure that the USB gateway modem does not recognize as ‘Unknown Device’. |
| 1.8.9 | USB device redetection test | Pass | Pass | Pass | Pass | Interchanging the USB gateway modem with another USB device, example, Intel USB camera. Make sure that the reinstallation of driver is not needed. |

1.9	Miscellaneous Configuration Test for DSL 305EU

| Test Title |  | Win98SE | WinME | Win2000 | WinXP | Test Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| 1.9.1 | WAN side HTTP server | Pass | Pass | Pass | Pass | When this option enable, user from other system can access the modem by entering the WAN address of the modem. |
| 1.9.2 | FTP server | Pass | Pass | Pass | Pass | When this option enable, the FTP connection can be established from both the LAN and WAN sides. |
| 1.9.3 | TFTP server | Pass | Pass | Pass | Pass | When this option enable, it can upgrade the image code with TFTP client application run either the LAN or WAN sides. |
| 1.9.4 | DMZ | Not Tested | Not Tested | Not Tested | Not Tested |  |
| 1.9.5 | DHCP Relay | Not Tested | Not Tested | Not Tested | Not Tested |  |
| 1.9.6 | IGMP Proxy | Not Tested | Not Tested | Not Tested | Not Tested |  |
| 1.9.7 | PPP reconnect on WAN access | Not Tested | Not Tested | Not Tested | Not Tested |  |
| 1.9.8 | PPP Half bridge | Pass | Pass | Pass | Pass | When connected to ISP in this mode, must be able to surf the Internet. |
| 1.9.9 | Virtual Server | Pass | Pass | Pass | Pass | Any local PCs must be able to be mapped to the UDP/TCP port on WAN side. |
| 1.9.10 | Bridge Filtering | Not Tested | Not Tested | Not Tested | Not Tested |  |

1.10	Application Compatibility

| Test Title |  | Win98SE | WinME | Win2000 | WinXP | Test Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| 1.10.1 | PPP over ATM (RFC 2364) | Pass | Pass | Pass | Pass | No software is required because the firmware is able to connect automatically. Connect against SingNet Broadband. |
| 1.10.2 | PPP over Ethernet (RFC 2516) | Pass | Pass | Pass | Pass | No software is required because the firmware is able to connect automatically. Connect against SingNet Broadband. |
| 1.10.3 | Bridged Ethernet over ATM (RFC 1483) | Pass | Pass | Pass | Pass | Using WinPoET software. Connect against SingNet Broadband. |
| 1.10.4 | Classical IP over ATM (RFC 1577) | Pass | Pass | Pass | Pass | Static IP therefore no software needed to connect. Connect against SingTel Magix. |
| 1.10.5 | RFC 1483 Routed | Pass | Pass | Pass | Pass | Static IP therefore no software needed to connect. Connect against SingTel Magix. |
| 1.10.6 | PC Anywhere | Not Tested | Not Tested | Not Tested | Not Tested | Use this software to test the ADSL modem compatibility. |

1.11	Gaming test

| Test Title |  | Win98SE | WinME | Win2000 | WinXP | Test Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| #### 1.11.1 | #### StarCraft | Not Tested | Not Tested | Not Tested | Not Tested | The gaming can be conducted with Internet Players |
| #### 1.11.2 | #### Half-Life | Pass | Pass | Pass | Pass | The gaming can be conducted with Internet Players |

For game tests, the TEST would failed if:

- The gaming party using ADSL disconnect repeatedly

- One of the gaming parties cannot connect to same game server

Note: The test is done in PPPoA Encapsulation protocol, router mode.

1.12	Netmeeting Test

| Test Title |  | Win98SE | WinME | Win2000 | WinXP | Test Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| 1.12.1 | PPP over ATM (RFC 2364) | Pass | Pass | Pass | Pass | Setup the Netmeeting on local PC, and try to make a call to remote PC. See whether the connection can be established, or local PC is able to view remote PC or not. Connect against SingNet Broadband. |
| 1.12.2 | PPP over Ethernet (RFC 2516) | Pass | Pass | Pass | Pass | Setup the Netmeeting on local PC, and try to make a call to remote PC. See whether the connection can be established, or local PC is able to view remote PC or not. Connect against SingNet Broadband. |
| 1.12.3 | Bridged Ethernet over ATM (RFC 1483) | Pass | Pass | Pass | Pass | Setup the Netmeeting on local PC, and try to make a call to remote PC. See whether the connection can be established, or local PC is able to view remote PC or not. Connect against SingNet Broadband. Using WinPoET to dial to ISP. |
| 1.12.4 | Classical IP over ATM (RFC 1577) | Pass | Pass | Pass | Not Tested | Setup the Netmeeting on local PC, and try to make a call to remote PC. See whether the connection can be established, or local PC is able to view remote PC or not. Connect against SingTel Magix. |
| 1.12.5 | RFC 1483 Routed | Pass | Pass | Pass | Not Tested | Setup the Netmeeting on local PC, and try to make a call to remote PC. See whether the connection can be established, or local PC is able to view remote PC or not. Connect against SingTel Magix. |
| 1.12.6 | Half Bridge | Pass | Pass | Pass | Pass | Setup the Netmeeting on local PC, and try to make a call to remote PC. See whether the connection can be established, or local PC is able to view remote PC or not. |

For Netmeeting tests, the TEST would fail if:

- The connection can’t be established at all