| System Unit | Dimensions (HxWxD) With Base |  |  | 18.63 x 10.63 x 22.67 in/47.32 x 27.0 x 57.58 cm |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weight |  |  | 63 lbs/28.35 kg (without hard drives) |  |  |  |  |  |  |  |  |
|  | Input Requirements |  |  |  |  |  |  |  |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |  |  |  |  |  |  |
| Range Line Voltage | 90-132 VAC/180-265 VAC |  |  |  |  |  |  |  |  |  |  |  |
|  | Nominal Line Voltage |  |  | 100-120 VAC/220-240 VAC |  |  |  |  |  |  |  |  |
|  | Line Frequency |  |  | 50-60 Hz |  |  |  |  |  |  |  |  |
|  | Heat |  |  | 1706 BTU |  |  |  |  |  |  |  |  |
|  | Steady-State Input Power |  |  | 500 W |  |  |  |  |  |  |  |  |
|  | Output |  |  | 325 W |  |  |  |  |  |  |  |  |
|  | Temperature Range (noncondensing) |  |  |  |  |  |  |  |  |  |  |  |
|  | Operating |  |  | 50° to 93°F/10° to 35° C |  |  |  |  |  |  |  |  |
|  | Shipping |  |  | -22° to 140° F/-30° to 60° C |  |  |  |  |  |  |  |  |
|  | Relative Humidity (noncondensing) |  |  |  |  |  |  |  |  |  |  |  |
|  | Operating |  |  | 20% to 80% |  |  |  |  |  |  |  |  |
|  | Nonoperating |  |  | 5% to 90% |  |  |  |  |  |  |  |  |
|  | Acoustic Noise IDLE (Fixed Disk Drives Spinning) OPERATING (Random Seeks to Fixed Disks) |  |  | NPEL (BELS) 6.0 6.1 |  |  |  | AVERAGE SPL (dBA) 43 44 |  |  |  |  |
| 1.44-MB Diskette Drive | Diskette Size |  |  | 3.5-in |  |  |  |  |  |  |  |  |
|  | LED Indicators (front panel) |  |  | Green |  |  |  |  |  |  |  |  |
|  | Read/Write Capacity per Diskette (high/low density) |  |  | 1.44 MB/720 KB |  |  |  |  |  |  |  |  |
|  | Drive Supported |  |  | One |  |  |  |  |  |  |  |  |
|  | Drive Height |  |  | One-third |  |  |  |  |  |  |  |  |
|  | Drive Rotation |  |  | 300 rpm |  |  |  |  |  |  |  |  |
|  | Transfer Rate (high/low) |  |  | 500/250 Kb/s |  |  |  |  |  |  |  |  |
|  | Bytes/Sector |  |  | 512 |  |  |  |  |  |  |  |  |
|  | Sectors/Track (high/low) |  |  | 18/9 |  |  |  |  |  |  |  |  |
|  | Tracks/Side (high/low) |  |  | 80/80 |  |  |  |  |  |  |  |  |
|  | Access Times |  |  |  |  |  |  |  |  |  |  |  |
|  | Track-to-Track (high/low) |  |  | 3/6 ms |  |  |  |  |  |  |  |  |
|  | Average (high/low) |  |  | 169/94 ms |  |  |  |  |  |  |  |  |
|  | Settling Time |  |  | 15 ms |  |  |  |  |  |  |  |  |
|  | Latency Average |  |  | 100 ms |  |  |  |  |  |  |  |  |
|  | Cylinders (high/low) |  |  | 80/80 |  |  |  |  |  |  |  |  |
|  | Read/Write Heads |  |  | Two |  |  |  |  |  |  |  |  |
| Pluggable Wide-Ultra SCSI-3 10,000 rpm Hard Drive | Capacity Height |  |  | 4.3-GB 4293.6 MB Third, 1.0 inch |  |  |  | 9.1-GB 9100.0 MB Half, 1.6 inch |  |  |  |  |
|  | Size |  |  | 3.5 in |  |  |  | 3.5 in |  |  |  |  |
|  | Interface |  |  | Wide-Ultra SCSI-3 |  |  |  | Wide-Ultra SCSI-3 |  |  |  |  |
| Pluggable Wide-Ultra SCSI-3 10,000 rpm Hard Drive (cont’d) | Transfer Rate Sector Interleave |  |  | 4.3-GB 40 MB/s 1:1 |  |  |  | 9.1-GB 40 MB/s 1:1 |  |  |  |  |
|  | Seek Time (typical, including settling) |  |  |  |  |  |  |  |  |  |  |  |
|  | Single Track |  |  | 0.8 ms |  |  |  | 0.8 ms |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 7.5 ms |  |  | 8.0 ms |  |  |  |  |  |  |  |  |
|  | Full Stroke |  |  | 18.0 ms |  |  |  | 18.0 ms |  |  |  |  |
|  | Rotational Speed |  |  | 10,000 rpm |  |  |  | 10,000 rpm |  |  |  |  |
|  | Physical Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | Cylinders |  |  | 4,811 |  |  |  | 4,811 |  |  |  |  |
|  | Heads |  |  | 11 |  |  |  | 23 |  |  |  |  |
|  | Sectors/Track |  |  | 126 to 207 |  |  |  | 126 to 207 |  |  |  |  |
|  | Bytes/Sector |  |  | 512 |  |  |  | 512 |  |  |  |  |
|  | Logical Blocks |  |  | 8,386,000 |  |  |  | 17,773,500 |  |  |  |  |
|  | Operating Temperature |  | 50° to 86°F/ 10° to 30°C |  |  | 50° to 86°F/ 10° to 30°C |  |  |  |  |  |  |
| Wide-Ultra SCSI-3 Hard Drives | Capacity |  | 2.1-GB 2097.4 MB |  |  | 4.3-GB 4293.6 MB | 9.1-GB 9100.0 MB |  | 18.2-GB 18209.8 MB |  |  |  |
|  | Height |  | Third, 1.0 inch |  |  | Third, 1.0 inch | Half, 1.6 \inch |  | Half, 1.6 inch |  |  |  |
|  | Size |  | 3.5 in |  |  | 3.5 in | 3.5 in |  | 3.5 in |  |  |  |
|  | Interface |  | Wide-Ultra SCSI-3 |  |  | Wide-Ultra SCSI-3 | Wide-Ultra SCSI-3 |  | Wide-Ultra SCSI-3 |  |  |  |
|  | Transfer Rate |  | 40 MB/s |  |  | 40 MB/s | 40 MB |  | 40 MB |  |  |  |
|  | Sector Interleave Seek Time (typical, including settling) |  | 1:1 |  |  | 1:1 | 1:1 |  | 1:1 |  |  |  |
|  | Single Track |  | 1.9 ms |  |  | 1.0 ms | 1.0 ms |  | 2.0 ms |  |  |  |
|  | Average |  | 9.5 ms |  |  | 7.9 ms | 7.9 ms |  | 8.5 ms |  |  |  |
|  | Full Stroke |  | 18.5 ms |  |  | 19.0 ms | 19.0 ms |  | 19.0 ms |  |  |  |
|  | Rotational Speed |  | 7200 rpm |  |  | 7200 rpm | 7200 rpm |  | 7200 rpm |  |  |  |
|  | Physical Configuration |  |  |  |  |  |  |  |  |  |  |  |
|  | Cylinders |  | 5178 |  |  | 4,811 | 4,811 |  | 4,811 |  |  |  |
|  | Heads |  | 5 |  |  | 11 | 23 |  | 23 |  |  |  |
|  | Sectors/Track |  | 119 to 186 |  |  | 126 to 207 | 126 to 207 |  | 126 to 207 |  |  |  |
|  | Bytes/Sector |  | 512 |  |  | 512 | 512 |  | 512 |  |  |  |
| 17,773,500 | Logical Blocks |  |  | 4,110,000 |  |  |  |  |  |  |  | 8,386,000 |
| 50° to 95°F/ 10° to 35°C17,773,500 | Operating Temperature |  |  | 50° to 95°F/ 10° to 35°C |  |  |  |  |  |  |  |  |
| 50° to 95°F/ 10° to 35°C50° to 95°F/ 10° to 35°C |  |  |  |  |  |  |  |  |  |  |  |  |
| 16X MAX CD-ROM Drive IDE | Disk Applicable Disk |  |  | CD-ROM (Mode 1 and 2), CD-DA, CD-XA (Mode 2, Form 1 and 2), Photo CD (Single and Multiple Session), Mixed Mode (Audio and Data Combined) |  |  |  |  |  |  |  |  |
|  | Capacity |  |  | 550 MB (Mode 1, 12 cm), 640 MB (Mode 2, 12 cm) |  |  |  |  |  |  |  |  |
|  | Block Size |  |  |  | 2048, 1024 bytes (Mode 1), 2340, 2336, 1024 bytes (Mode 2), 2352 bytes (CD-DA), 2328 bytes (CD-XA) |  |  |  |  |  |  |  |
| 16X MAX CD-ROM Drive IDE (cont’d) | Disk (cont’d) Diameter Rotational Speed Center Hole Thickness Track Pitch |  |  |  | 4.7 in, 3.15 in / 12 cm, 8 cm 4200 rpm maximum .6 in/15 mm diameter 1.2 mm 1.6 µm |  |  |  |  |  |  |  |
|  | Interface |  |  |  | IDE (ATAPI) |  |  |  |  |  |  |  |
|  | Access Times (typical) |  |  |  |  |  |  |  |  |  |  |  |
|  | Random |  |  |  | 100 ms |  |  |  |  |  |  |  |
|  | Full Stroke |  |  |  | 200 ms |  |  |  |  |  |  |  |
|  | Audio Output Level |  |  |  |  |  |  |  |  |  |  |  |
|  | Line Out |  |  |  | 0.7 VRMS at 47 kOhms |  |  |  |  |  |  |  |
|  | Headphone |  |  |  | 0.6 VRMS at 32 Ohms (maximum volume) |  |  |  |  |  |  |  |
|  | Data Transfer Rate |  |  |  |  |  |  |  |  |  |  |  |
|  | Sustained |  |  |  | 150 KB/s (single) |  |  |  |  |  |  |  |
|  | Burst |  |  |  | 1200 - 2400 MB/s |  |  |  |  |  |  |  |
|  | Cache Buffer |  |  |  | 128 KB |  |  |  |  |  |  |  |
|  | Start-Up Time (typical) |  |  |  | Less than 7 seconds |  |  |  |  |  |  |  |
|  | Stop Time |  |  |  | Less than 4 seconds |  |  |  |  |  |  |  |
|  | Laser Parameters |  |  |  |  |  |  |  |  |  |  |  |
|  | Type |  |  |  | Semiconductor Laser GaA1As |  |  |  |  |  |  |  |
|  | Wave Length |  |  |  | 780 ± 25 nm |  |  |  |  |  |  |  |
|  | Divergence Angle |  |  |  | 53.5° ± 1.5° |  |  |  |  |  |  |  |
|  | Output Power |  |  |  | .14 mW |  |  |  |  |  |  |  |
|  | Operating Conditions |  |  |  |  |  |  |  |  |  |  |  |
|  | Temperature |  |  |  | 41 - 113°F / 5 - 45°C |  |  |  |  |  |  |  |
|  | Humidity |  |  |  | 10 - 80% |  |  |  |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |  |  |  |  |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (HxWxD, maximum) |  |  |  | 1.69 x 5.75 x 8.19 in/4.29 x 15.0 x 20.8 cm |  |  |  |  |  |  |  |
|  | Weight |  |  |  | 2.66 lbs/1.2 kg |  |  |  |  |  |  |  |
| Integrated Wide-Ultra SCSI-3 Controller | Drives Supported Data Transfer Method SCSI Channel Transfer Rate |  |  |  | Up to seven SCSI-2 devices supported 32-Bit PCI bus master 40 MB/s |  |  |  |  |  |  |  |
|  |  | Maximum Transfer Rate per PCI Bus |  |  | 133 MB/s |  |  |  |  |  |  |  |
|  |  | SCSI Termination |  |  | Active Termination |  |  |  |  |  |  |  |
|  |  | SCSI Connectors |  |  | 1 internal or external (68-pin) |  |  |  |  |  |  |  |
|  |  | Logical Drives Supported |  |  | 32 |  |  |  |  |  |  |  |

Compaq Netelligent
10/100 TX UTP ControllerNetwork Interface10Base-T/100Base-TXData Transfer Method32-bit bus-master PCI

| Compaq Netelligent | Network Transfer Rate |  |
| --- | --- | --- |
| 10/100 TX UTP Controller | 10Base-T (Half-Duplex), 10Base-2 | 10 Mb/s |
| (cont’d) | 10Base-T (Full-Duplex) | 20 Mb/s |
|  | 100Base-TX (Half-Duplex) | 100 Mb/s |
|  | 100Base-TX (Full-Duplex) | 200 Mb/s |

|  | Connector Cable Support 10Base-T | RJ-45 and AUI Categories 3, 4, or 5 UTP (2 or 4 pair); up to 100 meters (328 feet) |
| --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |
| 100Base-TX | Category 5 UTP (2 pair); up to 100 meters (328 feet) |  |
|  | OS Driver Support |  |
| Novell NetWare Server v 3.12, 4.1x, IntranetWare, SFSFT III, Novell NetWare DOS IBM LAN Server NDIS 3.0: Microsoft Windows NT, Windows 95, Windows for Workgroups SCO Unix |  |  |

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

Compaq, Smart Start, Compaq Insight Manager, ProLiant, ProSignia, registered United States Patent and Trademark Office. Netelligent, SmartQ, ProSignia, MultiLock, QuickBlank, QuickLock, are trademarks and /or service marks of Compaq Computer Corporation.

Novell and NetWare are registered trademarks of Novell, Inc. Microsoft, Windows, Windows NT and SQL Server are trademarks of Microsoft Corporation. IBM and OS/2 are registered trademarks of IBM Corporation. SCO is a registered trademark of the Santa Cruz Operation, Inc. UNIX is a registered trademark of the Open Group in the US and other countries, Inc. ORACLE is a registered trademark and ORACLE7 is a trademark of Oracle Corporation. Cheyenne and ARCserve are registered trademarks of Cheyenne Software, Inc. JETserve is a trademark of Cheyenne Software, Inc.

Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1997 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A.

IEEE 802.3/802.3u compliant

Compatibility