| System Unit | Dimensions (HxWxD) | 2.0 x 12.6 x 10.2 in/50.8 x 320 x 259 mm |  |
| --- | --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Weight (modem installed) | 13.3² | 14.1² |  |
|  | CD-ROM configured |  |  |
|  | Without Power Cord | 8.7 lb/4.0 kg | 8.8 lb/4.0 kg |
|  | With Power Cord (total carrying weight) | 9.0 lb/4.2 kg | 9.2 lb/4.2 kg |
|  | FDD configured |  |  |
|  | Without Power Cord | 8.4 lb/3.8 kg | 8.5 lb/3.8 kg |
|  | With Power Cord (total carrying weight) | 8.7 lb/4.0 kg | 8.8 lb/4.0 kg |
|  | Weight Saver configured |  |  |
|  | Without Power Cord | 7.9 lb/3.6 kg | 8.0 lb/3.6 kg |
|  | With Power Cord (total carrying weight) | 8.2 lb/3.7 kg | 8.3 lb/3.8 kg |
|  | Weight (no modem installed) |  |  |
|  | CD-ROM configured |  |  |
|  | Without Power Cord | 8.7 lb/3.9 kg | 8.8 lb/4.0 kg |
|  | With Power Cord (total carrying weight) | 8.9 lb/4.1 kg | 9.1 lb/4.2 Kg |
|  | FDD configured |  |  |
|  | Without Power Cord | 8.4 lb/3.8 kg | 8.5 lb/3.8 kg |
|  | With Power Cord (total carrying weight) | 8.7 lb/4.0 kg | 8.8 lb/4.0 kg |
|  | Weight Saver configured |  |  |
|  | Without Power Cord | 7.8 lb/3.6 kg | 7.9 lb/3.6 kg |
|  | With Power Cord (total carrying weight) | 8.1 lb/3.7 kg |  |
| 8.2 lb/3.7 kg | Stand-Alone Power Requirements |  |  |
|  | Average Operating Power | 20 W |  |
|  | Peak Operating Power | 28 W |  |
|  | Power Supply |  |  |
|  | Rated Input Voltage | 100-120/220-240 VAC |  |
|  | Rated Input Current | 1.2/0.6A |  |
|  | Rated Frequency | 50-60 Hz |  |
|  | Temperature |  |  |
|  | Operating | 10 to 40°C |  |
|  | Nonoperating | -20 to 60°C |  |
|  | Relative Humidity |  |  |
|  | Operating | 10 to 90% relative humidity, non-condensing |  |
|  | Nonoperating | 5 to 90% relative humidity, 38.7°C Maximum wet bulb temperature |  |
|  | Shock |  |  |
|  | Operating | 10 G, 11 ms, half sine |  |
|  | Non operating | 240 G, 2 ms, half sine, 108 in/s, square wave |  |
|  | Vibration |  |  |
|  | Operating | .25 G, 50-500 Hz sine, 1/2 octave/minute sweep |  |
|  | Nonoperating | 1 G, 50-500 Hz sine, 1/2 octave/minute sweep |  |
|  | Maximum Altitude |  |  |
|  | Operating | 0 to 10,000 ft/3.05 km (14.7 to 10.1 psia) |  |
|  | Nonoperating | 0 to 30,000 ft/9.14 km (14.7 to 4.4 psia) |  |
| Display LCD | Dimensions Diagonal Size | 7.98 x 10.64 in/20.28 x 27.03 cm or 8.5 x 11.27 in/21.59 x 28.63 cm 13.3 in/33.79 cm or 14.1 in/35.81 cm |  |
|  | Mounting | Internal Panel Supports |  |

Number of ColorsUp to 16.8 million with dithering

| Display LCD (cont’d) | Contrast Ratio |  | 150:1 |
| --- | --- | --- | --- |
|  | Brightness |  | 130 nits |
|  | Pixel Resolution |  |  |
|  | Pitch |  | 0.264 x 0.264 mm |
|  | Format |  | 1024 x 768 |
|  | Configuration |  | RGB stripe |
|  | Backlight |  | CCFT (Cold Cathode Fluorescent Tube) |
|  | Character Display |  | 80 x 25 |
|  | Total Power Consumption (maximum) |  | 4.2 W |
| Hard Drives1 | Formatted Capacity SMART, Drive Fault prediction capable |  | 5.1231 GB or 8.1956 GB Yes |
|  | Drive Height |  | 19 mm |
|  | Transfer rate Media (5 GB) Interface (5 GB) Media (8 GB) Interface (8 GB) |  | 60.2 to 98.5 MB/s 16.6 MB/s 64.3 to 100.4 MB/s 16.6 MB/s |
|  | Seek times (typical including settling) |  |  |
|  | Track to track |  | 4 ms |
|  | Average |  | 12 ms |
|  | Maximum |  | 23 ms |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Disk Rotational Speed | 4902 rpm |  |  |
|  | Physical Configuration Cylinders (5 GB) Heads (5 GB) Sectors per track (5 GB) Bytes per sector (5 GB) Cylinders (8 GB) Heads (8 GB) Sectors per track (8 GB) Bytes per sector (8 GB) |  | 6975 8 137 to 225 512 8192 10 150 to 230 512 |
|  | Logical Configuration Cylinders Heads Sectors per track Bytes per sector Cylinders Heads Sectors per track Bytes per sector |  | 10592 15 63 512 15880 16 63 512 |
|  | Buffer size (5 GB) Buffer size (8 GB) |  | 468 KB 512 K total |
| Diskette Drive | Diskette Size |  | 3.5 inch |
|  | Light |  | on system |
|  | Height | .5 in/12.7 mm |  |
| Diskette Drive (cont’d) | Bytes per Sector | 512 |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Sectors per track High density Low density | 18 (1.44-MB)/15 (1.2-MB) 9 |  |  |
|  | Tracks per side |  |  |
|  | High density | 80 (1.44-MB)/80 (1.2-MB) |  |
|  | Low density | 80 |  |
|  | Read/Write heads | 2 |  |
|  | Access Times Track-to-track (high/low) Average (high/low) Settling time Latency average | 3 ms/6 ms 94 ms/174 ms 15 ms 100 ms |  |
| 20X Max2 CD-ROM Drive | Applicable Disc | CD-ROM (Mode 1, 2 and 3) CD-XA ready (Mode 2, Form 1 and 2) CD-I ready (Mode 2, Form 1 and 2) CD-R (read only) CD Plus Photo CD (single and multi-session) CD Extra Video CD CD-WO (fixed packets only) CD-Bridge |  |
|  | Center Hole Diameter | .59 in/15 mm |  |
|  | Disc Diameter | 12 cm, 8 cm |  |
|  | Disc Thickness | 1.2 mm |  |
|  | Track Pitch | 1.6 µm |  |
|  | Access Times (typical including settling) Random | Less than 170 ms |  |
|  | Full Stroke | Less than 600 ms |  |
|  | Audio Output Level |  |  |
|  | Line Out | .7 Vrms |  |
|  | Cache Buffer | 128 KB |  |
|  | Data Transfer Rates Sustained 1X Sustained 20X | 150 KB/s 3.0 MB/s (outer diameter) |  |
|  | Start up Time | Less than 10 seconds |  |

24X Max3 CD-ROM DriveApplicable DiscCD-ROM (Mode 1, 2 and 3)

| 24X Max3 CD-ROM Drive (cont’d) | Center Hole Diameter Disc Diameter | .59 in/15 mm 12 cm, 8 cm |  |
| --- | --- | --- | --- |
|  | Disc Thickness | 1.2 mm |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Track Pitch | 1.6 µm |  |  |
|  | Access Times (typical including settling) Random | 150 ms |  |
|  | Full Stroke | 300 ms |  |
|  | Audio Output Level |  |  |
|  | Line Out | 0.8+/- .27 Vrms |  |
|  | Cache Buffer | 128 KB |  |
|  | Data Transfer Rates Sustained 1X Sustained 24X Normal PIO Mode 4 (single burst) | 150 KB/s 3600 KB/s 16.66 MB/s |  |
|  | Start up Time | Less than 8 seconds |  |
| Li-Ion Battery Pack4 | Dimensions | .92 x 3.46 x 5.44 in/2.34 x 8.80 x 13.82 cm |  |
|  | Weight | .89 in/.40 Kg |  |
|  | Energy Voltage Amp-hour capacity Watt-hour capacity | 14.4 V 2.7 Ah 38.8 W |  |
|  | Performance Single battery life | over 2 hours |  |
|  | Temperature Operating | 50° to 104°F/10° to 40°C |  |
|  | Nonoperating - recommended storage |  | -12° to 140°F/-30° to 60°C |

1	Hard drive specifications may vary slightly depending on vendor.

2	Data transfer rates will vary from 150 KB/s to 3000 KB/s.

3	Data transfer rates will vary from 150 KB/s to 3600 KB/s.

4	Actual results may vary with individual usage. When tested in standard configuration, using the Ziff-Davis BatterMark Test version 2.0, 
the battery life of the Armada 7800 will range up to 2.3 hours. Standard configuration at time of announcement (4/2/98) is considered to be one battery, a 1024 x 768 (256 colors) panel set at 50% brightness, 64-MB RAM and no drives other than the hard drive active.

Compaq, registered United States Patent and Trademark Office. Armada, ArmadaStation, Ministation, MultiLock, QuickBlank, QuickLock, 
are trademarks and/or service marks of Compaq Computer Corporation.

Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A.

CD-XA ready (Mode 2, Form 1 and 2)

CD-I ready (Mode 2, Form 1 and 2)

CD-R (read only)

CD Plus

Photo CD (single and multi-session)

CD Extra

Video CD

CD-WO (fixed packets only)

CD-Bridge