***********************************************************************

Windows NT Device Drivers Version 1.51                    June 19, 1995

***********************************************************************

READ THIS FILE COMPLETELY BEFORE ATTEMPTING TO USE THIS PRODUCT !

*****************************

Contents

1.0     Introduction

1.1     Supported Resolutions

2.0     Installation Notes for Windows NT

2.1     Uninstalling the Windows NT drivers

3.0     Known Problems

4.0     Reporting Problems

*****************************

1.0     Introduction

The Graphics device drivers supplied here are for the Microsoft Retail

Release of the Windows NT 3.5 operating system.

The drivers will substitute 640x480 by 256 colors if the mode selected is

NOT supported by the mach64 graphics hardware.

*****************************

1.1     Supported Resolutions.

256 colors    65000 colors    16.7 million   TRUE color

Resolutions    8 bpp         16 bpp          24 bpp         32bpp *

640 x  480     2/4MB         2/4MB           2/4MB          2/4MB

800 x  600     2/4MB         2/4MB           4MB            2/4MB

1024 x  768     2/4MB         2/4MB           4MB            4MB

1152 x  864     2/4MB         2/4MB           4MB            4MB

1280 x 1024     2/4MB         4MB             4MB            N/A

1600 x 1200     2/4MB         4MB             N/A            N/A

2/4MB  -  Supported on 2 or 4 Megabyte mach64 products

4MB    -  ONLY supported on 4 Megabyte mach64 products

N/A    -  Not Supported

* Note:   TRUE color, provides 16.7 million colors using a 32 bpp data

format.  TRUE color provides higher performance but uses more

graphic memory.

** Note:  Not all color depths are supported at all refresh rates.

** Note: 1600x1200 not available on all DAC types.

****************************

2.0     Installation Notes for Windows NT 3.5

This driver package containing:

- ATI.SYS        (ATI Graphics miniport driver)

- ATI.DLL        (ATI Graphics display  driver)

- OEMSETUP.INF   (setup files used by Windows NT Setup program)

- TXTSETUP.OEM

- ATI_VID

- This README file

Procedure:

1. Run the Windows NT Display program located in the Control Panel, Main

group.

2. Select Change Display Type.

3. Select Change... from the display options.

4. Select Other...

5. NT will prompt you for the correct path, enter A:\ where A: is

the drive containing the diskette.

6. A list of all Graphics Accelerator drivers will appears, select

ATI Compatible Driver.

7. Restart Windows NT.  Windows NT will start up using the mach64 drivers.

8. NT Will boot into 640x480 mode and start the Display applet allowing

for mode selection.

Note: If you select a color depth or resolution, not supported by your

card, NT will substitute 640x480, 256 color for the display.

2.1     Uninstalling the Windows NT drivers

You can not uninstall the Mach64 driver.

*****************************

3.0     Known Problems

Some applications do not handle 24bpp and 32bpp properly resulting in the

colors not looking right.

What to do if  ACCELERATOR not found message appears:

Is there another adapter card in the system that CONFLICTS with I/O

locations in the 350-35F hex range?  Relocate the I/O to 300 to 31F range.

*****************************

4.0     Reporting Problems

If you experience any difficulties, open the file REPORT.TXT and complete

the Problem Report form.