July 1992

D-Link 8-Bit DE-100 Series Ethernet Card

Test Procedure

========================================

The following files will be used to test your DE-100/DE-100+ card:

Files           Contents

-----------     -------------------------

TE100.EXE       DE-100 series Test Program

TE100PL.EXE     DE-100+ series Test Program

DE-100.EXE      DE-100 series Driver Program

The TE100.EXE/TE100PL.EXE program can be run to see if your DE-100/DE-100+

card has any problems.  The procedure is as follows:

1. Copy DE-100 driver program DE-100.EXE into the directory where your

test program TE100.EXE/D100PL.EXE resides.

2. Adjust switches of your DE-100/DE-100+ card to the proper position and

insert the DE-100/DE-100+ card in your computer. See your DE-100/DE-100+

card's installation Guide for details.

3. Execute the TE100.EXE/TE100PL.EXE program on your computer.

4. The test program will show the default jumper settings on the screen.

If your DE-100/DE-100+ card's jumper settings are different from default

jumper settings, change the jumper settings on the screen to match your

DE-100/DE-100+ card's jumper settings.

Enter a name for reference. This name is any name of up to 8

characters long, used to represent your computer.

Press RETURN.  The DE-100/DE-100+ card's driver program will be loaded in

from the current directory and the test begins.

5. Execute an appropriate D-Link test program on at least one

other computer on the network.  Example: if that computer uses a

DE-100 series card, run TE100.EXE on that computer.  If that computer

uses a DE-150 card, run TE150.EXE on that computer.

6. If the network interface card is OK, the receive count will

steadily increase.

7. You can press F1 to reset receive count or ESC to terminate the test.

If the Test Program Fails

=========================

If test program fails, it does not necessarily mean that the

network interface card is defective. The failure may be due to

one or more jumper incorrect settings or to settings that pose

conflicts with other add-on cards present in your computer. The

failure may also be due to a defective trunk cable. Finally, the

failure may be caused simply by not inserting the network

interface card correctly in the computer.

The following lists the symptoms and their most likely causes.

Follow the remedies listed and retry the test program.

----------------------------------------------------------------

Symptom               Cause/Remedy

----------------------------------------------------------------

1.Different network   You have loaded a different driver program

interface program   before running the test program.

installed.

Reset your computer.

Re-run the test program.

2.Load execute file   The test program and its driver program are

name not found.     not at the same directory.

Copy the test program and its driver

program at the same directory.

Re-run the test program.