July 1992

DECnet DOS Driver Installation

for

D-Link 8-Bit DE-100 Series Ethernet Card

=========================================

The D-Link DECnet DOS Driver program is designed to let you run

the DECnet DOS version 2.0 on D-Link Ethernet interface cards.

With the DECnet DOS Driver program, your D-Link Ethernet cards

will function like a 3COM EtherLink card running DECnet DOS.

The following file will be used in this installation:

Files          Contents

-----------    -------------------------

DLL100.EXE     DECnet DOS Driver Program

Installation

1. Follow the DECnet DOS software installation procedure for

the 3COM EtherLink card (3C501).

2. Place this driver program diskette in drive A.

3. Change to the directory that DECnet DOS is installed.

A > CD\DECNET

4. Replace the contents of the DECnet DataLink Layer program

DLL.EXE on your boot disk with the contents of the D-Link

driver program DLL.EXE, by executing the following

command:

> COPY A:\DECNET\DLL100.EXE

You may now start your DECnet DOS.