July 1992

AT&T UNIX DRIVER PROGRAMS

for

D-Link DE-100 Series & DE-200 Series Ethernet Cards

======================================================

The following files will be used to in this installation:

Files           Contents

-----------     ----------------------

DE100.o         For DE-100 Driver

DE200.o         For DE-200 Driver

DLK

STRCF

Installation and configuration consist of the following steps:

1.  Login as the root user.

2.  Installing TCP/IP software first.

3.  If the install diskette has DOS format

Copy D-Link driver through the doscp utility

#mkdir /usr/lib/tcprt/dlk

#doscp a:/at&tunix/de100.o  /usr/lib/tcprt/dlk/de100.o

#doscp a:/at&tunix/de200.o  /usr/lib/tcprt/dlk/de200.o

#doscp a:/at&tunix/dlk  /usr/lib/tcprt/dlk/dlk

#doscp a:/at&tunix/strcf  /usr/lib/tcprt/dlk/strcf

#doscp a:/at&tunix/readme.doc  /usr/lib/tcprt/dlk/readme.doc

#chmod +x /usr/lib/tcprt/dlk/dlk

4.  Installing D-Link Ethernet driver

#cd /usr/lib/tcprt/dlk

#./dlk

Do you wish to install or delete the dlk driver?  type i to install

A display of information about driver.            type y to continue

Select D-link ethernet adapter (de100 or de200)   select one

Select interrupt vector                           value

Select I/O base address                           hexadecimal value

Select memory base address                        hexadecimal value

Do you want relink the kernel?                    type y to relink

Do you want this kernel to boot by default?       type y to default

Do you want the kernel environment rebuilt?       type y to rebuilt

Do you want to modify /etc/strcf?                 type y

5.  Reboot unix

#cd /

#shutdown -y -g0 -i6