MS Windows for Workgroups 3.11 Using a D-Link Card:

��You will need:���������������������������������������������������Ŀ

�MS Windows for Workgroups 3.11 Disks.                              �

�WWG311.EXE archive from D-Link (until the drivers get included on  �

�                                            our card driver disks.)�

�WINUP9.EXE archive from Novell (ONLY if you use a Novell Server.)  �

�                                                                   �

�The WWG311.EXE & WINUP9.EXE files can be found on D-Link's BBS.    �

�  (714) 455-1779 & (714) 455-9616                                  �

�  9600-2400 baud, 8 bit, No Parity, 1 Stop bit, Full Duplex        �

���������������������������������������������������������������������

��What step to start with:�����������������������������������Ŀ

�If you use a Novell Netware Server, start at Step 1.         �

�If you DON'T use a Novell Netware Server, start at Step 2.   �

�If you already installed MS Windows for Workgroups 3.11, then�

�  1. Run (click on) the Network Setup icon in Windows.       �

�  2. Start at Step 3.                                        �

���������������������������������������������������������������

Step 1 If you will be using Novell with Windows for Workgroup, then

you MUST setup your computer to LOGIN to the NetWare Server

with ODI drivers BEFORE you start installing Windows.

Instructions, for using ODI drivers, are located in the

A:\NETWARE\README.DOC file on your network card's driver disk.

Step 2 Start installing Windows.

A:\>SETUP

The Network settings come later in the setup process.  Follow

the Windows manuals for help until you get to the Network

setup screens.

Step 3 You will see this screen:

��������������������������������NETWORK SETUP��������������������������������ͻ

���Network Settings:������������������������������������������������Ŀ        �

��   Setup will install Windows support for the following  [Networks]�        �

��   network(s) on your computer:                                    �        �

��     Microsoft Windows Network (version 3.11)                      �        �

��                                                                   �        �

��                                                                   �        �

��   You can share your files and printers with others.    [Sharing ]�        �

��                                                                   �        �

��  �Network Drivers:����������������������������Ŀ        [Drivers ]�        �

��  �  NE2000 Compatible [ODI/NDIS3]              �                  �        �

��  �    Microsoft NetBEUI                        �                  �        �

��  �    IPX/SPX Compatible Transport with NetBIOS�                  �        �

��  �����������������������������������������������                  �        �

����������������������������������������������������������������������        �

�����������������������������������������������������������������������������ͼ

Notes: The Network Driver can be changed with the DRIVERS button.

If you are using a Novell Server, a card that isn't NE2000

compatible, or the card name displayed is wrong, then

press the DRIVERS button and do Step 4.

Step 4 The DRIVERS button was pressed from Step 3:

����������������������������NETWORK DRIVERS����������������������������������ͻ

���Network Drivers:�����������������������������������Ŀ      [Cancel      ]  �

�����������������������������������������������������Ŀ�      [Add Adapter ]  �

���(This will show the Card and Protocols)            ��      [Add Protocol]  �

���(This may be blank at this point)                  ��      [Remove      ]  �

������������������������������������������������������ٳ      [Setup       ]  �

��               [Set As Default Protocol]             �      [Help        ]  �

��������������������������������������������������������                      �

���Default Protocol�����������������������������������Ŀ                      �

��  Microsoft NetBEUI                                  �                      �

��������������������������������������������������������                      �

�����������������������������������������������������������������������������ͼ

Notes: If the Network Drivers box doesn't show the right card, then press

the REMOVE button, then press the ADD ADAPTER button (Step 5).

If you are using a Novell Server (requires ODI), then press the

SETUP button (Step 7).

Step 5 The ADD ADAPTER button was pressed from Step 4:

�����������������������������ADD NETWORK ADAPTER�����������������������������ͻ

� Select a Network Adapter to Install:                                        �

� ������������������������������������������������������������Ŀ    [  OK  ]  �

� �Unlisted or Updated Network Adapter                         �    [Cancel]  �

� �3Com EtherLink 16                                           �    [Detect]  �

� �3Com EtherLink II                                           �    [Help  ]  �

� �.                                                           �              �

� �.                                                           �              �

� ��������������������������������������������������������������              �

�����������������������������������������������������������������������������ͼ

Notes: If you have downloaded the D-Link Drivers (WWG311.EXE, as discussed at

the top of this document), then choose 'Unlisted' and go to step 6.

If you have a D-Link card that is compatible to the NE-2000 card

(DE-200 or DE-220), then you can choose 'Novell/Anthem NE2000' or

'NE2000 Compatible' and go to step 7.

Step 6 'Unlisted' was chosen from Step 5:

��������������������INSTALL DRIVER��������������������ͻ

� Insert unlisted, updated, or                [  OK  ] �

� vendor-provided network driver disk in:     [Cancel] �

� �����������������������������������������Ŀ [Browse] �

� �A:\                                      � [ Help ] �

� �������������������������������������������          �

������������������������������������������������������ͼ

Note: Change the 'A:\' to the directory that has the new drivers.

(ex: C:\WWG311)

��������UNLISTED OR UPDATED NETWORK ADAPTER�����������ͻ

� Network Adapters:                           [  OK  ] �

� �����������������������������������������Ŀ [Cancel] �

� �D-Link DE-100                            � [ Help ] �

� �D-Link DE-150                            �          �

� �D-Link DE-200/DE-205                     �          �

� �.                                        �          �

� �.                                        �          �

� �������������������������������������������          �

������������������������������������������������������ͼ

Note: Choose your D-Link card from the list, and click on the OK button.

Step 7 Set the Hardware settings to match what the card is using:

����������������������������NETWORK DRIVERS����������������������������������ͻ

���Network Drivers:�����������������������������������Ŀ      [Cancel      ]  �

�����������������������������������������������������Ŀ�      [Add Adapter ]  �

���D-Link DE-200     (or NE2000 or other D-Link card) ��      [Add Protocol]  �

���   Microsoft NetBEUI                               ��      [Remove      ]  �

������������������������������������������������������ٳ      [Setup       ]  �

��               [Set As Default Protocol]             �      [Help        ]  �

��������������������������������������������������������                      �

���Default Protocol�����������������������������������Ŀ                      �

��  Microsoft NetBEUI                                  �                      �

��������������������������������������������������������                      �

�����������������������������������������������������������������������������ͼ

Note: Press the SETUP button.

������������������D-LINK (name of the card)����������������ͻ

���Hardware Settings����������������������������Ŀ [  OK  ] �

�� Interrupt:              3                     � [Cancel] �

�� Base I/O Port:          0x0300                � [ Help ] �

�� Base Memory Address:    (Automatic or Unused) �          �

��������������������������������������������������          �

���Driver Type����������������������������������Ŀ          �

�� Real Mode NDIS Driver                         �          �

��������������������������������������������������          �

�����������������������������������������������������������ͼ

Notes: If the Interrupt, Base I/O or Base Memory settings don't match the

settings on your card, then change them here.

If any of the lines say 'Automatic or Unused' then you don't have

to set that line.

If you use a Novell Server and have done Step 1, then change

the DRIVER TYPE to 'ODI Driver'.  It's a drop down list.

There are a few choices for the DRIVER TYPE:

Real Mode and Enhanced Mode NDIS Driver   (only some cards)

Enhanced Mode NDIS Driver                 (only some cards)

Real Mode NDIS Driver

ODI Driver

Step 8 If you are installing Windows for Workgroups for the first time, these

screens will come up:

���������������MICROSOFT WINDOWS NETWORK NAMES��������������ͻ

�   These names identify you...                       [ OK ] �

� User Name:     John                                 [Help] �

� Workgroup:     WORKGROUP                                   �

� Computer Name: John                                        �

������������������������������������������������������������ͼ

Note: Press the OK button.

Step 9 If you install using an ODI driver (for Novell), then you may get

this message box:

�������������NOVELL NETWARE (WORKSTATION SHELL 3.X)�������������ͻ

� The file NETWARE.DRV, which is required for this      [Browse] �

� component, is not included with this version of       [ Scan ] �

� Windows.                                              [ Help ] �

� The file may be on one of the disks provided by                �

� your vendor.  If you do not have this file, contact            �

� your vendor.                                                   �

� Enter a path where Setup can copy the file from:               �

� ������������������������������������������������������������Ŀ �

� �C:\WINDOWS                                                  � �

� �������������������������������������������������������������� �

�               [  OK  ]     [ Skip ]     [Cancel]               �

����������������������������������������������������������������ͼ

Notes: This screen repeats for NETWARE.HLP, NWPOPUP.EXE, VIPX.386 and

VNETWARE.386.  These are the files included in WINUP9.EXE

as discussed at the top of this document.

You will need to get these files before you can continue!

Examples:

CONFIG.SYS:

Adds: DEVICE=C:\WINDOWS\IFSHLP.SYS      (to the end of CONFIG.SYS)

LASTDRIVE=P

STACKS=9,256

AUTOEXEC.BAT:

Adds C:\WINDOWS\SMARTDRV.EXE            (to the top of AUTOEXEC.BAT)

SET TEMP=C:\WINDOWS\TEMP

C:\WINDOWS\NET START

C:\WINDOWS\ODIHLP.EXE         (If you use the Novell ODI drivers)

SETUP [/N] [/I] [/O:file] [/S:path] [/B] [/T] [/C] [/A] [/H:file] [/F]

/N       - Sets up a shared copy of Windows from a network server.

/I       - Ignores hardware detection.

/O:file  - Specifies the SETUP.INF file.

/S:path  - Specifies a path to the setup disk(s).

/B       - Use monochrome monitor on a CGA card.

/T       - Searches the drive for TSR programs?

/C       - Turns off the search for TSR programs.

/A       - Administrative setup.  Places ALL Windows files on a network

server, and marks them read-only.

/H:file  - Batch mode setup.  This file has system settings so the user

doesn't have to type anything.

/F       - Forces network card detection.

'WFW311.INS' by RCZJ on 5/9/94#