December 1992

D-Link DE-150 8-Bit Ethernet Card

3Com 3+ Share Driver Installation

=================================

The D-Link 3Com 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link DE-150 8-Bit Ethernet card.

The following files will be used in this installation:

Files            Contents

-----------      -------------------------

DE150PD.SYS      DE-150 Driver Program

ETH.SYS          3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link DE-150 8-Bit Ethernet card to the boot disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure in step 1

above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.  The order of these two

statement must be as shown below.

DEVICE = DE150PD.SYS <packet_int_no> <int_level> <mem_base> <io_addr>

DEVICE = ETH.SYS

You may now boot and start your 3COM workstation.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 3 if this parameter is not

specified.

<mem_base>      is base memory address set on your network interface card.

The default address is D000H, if this parameter is not

specified.

<io_base>       is base I/O address set on your network interface card.

The default address of 300H is used, if this parameter

is not specified.