DE1500-MIB DEFINITIONS ::= BEGIN

IMPORTS

mgmt, OBJECT-TYPE, NetworkAddress, IpAddress,

Counter, Gauge, TimeTicks

FROM RFC1155-SMI;

private    OBJECT IDENTIFIER ::= { internet 4 }

enterprises OBJECT IDENTIFIER ::= { private 1 }

d-link     OBJECT IDENTIFIER ::= { enterprises 171}

hubmib             OBJECT IDENTIFIER ::= {d-link 2}

basicGroup         OBJECT IDENTIFIER ::= {hubmib 1}

monitorGroup       OBJECT IDENTIFIER ::= {hubmib 2}

addrTrackGroup     OBJECT IDENTIFIER ::= {hubmib 3}

-- The D-Link Hub Basic Group 1.3.6.1.4.1.171.2.1

-- 1.3.6.1.4.1.171.2.1.1.0

basicHubMIBVersion OBJECT-TYPE

SYNTAX  DisplayString

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 1 }

-- 1.3.6.1.4.1.171.2.1.2.0

basicHubVender OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 2 }

-- 1.3.6.1.4.1.171.2.1.3.0

basicHubProduct OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 3 }

-- 1.3.6.1.4.1.171.2.1.4.0

basicHubVersion OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 4 }

-- 1.3.6.1.4.1.171.2.1.5.0

basicHubHealthState OBJECT-TYPE

SYNTAX  INTEGER {

other(1),         -- undefined or unknown

ok(2),            -- no known failures

hubFailure(3),    -- hub-related failure

groupFailure(4),  -- group-related failure

portFailure(5),   -- port_related failure

generalFailure(6) -- failure, unspecified type

}

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 5 }

-- 1.3.6.1.4.1.171.2.1.6.0

basicHubHealthText OBJECT-TYPE

SYNTAX  DisplayString

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 6 }

-- 1.3.6.1.4.1.171.2.1.7.0

basicHubHealthData OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 7 }

-- 1.3.6.1.4.1.171.2.1.8.0

basicHubReset OBJECT-TYPE

SYNTAX  INTEGER {

noReset(1),     -- do nothing

reset(2)        -- reset the hub

}

ACCESS  read-write

STATUS  mandatory

::= { basicGroup 8 }

-- 1.3.6.1.4.1.171.2.1.9.0

basicHubSelfTest1 OBJECT-TYPE

SYNTAX  INTEGER {

noSelfTest1(1),

selfTest1(2)

}

ACCESS  read-write

STATUS  mandatory

::= { basicGroup  9 }

-- 1.3.6.1.4.1.171.2.1.10.0

basicHubSelfTest2 OBJECT-TYPE

SYNTAX  INTEGER {

noSelfTest2(1),

selfTest2(2)

}

ACCESS  read-write

STATUS  mandatory

::= {basicGroup  10 }

-- 1.3.6.1.4.1.171.2.1.11.0

basucHubIntrusion OBJECT-TYPE

SYNTAX  INTEGER {

noIntrusion(1),

intrusion(2)

}

ACCESS  read-write

STATUS  mandatory

::= { basicGroup 11 }

-- 1.3.6.1.4.1.171.2.1.12.0

basicHubClearStats OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

clearstatsistics(2)

}

ACCESS  read-write

STATUS  mandatory

::= { basicGroup 12 }

-- 1.3.6.1.4.1.171.2.1.13.0

basicHubLed OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 13 }

-- 1.3.6.1.4.1.171.2.1.14.0

basicHubNumofPorts OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 14 }

-- The D-Link Hub Basic Port Table 1.3.6.1.4.1.171.2.1.15

-- 1.3.6.1.4.1.171.2.1.15

basicPortTable OBJECT-TYPE

SYNTAX  SEQUENCE OF BasicPortEntry

ACCESS  read-only

STATUS  mandatory

::= { basicGroup 15 }

-- 1.3.6.1.4.1.171.2.1.15.1

basicPortEntry OBJECT-TYPE

SYNTAX  BasicPortEntry

ACCESS  read-only

STATUS  mandatory

::= { basicPortTable 1 }

BasicPortEntry ::= SEQUENCE {

basicPortID

INTEGER,

basicPortAdminState

INTEGER,

basicPortAutoPartState

INTEGER,

basicPortName

DisplayString,

basicPortType

INTEGER,

basicPortLink

INTEGER,

basicPortClearStats

INTEGER

}

-- 1.3.6.1.4.1.171.2.1.15.1.1.0xFF

basicPortID OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

::= { basicPortEntry 1 }

-- 1.3.6.1.4.1.171.2.1.15.1.2.0xFF

basicPortAdminState OBJECT-TYPE

SYNTAX  INTEGER {

disabled(1),

enabled(2)

}

ACCESS  read-write

STATUS  mandatory

::= { basicPortEntry 2 }

-- 1.3.6.1.4.1.171.2.1.15.1.3.0xFF

basicPortAutoPartState OBJECT-TYPE

SYNTAX  INTEGER {

autoPartitioned(1),

notAutoPartitioned(2)

}

ACCESS  read-only

STATUS  mandatory

::= { basicPortEntry 3 }

-- 1.3.6.1.4.1.171.2.1.15.1.4.0xFF

basicPortName OBJECT-TYPE

SYNTAX  DisplayString

ACCESS  read-write

STATUS  mandatory

::= { basicPortEntry 4 }

-- 1.3.6.1.4.1.171.2.1.15.1.5.0xFF

basicPortType OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

tenBASET(2),

tenBASE2(3),

tenBASE5(4)

}

ACCESS  read-only

STATUS  mandatory

::= { basicPortEntry 5 }

-- 1.3.6.1.4.1.171.2.1.15.1.6.0xFF

basicPortLink OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

down(2),

up(3)

}

ACCESS  read-only

STATUS  mandatory

::= { basicPortEntry 6 }

-- 1.3.6.1.4.1.171.2.1.15.1.7.0xFF

basicPortClearStats OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

clearportstats(2)

}

ACCESS  read-only

STATUS  mandatory

::= { basicPortEntry 7 }

-- The D-Link Monitor Group  1.3.6.1.4.1.171.2.2

-- The D-Link Hub Monitor Table

-- 1.3.6.1.4.1.171.2.2.1.0

monHubCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monitorGroup 1 }

-- 1.3.6.1.4.1.171.2.2.2.0

monHubBadFrames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monitorGroup 2 }

-- 1.3.6.1.4.1.171.2.2.3.0

monHubReadableOctets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monitorGroup 3 }

-- 1.3.6.1.4.1.171.2.2.4.0

monHubReadableFrames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monitorGroup 4 }

-- 1.3.6.1.4.1.171.2.2.5.0

monHubLedFlag  OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  mandatory

::= { monitorGroup 5 }

-- 1.3.6.1.4.1.171.2.2.6.0

monHubUtilization OBJECT-TYPE

SYNTAX  INTEGER {

onePercentage(1),

twoPercentage(2),

fivePercentage(3),

tenPercentage(4),

fifteenPercentage(5),

twentyPercentage(6),

twenty-fivePercentage(7),

thirtyPercentage(8),

thirty-fivePercentage(9),

fortyPercentage(10),

fifty-fivePercentage(11),

finety-ninePercentage(12)

}

ACCESS  read-only

STATUS  mandatory

::= { monitorGroup 6 }

-- The D-Link Port Monitor Table 1.3.6.1.4.1.171.2.2.7

-- 1.3.6.1.4.1.171.2.2.7

monPortTable OBJECT-TYPE

SYNTAX  SEQUENCE OF MonPortEntry

ACCESS  read-only

STATUS  mandatory

::= { monitorGroup 7 }

-- 1.3.6.1.4.1.171.2.2.7.1

monPortEntry OBJECT-TYPE

SYNTAX  MonPortEntry

ACCESS  read-only

STATUS  mandatory

::= { monPortTable 1 }

MonPortEntry ::= SEQUENCE {

monPortID

INTEGER,

monPortReadableFrames

Counter,

monPortReadableOctets

Counter,

monPortFcsErrors

Counter,

monPortAlignmentErrors

Counter,

monPortFrameTooLongs

Counter,

monPortFrameTooShorts

Counter,

monPortRunts

Counter,

monPortCollisions

Counter,

monPortLateCollisions

Counter,

monPortDataRateMismatchs

Counter,

monPortAutoPartitions

Counter,

monPortBadFrames

Counter

}

-- 1.3.6.1.4.1.171.2.2.7.1.1.0xFF

monPortID OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 1 }

-- 1.3.6.1.4.1.171.2.2.7.1.2.0xFF

monPortReadableFrames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 2 }

-- 1.3.6.1.4.1.171.2.2.7.1.3.0xFF

monPortReadableOctets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 3 }

-- 1.3.6.1.4.1.171.2.2.7.1.4.0xFF

monPortFcsErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 4 }

-- 1.3.6.1.4.1.171.2.2.7.1.5.0xFF

monPortAlignmentErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 5 }

-- 1.3.6.1.4.1.171.2.2.7.1.6.0xFF

monPortFrameTooLongs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 6 }

-- 1.3.6.1.4.1.171.2.2.7.1.7.0xFF

monPortFrameTooShorts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 7 }

-- 1.3.6.1.4.1.171.2.2.7.1.8.0xFF

monPortRunts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 8 }

-- 1.3.6.1.4.1.171.2.2.7.1.9.0xFF

monPortCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 9 }

-- 1.3.6.1.4.1.171.2.2.7.1.10.0xFF

monPortLateCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 10 }

-- 1.3.6.1.4.1.171.2.2.7.1.11.0xFF

monPortDataRateMismatchs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 11 }

-- 1.3.6.1.4.1.171.2.2.7.1.12.0xFF

monPortAutoPartitions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 12}

-- 1.3.6.1.4.1.171.2.2.7.1.13.0xFF

monPortBadFrames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { monPortEntry 13 }

-- The D-Link Port Address Tracking Table 1.3.6.1.4.1.171.2.3

-- 1.3.6.1.4.1.171.2.3.1

addPortTable OBJECT-TYPE

SYNTAX  SEQUENCE OF AddPortEntry

ACCESS  read-only

STATUS  mandatory

::= { addrTrackGroup 1 }

-- 1.3.6.1.4.1.171.2.3.1.1

addPortEntry OBJECT-TYPE

SYNTAX  AddPortEntry

ACCESS  read-only

STATUS  mandatory

::= { addPortTable 1 }

AddPortEntry ::= SEQUENCE {

addPortID

INTEGER,

addPortLastSourceAddress

OCTET STRING,

addPortSourceAddrChanges

Counter,

addPortAuthorized

INTEGER

}

-- 1.3.6.1.4.1.171.2.3.1.1.1.0xFF

addPortID OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

::= { addPortEntry 1 }

-- 1.3.6.1.4.1.171.2.3.1.1.2.0xFF

addPortLastSourceAddress OBJECT-TYPE

SYNTAX  OCTET STRING (SIZE(6))

ACCESS  read-write

STATUS  mandatory

::= { addPortEntry 2 }

-- 1.3.6.1.4.1.171.2.3.1.1.3.0xFF

addPortSourceAddrChanges OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

::= { addPortEntry 3 }

-- 1.3.6.1.4.1.171.2.3.1.1.4.0xFF

addPortAuthorized OBJECT-TYPE

SYNTAX  INTEGER {

unauthorized(1),

authorized(2)

}

ACCESS  read-write

STATUS  mandatory

::= { addPortEntry 4 }

END