Nov. 1994

D-Link Drivers Installation Readme File

for

Windows NT V3.1

==============================================

***********************************************

*  README for Windows NT V3.1 Installation    *

***********************************************

The DE250 Ethernet LAN drivers of the DE-250 cards for Windows NT contained

in this diskette are as follows:

\NT31\DE250.SYS     ; NDIS 3.0 driver for Windows NT

\NT31\DE250.DLL     : Installation menu for Windows NT

\NT31\OEMSETNT.INF  ; OEMSETUP file for Windows NT

\DISK1

Installation Procedure:

(1)  When a LAN driver is installed, Windows NT will present a list of all

available drivers.  Since there will be no D-Link DE-250 driver

initially, select OTHERS for 3rd party adapters not in the driver list.

(2)  Windows NT will ask for the driver/path containing the DE-250 Windows NT

drivers.  Insert the D-Link driver diskette in drive A:, and then type

the following

A:\NT31

(3)  Windows NT will attempt to locate an OEMSETNT.INF file in the specified

path. If you have entered the path name correctly, Windows NT should copy

the appropriate drivers to the Windows NT system.

(4)  At this point, the drivers would have been properly copied and the

installation would proceed to the next step.  Please refer to your

Windows NT installation guide for the correct procedure.

(5)  After the initial installation, the DE-250 drivers will be permanently

copied to the Windows NT system.  Subsequent network driver lists will

already contain the D-Link DE-250 drivers.