July 1992

PC-NFS Driver Installation

for

D-Link DE-300/DE-300TP Ethernet Cards

=====================================

The D-Link PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS

on your PC with the D-Link MCA Ethernet Cards.

The following files will be used in this installation:

Files         Contents

-----------   -------------------------

PDNFS.SYS     PC-NFS Driver Program

DE100PD.SYS   DE-300 Driver Program

DE100PD.COM   DE-300 Driver Program

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure described in

the PC-NFS manual.  On the screen of the PC-NFS install program, select

3C501 (the 3Com EtherLink card) as your network interface card.

When the screens prompt you, select 3 for the interrupt level (IRQ),

and select 300h for the base I/O address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE100PD.SYS

DE100PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=c:\NFS\DE100PD.SYS <packet_int_no>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate other computers

through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure described in

the PC-NFS manual.  On the screen of the PC-NFS install program, select

3C501 (the 3Com EtherLink card) as your network interface card.

When the screens prompt you, select 3 for the interrupt level (IRQ),

and select 300h for the base I/O address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE100PD.SYS

DE100PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

DE100PD <packet_int_no>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.