July 1992

3Com 3+ Share Driver Installation

for

D-Link DE-300/DE-300TP Ethernet Cards

=====================================

The D-Link 3Com 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link MCA Ethernet cards.

The following files will be used in this installation:

Files            Contents

-----------      -------------------------

DE100PD.SYS      Packet Driver Program

ETH.SYS          3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link MCA Ethernet cards to the boot disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure in step 1

above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.  The order of these two

statement must be as shown below.

DEVICE = DE100PD.SYS <packet_int_no>

DEVICE = ETH.SYS

You may now boot and start your 3COM workstation.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.