July 1992

FTP PC/TCP Installation

for

D-Link DE-300/DE-300TP Ethernet Cards

=====================================

The following files will be used in this installation:

Files         Contents

-----------   ---------------------------------

DE100PD.SYS   DE-300 Packet Driver Program

DE100PD.COM   DE-300 Packet Driver Program

The D-Link packet driver programs above let you run FTP's PC/TCP with

the D-Link DE-300 Ethernet cards.  Two methods can be used:

METHOD 1:

If you use the DE100PD.SYS program, the following statement

must be included in your CONFIG.SYS file:

DEVICE = DE100PD.SYS <packet_int_no>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DE100PD.COM program, follow the procedure below:

1. Execute the DE100PD.COM as follows:

>DE100PD <-d> <packet_int_no>

2. Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.