NDIS Driver Program Installation	       July 1992

===========================================

The NDIS driver programs are for use with the following D-Link

products:

DE-300

DE-300TP

This diskette is labeled as LAN_DRV_DLK and the files are organized as

follows:

Directory	Contents

---------	----------------------------------------------

\NDIS\DOS	NDIS driver for DOS

DE300.DOS

DE300.NIF

PROTOCOL.INI

\NDIS\OS2	NDIS driver for OS/2

DE300.OS2

DE300.NIF

PROTOCOL.INI

D-Link NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

D-Link NDIS driver program support the following network operating systems:

MS LAN Manager v2.0

DECnet PathWork v4.0

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.00(2)

IBM LAN Support Program v1.2.1

***********************************************

*  How to install DE300.DOS on MS LAN MANAGER *

***********************************************

1)  Prepare a diskette with volume name "DOS_DRV_DLK" and create directories

in the following manner:

>label a: DOS_DRV_DLK	  --insert a blank disk in drive 'A'

>md MSLANMAN.DOS	  --make a directory called MSLANMAN.DOS

>cd MSLANMAN.DOS

>md DRIVERS		  --make a directory called DRIVERS

>cd DRIVERS

>md ETHERNET		  --make a directory called ETHERNET

>md NIF 		  --make a directory called NIF

>cd ETHERNET

>md DE300		  --make a directory called DE300

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|		  |

ETHERNET		 NIF

|

-------------

|	     |

DE300

2) Insert the DE-300 driver program diskette in drive 'A' and insert the

prepared diskette (DOS_DRV_DLK) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:			--go to drive 'A'

>cd ndis\dos

>copy DE300.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DE300

>copy PROTOCOL.INI  B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DE300

>copy DE300.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for DE-300 card.

-If you are installing LAN Manager for the first time, do the following:

Upon completion of retrieval of the last workstation's disk contents, you

will be asked if you want to import additional network drivers.

Answer "YES".

-If you have already installed LAN Manager in your hardisk, do the following:

Select the Configuration menu, and then select "Add New Config" option.

Afterwards, choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_DLK) in drive 'A' and follow the

instructions on the screen.	The SETUP program will copy the necessary

NDIS driver programs from the DOS_DRV_DLK diskette to your hardisk (into

the LAN Manager directory).

5) After completion of the SETUP, your CONFIG.SYS file will be modified and

contain statements referring to the D-Link NDIS driver programs.

Reboot your computer and follow the NDIS binding procedure to use the D-Link

NDIS driver programs.

*******************************************************

*  How to install DE300.DOS on WIN/TCP PathWay Access *

*******************************************************

1) Copy DE300.DOS to directory C:\LANMAN.

2) Go to the directory C:\LANMAN, and create a text file called PROTOCOL.INI:

[DE300]

drivername = DE300$

adapterid = 0xfcc

3) Add the following statements to your CONFIG.SYS file:

device=c:\lanman\DE300.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis -d:1

winbind

wintcp

****************************************************

* How to install DE300.DOS on DECnet PathWork v4.0 *

****************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install D-Link

NDIS driver program.  Please refer to  DECnet  installation manual for

detailed information.	The following instructions assume that you have

already installed the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

device = c:\decnet\ELNKII.SYS

to

device = c:\decnet\DE300.DOS

2) Modify the PROTOCOL.INI file as follows:

from:

---------------------------------

[ELNKMACII]

drivername = ELNKII$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]

:

bindings = ELNKMACII

:

[DATALINK]

:

bindings = ELNKMACII

:

to:

-----------------------------------

[DE300_NIF]

drivername = DE300$

adapterid = 0xfcc

[IPX4MAC]

:

bindings = DE300_NIF

:

[DATALINK]

:

bindings = DE300_NIF

:

********************************************

* How to install DE300.DOS on 3Com 3+ Open *

********************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy DE300.DOS to  C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device = c:\3open\doswksta\lanman\drivers\DE300.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = DE300$

adapterid = 0xfcc

:

*****************************************************

* How to install DE300.DOS on Banyan VINES v4.00(2) *

*****************************************************

NOTE : To install D-Link NDIS driver program, you need BANYAN VINES v4.00(2)

or above.

1. Create a bootable diskette (booted with DOS 3.1 or above) which will be

used as boot diskette of Banyan VINES workstation or create a sub-directory

on your hard disk.

There are two methods to copy the required files for Banyan VINES workstation

to the bootable diskette or sub-directory of hard disk.

Method 1.

1) Execute 'INSTALL' command which was saved in the diskette labeled

"VINES MASTER DISK".

2) Follow the instructions to copy the files to the bootable diskette

or sub-directory of hard disk.

Method 2.

1) Execute 'BAN' command.

2) Login VINES system.

3) Execute 'PCCOPY' command.

4) Select 'NDIS Ethernet' item and follow instructions as menu driven

for copying the files to the bootable diskette or sub-directory of

hard disk.

2. Insert the DE-300 driver program diskette in drive 'A' and insert the

prepared bootable diskette in drive 'B'. Afterwards, copy the appropriate

NDIS driver files DE300.DOS from drive 'A' to 'B' or sub-directory of hard

disk.

3. Create a CONFIG.SYS file which contains the following statements:

DEVICE=PROTMAN.DOS /I:A:\

DEVICE=DE300.DOS

NOTE:" A:\" is the path which may be changed dependeding on the location

of the files for BANYAN VINES workstation.

4. Create a DE300.NIF file and include the following statements in this file:

Model = D-Link DE-300 Ethernet Card

Path = A:\

DeviceDriver = DE300.DOS

DeviceName = DE300$

Type = NDIS

NOTE:" A:\" is the path which may be changed dependeding on the location

of the files for BANYAN VINES workstation.

5. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "DE300_NIF"

[DE300_NIF]

drivername = DE300$

adapterid = 0xfcc

6. Execute 'PCCONFIG' command

1) The "CONFIGURING A PC FOR THE NETWORK" is displayed on the screen

then select item '1 - Network Card Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

The "NDIS Ethernet" is displayed on the screen

then set your choice for D-Link NIC

Ex. Interrupt Level (2-15) : 3

PROTOCOL.INI bindings = DE300

|

2) Return to Screen "CONFIGURING A PC FOR THE NETWORK"

then select item '2 - Login Environment Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

then 'PCCONFIG.DB' file will be generated.

7. Reboot your computer.

8. Login into VINES by executing the 'BAN' command.

**************************************************************

* How to install DE300.DOS on IBM LAN Support Program v1.2.1 *

**************************************************************

Supposing that you have installed the IBM LAN Support Program in your

computer, you can use DE300.DOS to replace the installed driver program.

Follow the instructions below to change the contents of CONFIG.SYS and

PROTOCOL.INI files.

1. Copy DE300.DOS to the directory where your IBM LAN Support program

resides.

2. Modify the original NDIS device driver statement in CONFIG.SYS as

follows:

DEVICE = [driver]:\path\DE300.DOS

3. Modify the PROTOCOL.INI file as follows:

BINDINGS = DE300

:

:

[DE300]

DRIVERNAME = DE300$

ADAPTERID = 0xFCC

==========================================================================

D-Link NDIS PROTOCOL.INI User Guide

The keywords used to create the PROTOCOL.INI file for DE-300 is as follows:

DRIVERNAME -This specifies the network card's driver name.

ADAPTERID  -This keyword specifies the adapter ID used by D-Link MCA bus card.

This value must match the card ID jumper setting on your MCA adapter.

NETADDRESS -This specifies the node address (hex) used by your network card.

However, you can specify a node address different from the one

that is burned into your network card.  In such case, the user

specified node address will be used instead of the card's

actual address.

Following is an example of a PROTOCOL.INI file that can be used with D-Link

Ethernet DE-300 card:

Drivername = DE300$

Adapterid = 0xFCC