July 1992

NetBIOS Emulator Installation

for

DE-320CT

EtherSmart MCA Coax/10BaseT Card

======================================

The D-Link NetBIOS Emulator is a standardized network interface

program that allows this adapter to run NetBIOS-based network

operating systems like the IBM PC LAN Program and others.  No

software modification or additional installation is necessary.

All operations as well as installation procedures for the network

operating system will remain unaltered.

Ths files that will be used in this installation:

Files           Contents

-----------     -------------------------

DE-320.EXE      DE-320 Driver Program

NBIOS.EXE       NetBIOS Emulator Program

To install the NetBIOS Emulator, do as follows:

1. Load in the driver program DE-320.EXE.

>DE-320

2. Load in the NetBIOS Emulator program NBIOS.EXE.  This program is

memory-resident and will stay in your computer's memory until its

power is turned off or until you reboot.

>NBIOS [/Sm] [/Ck]

Typing the parameter after NBIOS is optional.  The parameter is used

as follows:

/Sm   specifies the maximum number of sessions allowed on your

computer, where m can be a value from 1 to 254.  If this

parameter is not specified, the default number of sessions

will be set to 32.

/Ck   specifies the maximum number of NetBIOS command available for

your computer where k can be a value from 1 to 255.  If this

parameter is not specified, the default number of NetBIOS

commands will be set to 32.

Your computer now is ready to run a NetBIOS-based network

operating system.