Mar. 01, 1995

NT V3.5 Driver Instatllation

For

DE-320CT

EtherSmart MCA Coax/10BaseT Card

=====================================

The DE-320 EtherSmart MCA drivers of DE-320CT card for Windows NT contained

in this diskette are as follows:

\NT35\DE320.SYS       NDIS 3.0 driver for Windows NT

\NT35\OEMSETUP.INF    OEMSETUP file for Windows NT

\NT35\Readme.doc

Installation Procedure:

(1) When LAN drivers is installed, Windows NT will present a list of all

available drivers.	Since there will be no D-Link DE-320 drivers

initially, select OTHERS for 3rd party adapters not in the drivers

list.

(2) Windows NT will ask for the drive/path containing the DE-320 Windows NT

drivers.  Insert the D-Link driver diskette in the drive A: and respond

with the following:

A:\NT35

(3) Windows NT will attempt to locate an OEMSETNT.INF file in the specified

path.  If you have entered the path name correctly, Windows NT should

copy the appropriate drivers to the Windows NT system.

(4) At this point, the driver would have been properly copied and the

installation would proceed to the next setp.  Please refer to your

Windows NT installation guide for correct procedures.

(5) After the initial installation, the DE-320 drivers will be permanently

copied to the Windows NT system.  Subsequent network driver lists will

already contain D-Link DE-320 driver.