July 1992

PC-NFS Driver Installation

for

DE-320CT

EtherSmart MCA Coax/10BaseT Card

=====================================

The D-Link PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS

on your PC with the D-Link MCA Ethernet Cards.

The following files will be used in this installation:

Files         Contents

-----------   -------------------------

PDNFS.SYS     PC-NFS Driver Program

DE320PD.SYS   DE-320 Driver Program

DE320PD.COM   DE-320 Driver Program

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure described in

the PC-NFS manual.  On the screen of the PC-NFS install program, select

3C501 (the 3Com EtherLink card) as your network interface card.

When the screens prompt you, select 3 for the interrupt level (IRQ),

and select 300h for the base I/O address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE320PD.SYS

DE320PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=c:\NFS\DE320PD.SYS <-d> <packet_int_no> <-w>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-w>            the -w parameter is for use with the Windows environment only.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate other computers

through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure described in

the PC-NFS manual.  On the screen of the PC-NFS install program, select

3C501 (the 3Com EtherLink card) as your network interface card.

When the screens prompt you, select 3 for the interrupt level (IRQ),

and select 300h for the base I/O address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE320PD.SYS

DE320PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

DE320PD <packet_int_no>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.