Installing D-Link network adapters for MS Windows for Workgroups

================================================================

D-Link network adapters are supplied with a driver disk which includes the

NDIS driver.  The NDIS driver file naming convention used by D-Link is

<NICtype.DOS>, where NICtype stands for DE100, DE150, DE200, DE220, and etc.

The D-Link NDIS driver programs for DOS are developed following Microsoft

NDIS specification version 2.0.1.  This document is a detailed step-by-step

installation procedure for using D-Link network adapters with MS Windows

for Workgroups operating system.

Naming Conventions

==================

Starting at this point, we will refer to Windows for Workgroups as

WFW for simplicity.  Any appearance of WFW within this text will refer

to the Microsoft Windows for Workgroups operating system.

Installation Procedures

=======================

1.  Preinstall WFW using the included Novell NE2000 compatible driver. WFW

will generate a default PROTOCOL.INI and CONFIG.SYS file using the

selected NE2000 network driver.

2.  You must modify the generated PROTOCOL.INI and CONFIG.SYS files using

any text editor. (i.e. MSDOS EDIT)

--------- modify PROTOCOL.INI ----------

3.  The PROTOCOL.INI file will have multiple references to the "ms$ne2clone"

name.  Change all occurences of this name to "D-Link".  In a standard

PROTOCOL.INI, there should be a total of 7 replacements.

There would be 4 replacements in the [network.setup] section.  Please

take note of the following lines in [network.setup] section,

netcard=...

lana0=...

lana1=...

There will be a [ms$ne2clone] section.  This should also be changed to

[D-Link].

In both [MS$NETBEUI] and [MS$IPX] sections, there will be a BINDINGS=

command line.  The BINDINGS command must be bound to "D-Link".

4.  There will also be a reference to "ms2000$" in the [D-Link] section

which needs to be changed to the D-Link NIC type name. (i.e. DE100$,

DE150$, DE200$, DE220$, DE300$, DE320$, DE400$, DE600$, DE620$, DE650$,

etc.)

5.  In the [D-Link] section, the "IOBASE" keyword must be changed to

"IOADDRESS".

The succeeding pages include working sample of CONFIG.SYS and PROTOCOL.

INI files for WFW using D-Link DE-320 network adapter.

--------- modify CONFIG.SYS ----------

6.  Your CONFIG.SYS will have the following statement for loading the

"ms2000.DOS" device driver.

DEVICE=C:\WINDOWS\MS2000.DOS

Change this line to load the appropriate D-Link NDIS device driver

of your network adapter.  Using D-Link DE-150 adapter, the line would

appear as,

DEVICE=C:\WINDOWS\DE320.DOS

Note: You would have to copy the appropriate D-Link NDIS driver to

your WFW directory. (i.e. C:\WINDOWS)

D-Link NDIS PROTOCOL.INI User Guide

===================================

This guide lists the available PROTOCOL.INI keywords for all

D-Link NDIS device drivers.  Most of the keywords are optional

and have default values.  The asterisk symbol (*) indicates the

list of D-Link adapters where the keyword is applicable.

IOADDRESS - This keyword specifies the base I/O port address used

by the D-Link Network Interface Card. This value must

match the base I/O address jumper setting on your NIC.

If this keyword is not present, the default value will

be 300h.

*all D-Link network adapters

INTERRUPT - This keyword specifies the interrupt line used by the

D-Link NIC. This value must match the IRQ jumper setting

on your NIC.  If this keyword is not present, the default

value will be 3.

*all D-Link network adapters

DRIVERNAME- This keyword specifies the NIC driver's name.  The NIC

driver name follows the NIC adapter type.  (i.e. DE100,

DE150, DE200, DE220 etc.)

*all D-Link network adapters

PRINTPORT - This keyword specifies the printer port used by your

D-Link Pocket LAN Adapter.

*DE-600, DE-620

BASEMEMORY- This keyword specifies the mapped memory address used

by the D-Link NIC.  This value must match the base

memory jumper setting on your NIC.  Base memory will

only be in effect if the remote boot ROM jumper is

enabled.

*DE-100, DE-150, DE-200

ADAPTERID - This keyword specifies the adapter ID used by D-Link

MCA bus card. This value must match the card ID jumper

setting on your MCA adapter.

*DE-300, DE-320

NETADDRESS- This keyword specifies the node address (hex) used by

your network interface card.  This option is used only

if you want to override the node address burned in your

NIC.  In this case, the specified address will be used

instead of the NIC's actual node address.

*all D-Link network adapters

Working samples of WFW CONFIG.SYS and PROTOCOL.INI

==================================================

CONFIG.SYS (using D-Link DE-320CT NIC)

---------------------------------------

:

device=c:\windows\himem.sys

device=c:\windows\protman.dos /i:c:\windows

device=c:\windows\workgrp.sys

device=c:\windows\msipx.sys

device=c:\windows\de320.dos            <������� D-Link NDIS driver

:

AUTOEXEC.BAT

------------

:

c:\windows\net start

c:\windows\msipx

c:\windows\netx

:

C:\WINDOWS\PROTOCOL.INI (using D-Link DE-320CT NIC)

----------------------------------------------------

[network.setup]

version=0x3100

netcard=D-Link,1,D-Link                <������Ŀ

transport=ms$netbeui,MS$NETBEUI                |  Use D-Link NIC

lana0=D-Link,1,ms$netbeui              <��������� for Netbios and

transport=ms$ipx,MS$IPX                        |  IPX

lana1=D-Link,ms$ipx                    <��������

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[D-Link]                               <������Ŀ

Drivername = de320$                            �  This section for

ioaddress = 0x300                              �� D-Link NIC

interrupt = 3                                  �

basememory = 0xd000                    <��������

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=5

NCBS=10

BINDINGS=D-Link                        <������Ŀ

LANABASE=0                                     |

|

[MS$IPX]                                       �� Bind to [D-Link]

DriverName=IPX$                                |

MediaType=Novell/Ethernet                      |

BINDINGS=D-Link                        <��������