NetWare Driver Installation                 July 1992

========================================

The NetWare driver programs in this disk support the D-Link Ethernet

DE-320CT EtherSmart MCA Coax/10BaseT card.

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory             Contents

---------             --------------------------------------------------------

\NETWARE\V2xx         NetWare v2.x server & bridge drivers

\NETWARE\V215B        NetWare v2.15b workstation drivers

\NETWARE\V215C        NetWare v2.15c workstation drivers

\NETWARE\NET386       NetWare 386 server or bridge drivers

( support NetWare v 3.12 )

\NETWARE\ODI.DOS      ODI workstation driver for DOS

\NETWARE\ODI.OS2      ODI workstation driver for OS/2

\NETWARE\NW40.SRV     NetWare 4.0 server drivers

( support NetWare v 4.01 )

***********************

*  WORKSTATION SETUP  *

***********************

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

DE320ODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example 1:  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\NET386\*.*  B:\   --SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load A:\386DE320.LAN   --Select your network card settings

:Bind ipx to 386DE320

:load install           --Follow the NetWare installation instructions

Example 2:  NetWare v2.2 Server/Bridge Setup

============================================

1) Copy the appropriate NetWare driver files (from this diskette) to the

root directory of the diskettes labeled SYSTEM-1 and LAN_DRV_DLK.

>Copy A:\NETWARE\V2XX\*.LAN  A:\  --SYSTEM-1 disk must be in drv A

>Copy A:\NETWARE\V2XX\*.OBJ  A:\  --Copy the files to the root directory

of the diskette labeled LAN_DRV_DLK

2) Execute Novell's INSTALL program, which resides in the SYSTEM-1 diskette.

Choose the "D-Link Ethernet DE-320" driver when asked for a network driver.

Example 3:  NetWare v2.15b Server/Brige Setup

=============================================

1) Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled LAN_DRV_DLK.

>Copy A:\NETWARE\V2XX\*.LAN  A:\ --AUXGEN disk must be in drive A

>Copy A:\NETWARE\V2XX\*.OBJ  A:\ --LAN_DRV_DLK disk must be in drive A

2) Execute Novell's NETGEN program, which resides in the NETGEN diskette.

Choose the "D-Link Ethernet DE-320" driver, when asked for a network driver.

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocol stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.  For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE320ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE320ODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>DE320ODI          --Hardware Specific Module provided by D-Link

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy DE320ODI.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\DE320ODI.SYS

3)  D-Link ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".

*********************************************

* NetWare 4.0 SERVER and ODI CLIENT DRIVERS *

*********************************************

NetWare 4.0 Server Setup

==========================

1. Install the CDROM properly.

2. Prepare a D-Link LAN driver diskette that includes the following files

Insert D-Link driver diskette in A: driver

Insert D-Link LAN driver diskette in B: driver

1) Copy NWDE320.LAN, NWDE320.LDI from D-Link driver programs diskette

as follows,

copy A:\NETWARE\NW40.SRV\NWDE320.LAN B:

copy A:\NETWARE\NW40.SRV\NWDE320.LDI B:

2) Copy ETHERTSM.NLM, MSM.NLM from NETWARE 4.0 CD ROM or Diskette

copy L:\NETWARE.40\________\LANDRV\ETHERTSM.NLM B:

copy L:\NETWARE.40\________\LANDRV\MSM.NLM      B:

3. Install NetWare 4.0 SERVER

L: (CD ROM drive)

L:\NETWARE.40\ENGLISH\install

4. Loading the LAN driver

Choose a LAN driver that corresponds to the network interface board in

the server, repeat this step for any additional drivers.

1) Press <INS> to load a LAN driver not listed.

2) Press <F3> to specify a different path,

> B:\

( Put D-Link LAN driver into the diskette drive which contains

ETHERTSM.NLM and MSM.NLM )

3) Netware will display the list including the additional new driver.

Select the new LAN driver to install

NWDE320.LAN

D-Link DE-320 LAN Driver v2.00 (930801)

4) Set driver NWDE320 parameters such as following :

Port value : 300H

Node address :

Frame type = Ethernet_802.2

Retries = 5

5) Bind the interface card to a protocol

assign a unique IP Network address to bind IPX to NWDE320

(frame Ethernet_802.2)

6) Proceed and finish with installation

ODI Client for DOS

===================

D-Link ODI Driver allows you to load multiple protocol stacks. It also allows

you to install up to two network adapters in a workstation. D-Link ODI driver

supports "port, node address and frame" options. You can use these options to

customize the NET.CFG file.  For detailed info on Keywords, please refer to

Novell manual "NetWare ODI shell for DOS - NET.CFG options".

Please install the NetWare ODI client driver files following the procedures

as described in the NetWare installation guide.

If you want to generate your own NetWare ODI client driver, please do the

following:

1)  Make a backup of WSDRV_1 diskette.

2)  Copy the appropriate DE320ODI.COM, DLKDRVS.DOS and DE320ODI.INS driver

files to DOS directory of WSDRV_1 diskette (or backup):

Insert D-Link driver diskette in A: driver.

Insert NetWare WSDOS_1 diskette in B: driver.

>Copy  A:\NETWARE\ODI.DOS\*.*  B:\DOS  --For NetWare client workstation

Append contents of DLKDRVS.DOS to DRIVERS.DOS

Contents of DLKDRVS.DOS is as follows:

" D-Link DE-100 Ethernet for ISA,DE100ODI.COM,DE100ODI.INS,IE0

D-Link DE-200 Ethernet for ISA,DE200ODI.COM,DE200ODI.INS,IE0

D-Link DE-220 Series Ethernet for ISA,DE220ODI.COM,DE220ODI.INS,IE0

D-Link DE-320 Ethernet for Micro Channel,DE320ODI.COM,DE320ODI.INS,00M

D-Link DE-400 Ethernet for EISA,DE400ODI.COM,DE400ODI.INS,0E0

D-Link DE-600 Ethernet Pocket LAN,DE600ODI.COM,DE600ODI.INS,IEM

D-Link DE-620 Ethernet Pocket LAN,DE620ODI.COM,DE620ODI.INS,IEM

D-Link DE-650 Ethernet PCMCIA,DE650ODI.COM,DE650ODI.INS,IEM             "

3)  Execute Novell's INSTALL program, which resides in the WSDOS_1 diskette.

When you are asked to install the driver for your network board, press

the INS key.  You will be asked to insert a WSDRV_1 diskette.  Insert

this diskette (or backup) which contains DE320ODI.COM, DRIVERS.DOS and

DE400ODI.INS in drive A.

4)  Follow the NetWare Client Install normal installation procedures, Which

resides in the WSDOS_1 diskette. When asked to select a network driver,

simply choose "D-Link DE-320 Ethernet for Micro Channel" for selecting

parameter like as Base I/O Port, Media Frame Type(s) and Optional Node

Address.

Upon completion, a new NET.CFG file will be created in your DOS directory.

A typical of NET.CFG is as follows:

Link Driver DE320ODI

PORT 300

FRAME Ethernet_802.2

NetWare DOS Requester

FIRST NETWORK DRIVE = F

:

:

5)  A typical startnet.bat for ODI workstation is as follows:

SET NWLANGUAGE=ENGLISH  --Set NetWare 4.0 to English language

LSL                     --Link Support Layer Module provided by Novell

DE320ODI                --Hardware Specific Module provided by D-Link

IPXODI                  --IPX Protocol Stack Module provided by Novell

VLM                     --DOS Support Module provided by Novell

F:                      --Change to connected device

LOGIN .ADMIN.NV40SRV    --Login to file server NV40SRV as user ADMIN

6) You may run STARTNET to access the Novell network.