August 1992

FTP PC/TCP Installation

for

DE-400 EISA Bus Master Ethernet Card

========================================

The following files will be used in this installation:

Files         Contents

-----------   ---------------------------------

DE400PD.SYS   Packet Driver Program

DE400PD.COM   Packet Driver Program

The D-Link packet driver programs above let you run FTP's PC/TCP

with the D-Link DE-400 EISA Bus Master Ethernet Card.

Two methods can be used:

METHOD 1:

If you use the DE400PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE400PD.SYS <packet_int_no> <slot_number> <-k>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DE400PD.COM program, follow the procedure below:

1. Execute the DE400PD.COM as follows:

>DE400PD <-d> <packet_int_no> <slot_number> <-k>

2. Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<slot_number>   is the number of the slot where DE400 card is inserted. If

slot_number is equal to 0, the slot number will be detected

automatically.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-k>            You have to use this parameter to enable the DE-400 drivers

to work with other support drivers loaded in high memory.

The DE-400 drivers themselves cannot be loaded into high memory.