August 1992

SCO UNIX V.3 Driver Installation

for

DE-400 EISA Bus Master Ethernet Card

======================================

The following files will be used in this installation:

Files          Contents

-----------    -----------------------------------------

de400.o        DE-400 Driver Program

dlk4

tcp

1. Login as the root user.

2. Installing TCP/IP software through the custom utility.

# custom

Select install

Select A New Product

Select entire to install all of the packages

3. Rename /usr/lib/mkdev/tcp to /usr/lib/mkdev/tcp.save if you want to keep

original /usr/lib/mkdev/tcp file. (it will be modified by next step)

4. Installation diskette in DOS format

Put your D-Link Installation Diskette to floppy A and use doscp command

to copy D-Link SCO UNIX Driver to following directories.

# cd /

# mkdir /usr/lib/tcprt/dlk

# doscp A:readme.doc /usr/lib/tcprt/dlk/readme.doc

# doscp A:de400.o /usr/lib/tcprt/dlk/de400.o

# doscp A:dlk4 /usr/lib/mkdev/dlk4

# doscp A:tcp /usr/lib/mkdev/tcp

# chmod +x /usr/lib/mkdev/dlk4

# chmod +x /usr/lib/mkdev/tcp

5. Installing D-Link SCO UNIX Driver

# mkdev dlk4

Do you wish to install or delete the dlk driver?  type i to install

A display of information about driver.            type y to continue

Select interrupt vector                           value

Which slot the adapter occupied                   input 1 to F

Do you want relink the kernel?                    type y to relink

Do you want this kernel to boot by default?       type y to default

Do you want the kernel environment rebuilt?       type y to rebuilt

6. Configuring Network interface

# mkdev tcp

which driver(s) will TCP/IP be using?           select dlk

DOMAIN name                                     enter key

IP address                                      n.n.n.n

broadcast address of all 0's?                   [y/n]

confirm broadcast address                       enter key

confirm netmask                                 enter key

which driver(s) will TCP/IP be using?           quit

7. Reboot unix

# shutdown -y -g0