NetWare Driver Installation               November 1993

========================================

The NetWare driver programs in this disk support the D-Link DE-400 EISA Bus

Master card.

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory             Contents

-----------------     --------------------------------------------------------

\NETWARE\V2xx         NetWare v2.x server & bridge drivers

\NETWARE\V215C        NetWare v2.15c workstation drivers

\NETWARE\NET386       NetWare 386 server or bridge drivers

( support NetWare v 3.12 )

\NETWARE\ODI.DOS      ODI workstation driver for DOS

\NETWARE\ODI.OS2      ODI workstation driver for OS/2

\NETWARE\NW40         NetWare 4.X server and ODI client drivers

( support NetWare v 4.01 )

*******************************************************

*  QUICK SETUP FOR WORKSTATION WITH MEMORY MANAGER    *

*******************************************************

** DOUBLEBUFFER **

DOUBLEBUFFER :

You have to use this parameter to enable the DE-400 drivers to work with

other support drivers loaded in high memory.

The DE-400 drivers themselves cannot be loaded into high memory.

Note1 : When you generate IPXD400.COM by selecting

"1:Driver Configurable by JUMPERS Utility"

Note2 : The JUMPERS utility is included with NETWARE, the procedure is

as follows:

Note3 : This procedure will change the IPXDE400.COM file.  You might

want to backup the current IPXDE400.COM before proceeding.

a. A:\>JUMPERS

b. Enter the file "IPXDE400.COM" to modify

c. Screen display is as follows:

DE-400 Options

===============

EISA solt           N/A

Transmit buffer     N/A

d. Select Transmit buffer then press "Enter"

(window assistance)

e. Select "Double-buffering"

f. Save the changes

g. The change will be written to IPXDE400.COM.

***********************

*  WORKSTATION SETUP  *

***********************

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

DE400ODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example 1:  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\NET386\*.*  B:\   --SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load A:\386DE400.LAN   --Select your network card settings

:Bind ipx to 386DE400

:load install           --Follow the NetWare installation instructions

Example 2:  NetWare v2.2 Server/Bridge Setup

============================================

1) Copy the appropriate NetWare driver files (from this diskette) to the

root directory of the diskettes labeled SYSTEM-1 and LAN_DRV_DLK.

>Copy A:\NETWARE\V2XX\*.LAN  A:\  --SYSTEM-1 disk must be in drv A

>Copy A:\NETWARE\V2XX\*.OBJ  A:\  --Copy the files to the root directory

of the diskettes labeled LAN_DRV_DLK

2) Execute Novell's INSTALL program, which resides in the SYSTEM-1 diskette.

Choose the "D-Link Ethernet DE-400" driver when asked for a network driver.

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocol stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.  For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE400ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE400ODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>DE400ODI          --Hardware Specific Module provided by D-Link

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy DE400ODI.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\DE400ODI.SYS

3)  D-Link ODI driver supports "slot, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".

*********************************************

* NetWare 4.0 SERVER and ODI CLIENT DRIVERS *

*********************************************

NetWare 4.0 Server Setup

==========================

1. Install the CDROM properly.

2. Prepare a D-Link LAN driver diskette that includes the following files

Insert D-Link driver diskette in A: driver

Insert D-Link LAN driver diskette in B: driver

1) Copy NWDE400.LAN, NWDE400.LDI from D-Link driver programs diskette

as follows,

copy A:\NETWARE\NW40\SRV\NWDE400.LAN B:

copy A:\NETWARE\NW40\SRV\NWDE400.LDI B:

2) Copy ETHERTSM.NLM, MSM.NLM from NETWARE 4.0 CD ROM or Diskette

copy L:\NETWARE.40\________\LANDRV\ETHERTSM.NLM B:

copy L:\NETWARE.40\________\LANDRV\MSM.NLM      B:

3. Install NetWare 4.0 SERVER

L: (CD ROM drive)

L:\NETWARE.40\ENGLISH\install

4. Loading the LAN driver

Choose a LAN driver that corresponds to the network interface board in

the server, repeat this step for any additional drivers.

1) Press <INS> to load a LAN driver not listed.

2) Press <F3> to specify a different path,

> B:\

( Put D-Link LAN driver into the diskette drive which contains

LAN drivers, THERTSM.NLM and MSM.NLM )

3) Netware will display the list including the additional new driver.

Select the new LAN driver to install

NWDE400.LAN

D-Link DE-400 LAN Driver v2.00 (931001)

4) Set driver NWDE400 parameters such as following :

Slot Number:  4

Node address:

Retries:      5

Frame types:  (select for frame list)

5) Bind the interface card to a protocol

assign a unique Network address to bind IPX to NWDE400

(for each frame type)

6) Proceed and finish with installation

ODI Client for DOS

===================

D-Link ODI Driver allows you to load multiple protocol stacks. It also allows

you to install up to two network adapters in a workstation. D-Link ODI driver

supports "port, node address and frame" options. You can use these options to

customize the NET.CFG file.  For detailed info on Keywords, please refer to

Novell manual "NetWare ODI shell for DOS - NET.CFG options".

Please install the NetWare ODI client driver files following the procedures

as described in the NetWare installation guide.

If you want to generate your own NetWare ODI client driver, please do the

following:

1)  Make a backup of WSDRV_1 diskette.

2)  Copy the appropriate DE400ODI.COM, DLKDRVS.DOS and DE400ODI.INS driver

files to DOS directory of WSDRV_1 diskette (or backup):

Insert D-Link driver diskette in A: driver.

Insert NetWare WSDOS_1 diskette in B: driver.

>Copy  A:\NETWARE\NW40\ODI.DOS\*.*  B:\DOS  --For NetWare client workstation

Append contents of DLKDRVS.DOS to DRIVERS.DOS

Contents of DLKDRVS.DOS is as follows:

" D-Link DE-100 Ethernet for ISA,DE100ODI.COM,DE100ODI.INS,I00

D-Link DE-200 Ethernet for ISA,DE200ODI.COM,DE200ODI.INS,IE0

D-Link DE-220 Series Ethernet for ISA,DE220ODI.COM,DE220ODI.INS,IE0

D-Link DE-320 Ethernet for Micro Channel,DE320ODI.COM,DE320ODI.INS,0E0

D-Link DE-400 Ethernet for EISA,DE400ODI.COM,DE400ODI.INS,00M

D-Link DE-600 Ethernet Pocket LAN,DE600ODI.COM,DE600ODI.INS,IEM

D-Link DE-620 Ethernet Pocket LAN,DE620ODI.COM,DE620ODI.INS,IEM

D-Link DE-650 Ethernet PCMCIA,DE650ODI.COM,DE650ODI.INS,IEM             "

3)  Execute Novell's INSTALL program, which resides in the WSDOS_1 diskette.

When you are asked to install the driver for your network board, press

the INS key.  You will be asked to insert a WSDRV_1 diskette.  Insert

this diskette (or backup) which contains DE400ODI.COM, DRIVERS.DOS and

DE400ODI.INS in drive A.

4)  Follow the NetWare Client Install normal installation procedures, Which

resides in the WSDOS_1 diskette. When asked to select a network driver,

simply choose "D-Link DE-400 Ethernet for EISA " for selecting

parameter like as Media Frame Type(s), Optional Node Address and Slot.

Upon completion, a new NET.CFG file will be created in your DOS directory.

A typical of NET.CFG is as follows:

Link Driver DE400ODI

FRAME Ethernet_802.2

SLOT 3

NetWare DOS Requester

FIRST NETWORK DRIVE = F

:

:

5)  A typical startnet.bat for ODI workstation is as follows:

SET NWLANGUAGE=ENGLISH  --Set NetWare 4.0 to English language

LSL                     --Link Support Layer Module provided by Novell

DE400ODI                --Hardware Specific Module provided by D-Link

IPXODI                  --IPX Protocol Stack Module provided by Novell

VLM                     --DOS Support Module provided by Novell

F:                      --Change to connected device

LOGIN .ADMIN.NV40SRV    --Login to file server NV40SRV as user ADMIN

6) You may run STARTNET to access the Novell network.