August 1992

LANsmart Driver Installation

for

D-Link DE-400 EISA Bus Master Ethernet Card

===============================================

This installation procedure applies to the D-Link DE-400 EISA Bus Master

Ethernet Card.

The following files will be used in this installation:

Files           Contents

-----------     --------------------------------------------------------

DE-400.DRV      LANsmart driver program for the DE-400 series card

DE-400.EXE      LANsmart driver program for the DE-400 series card

1. RUN LANsmart v.1.26 and v.1.30

==================================

Follow the procedure below to use the new LANsmart driver program on

this diskette with LANsmart v.1.26 and v.1.30.

a. Replace the LANsmart program LANSMA05.DRV with the new driver program.

Example:

> COPY A:\LANSMART\DE-400.DRV  C:\LANSMART\LANSMA05.DRV

b. Now, you can use both the menu mode and the command mode to log onto

the network.  The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 [other parameters]

2. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:DE-400 [other parameters]

The driver program DE-400.DRV should first be copied to the

directory that contains LOGON.EXE program. /NET:DE-400 specifies

to LANsmart your network adapter card.