SEPTEMBER 1993

SCO UNIX V.3 (TCP/IP V1.2X) Driver

Installation

for

DE-100/DE-200/DE-220/DE-400/DE-500 Ethernet Card

==================================================

****************************************************

* Installation for single-cards, for example DE500 *

****************************************************

1. Login as root user. (On maintenance mode)

2. Installation Diskette should be in DOS format.

Put your D-Link Installation Diskette to floppy A and use doscp command

to copy D-Link SCO UNIX Driver into UNIX's directories.

# cd /

# doscp drive:/scounix/setup500 /setup500	/* drive=A or B */

# chmod +x /setup500

# ./setup500  drive:/scounix

3. Configure network interface

# netconfig

then follow the instruction of netconfig displayed on screen

4. Reboot unix

# shutdown -y -g0

********************************

* Installation for multi-cards *

********************************

This installation is for DE-100/DE-200/DE-220/DE-400/DE-500 multi-cards drivers

of SCO UNIX (Runtime v3.2.4x) with TCP/IP v.1.2X.

1). It is allowed to install 4 NICs. The following is as an example:

DEX00 *1 + DEX00 *1 + DEX00 * 1 + DE220 *1 ; X = 1,2,3,4,5

DE100 *4 or

DE200 *4 or

DE220 *4 or

DE400 *4 or

DE500 *4

2). Support gateway function. ( routing function )

3). Statistic function is added when the following command is executed:

# nstatat -i

Name	 Mtu   Network	 Address   Ipkts  Ierrs Opkts	Oerrs Collis

d10e0  1500  192.9.200 unix2	   423	   0	 423	 0     0

d22e0  1500  15	 unix3	   423	   0	 423	 0     0

d40e0  1500  170.23	 unix1	   15322   0	 552	 0     0

d20e0  1500  168.8	 unix	   556	   0	 890	 0     0

lo0	 2048  170.23	 localhost 180	   0	 180	 0     0

*********************

* Install Procedure *

*********************

The following files will be used in this installation:

Files	       Contents

-----------    -------------------------------------------------

driver.o       DE-100/DE-200/DE-220/DE-400/DE-500 Driver Program

setup100       DE-100 software setup program

setup200       DE-200 software setup program

setup220       DE-220 software setup program

setup400       DE-400 software setup program

setup500       DE-500 software setup program

dlist	       Used by netconfig

dxxe	       Used by netconfig

info	       Used by netconfig

init	       Used by netconfig

master	       Used by netconfig

node	       Used by netconfig

reconf	       Used by netconfig

remove	       Used by netconfig

space.c        Used by netconfig

system	       Used by netconfig

1. Login as root user. (On maintenance mode)

2. Install TCP/IP software through the custom utility.

# custom

Select Install

Select A New Product

Select entire to install all of the packages

3.  Installation Diskette should be in DOS format.

Put your D-Link Installation Diskette to floppy A and use doscp command

to copy D-Link SCO UNIX Driver into UNIX's directories.

# cd /

# doscp A:/scounix/tcpip1.2x/setup100 /setup100

# doscp A:/scounix/tcpip1.2x/setup200 /setup200

# doscp A:/scounix/tcpip1.2x/setup220 /setup220

# doscp A:/scounix/tcpip1.2x/setup400 /setup400

# doscp A:/scounix/setup500 /setup500

# chmod +x /setup100

# chmod +x /setup200

# chmod +x /setup220

# chmod +x /setup400

# chmod +x /setup500

# ./setup100

# ./setup200

# ./setup220

# ./setup400

# ./setup500

4. Install D-Link SCO UNIX Driver

# mkdev d50e (d10e, d20e, d22e, d40e)

4.1) Do you wish to Add,Remove,or Modify the dlk driver?  type 'a' to add

4.2) Select IRQ value					   select a value

4.3) IRQ is default value '9'

or

Select a different interrupt vector.

4.4) Enter slot number

It depends on which slot the DE-500 card exist in.

4.5) Do you want to install another NIC (Y/N) ?	   select Y or N

Warning !!

If you use multiple the same type cards in this PC,

it is expected that you are repeating the process for several cards.

Max. NIC : 4 pcs

4.6) Do you want to relink the kernel? 		   type y to relink

5. Configure network interface

# netconfig

5.1) Do you wish to add/remove/reconfigure an element	   select add

in a chain:

5.2) Select top level of chain to Add or q to quit?	   select 1

5.3) Select next level of chain to Add or q to quit	   select DE-100/DE-200

/DE-220/DE-400

5.4) Add chain sco_tcp  -> d50e0 unit 0 (y/n)			 /DE-500

:

:

Add chain sco_tcp  -> d50e3 unit 3 (y/n)

It depends on card number.

5.5) IP address					   n.n.n.n

5.6) Confirm netmask					   enter key

5.7) Broadcast address of all 0's?                        [y/n]

5.8) Confirm broadcast address 			   enter key

5.9) Available options:				   enter key

1. Add a chain

2. remove a chain

3. reconfigure an element in a chain

q. Quit

6. Reboot unix

# shutdown -y -g0