FTP PC/TCP Installation               Sept. 1993

=======================

The D-Link Packet Driver allows you to run FTP's PC/TCP with

the D-Link DE-500 Ethernet Card.

The following files will be used in this installation:

Files         Contents

-----------   ---------------------------------

DE500PD.SYS   DE-500 Packet Driver

DE500PD.COM   DE-500 Packet Driver

1) You can include the DE500PD.SYS program in your CONFIG.SYS file:

DEVICE = DE500PD.SYS <packet_int_no> <int_level> <io_addr> <-d> <-w>

Or, you can execute the DE500PD.COM program from the DOS prompt:

>DE500PD <packet_int_no> <int_level> <io_addr> <-d> <-w>

2) Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

NOTES

=====

<packet_int_no> is the software interrupt vector number used by the packet

driver program, ranging from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 5 if this parameter is not

specified.

<io_addr>       is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first.

<-w>            the -w parameter is for the Windows environment only.