SEPTEMBER 1993

AT&T UNIX SVR4.0 DRIVER PROGRAMS

for

D-Link DE-500 VESA Bus Master Ethernet Card

===============================================

The following files will be used to in this installation:

Files	       Contents

-----------    ----------------------------------------------------------

de500.o        DE-500 series driver program

d50e

strcf

Installation and configuration consist of the following steps:

1.  Login as the root user.

2.  Installing TCP/IP software first.

3.  If the install diskette is DOS format

Copying D-Link driver through the doscp utility

#mkdir /usr/lib/tcprt/dlk

#doscp a:/attunix/svr4_0/de500.o  /usr/lib/tcprt/dlk/de500.o

#doscp a:/attunix/svr4_0/d50e  /usr/lib/tcprt/dlk/d50e

#doscp a:/attunix/svr4_0/strcf  /usr/lib/tcprt/dlk/strcf

#doscp a:/attunix/svr4_0/readme.doc  /usr/lib/tcprt/dlk/readme.doc

#chmod +x /usr/lib/tcprt/dlk/d50e

4.  Installing D-Link Ethernet driver

#cd /usr/lib/tcprt/dlk

#./d50e

Do you wish to install or delete the dlk driver?  type i to install

A display of information about driver.		  type y to continue

Select interrupt vector 			  value

Select I/O base address 			  hexadecimal value

Select memory base address			  hexadecimal value

Do you want relink the kernel?			  type y to relink

Do you want this kernel to boot by default?	  type y to default

Do you want the kernel environment rebuilt?	  type y to rebuilt

Do you want to modify /etc/strcf?		  type y

5.  Reboot unix

#cd /

#shutdown -y -g0 -i6