PC-NFS Driver Installation              Sept. 1993

==========================

The D-Link PC-NFS Driver Program allows you to run Sun MicroSystems PC-NFS

on your PC with the D-Link DE-500 Ethernet Card.

The following files will be used in this installation:

Files           Contents

-----------     ----------------------------------------------------------

PDNFS.SYS       PC-NFS Driver Program

DE500PD.SYS     DE-500 Packet Driver

DE500PD.COM     DE-500 Packet Driver

1) Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  During the PC-NFS installation, select the

3C501 driver (3COM EtherLink card) for your network card.

When prompted for the network card settings, select IRQ=5, and IO=300.

2) Copy the following files to your \NFS directory:

PDNFS.SYS

DE500PD.SYS

DE500PD.COM

3) Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\DE500PD.SYS <packet_int_no> <int_level> <io_addr> <-d> <-w>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

Modify the AUTOEXEC.BAT file as follows:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john          --the name "john" is used just as an example

NET USER F: \\user\john

4) Reboot your PC and start PC-NFS.  You may now execute the PC-NFS

commands to communicate with other computers through the TCP/IP protocol.

OPTIONAL METHOD

===============

As shown in Step 3 above, you can load the DE500PD.SYS program from the

CONFIG.SYS file.  Or, you can load the DE500PD.COM program from the

AUTOEXEC.BAT file as follows:

Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

Modify the AUTOEXEC.BAT file as follows:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

DE500PD <packet_int_no> <int_level> <io_addr> <-d> <-w>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

NOTES

=====

<packet_int_no> is the software interrupt vector number used by the packet

driver program, ranging from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 5 if this parameter is not

specified.

<io_addr>       is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first.

<-w>            the -w parameter is for the Windows environment only.