SEPTEMBER 1993

LANsmart Driver Installation

============================

The following file will be used in installing DE-500 card on a LANsmart

network:

Files           Contents

-----------     --------------------------------------------------------

DE-500.DRV      LANsmart driver program for the DE-500 series card

DE-500.EXE      LANsmart driver program for the DE-500 series card

RUN LANsmart v.1.26 and v.1.30

==============================

Follow the procedure below to use the new LANsmart driver program on this

diskette with LANsmart v.1.26 and v.1.30.

1. Replace the LANsmart program LANSMA05.DRV with the new driver program.

Example:

> COPY A:\LANSMART\DE-500.DRV  C:\LANSMART\LANSMA05.DRV

2. Now, you can use both the menu mode and the command mode to log onto

the network. Specify the "/NET:5 /IRQ:X /IOP:XXX" parameters when you

logon to LANsmart.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 /IRQ:5 /IOP:300 [other parameters]

2. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The driver program DE-500.DRV should first be copied to the

directory that contains LOGON.EXE program.

1)  Copy the DE500.DRV file to the LANsmart directory:

> COPY  A:\LANSMART\DE-500.EXE   C:\LANSMART

2)  Execute DE-500

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /IRQ:5 /IOP:300 [other parameters]

NOTES

=====

/NET    this parameter specifies the type of network card you are using

/IRQ    this parameter specifies the IRQ setting of the network card

/IOP    this parameter specifies the I/O address of the network card