NDIS Driver for DE-500 card                Sept. 1993

===========================

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory       Contents

---------       -----------------------------

\NDIS\DOS       NDIS driver for DOS

\NDIS\OS2       NDIS driver for OS/2

D-Link NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

D-Link NDIS driver program for DOS support the following NOS:

MS Windows for Workgroups v3.1

MS LAN Manager v2.1

DECnet PathWork v4.0

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.00(2)

IBM LAN Support Program v1.2.1

IBM Lan Server v3.0

*********************************************************

*   How to Install DE-500 on MS Windows for Workgroups  *

*********************************************************

If you are installing Windows for Workgroups for the first time, we advise

you to run the SETUP program with /I option (ignore hardware):  >SETUP /I

If you don't use the /I option, the computer may hang up during installation.

When this happens, reboot the machine, and run the SETUP program again.

If you have already installed WFW, you can go to the "Main => Control Panel =>

Network => Adapter" menu and answer the following questions.

1) When asked for the name of network card, select "Unlisted Network Card"

2) Insert the DE-500 Driver Disk in floppy drive 'A'

3) When asked for the path of VENDOR-PROVIDED driver file, type A:\NDIS

4) Select the "DE-500 VESA Ethernet" adapter

5) Select the proper IRQ and I/O settings for the DE-500 adapter

After WFW finishes the setup for DE-500, it will create the following files:

\CONFIG.SYS

===========

files=100

buffers=20

stack=9,256

device=c:\windows\protman.dos /i:c:\windows

device=c:\windows\workgrp.sys

device=c:\windows\de500.dos

\AUTOEXEC.BAT

=============

c:\windows\net start

\WINDOWS\PROTOCOL.INI

=====================

[network.setup]

version=0x3100

netcard=d$de500,1,D$DE500

transport=ms$netbeui,MS$NETBEUI

lana0=d$de500,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=D$DE500

LANABASE=0

[D$DE500]

DriverName=de500$

IOADDRESS=0x300

INTERRUPT=5

***********************************************

*   How to install DE-500 on MS LAN MANAGER   *

***********************************************

1)  Prepare a diskette with volume name "DOS_DRV_DLK" and create directories

in the following manner:

>label a: DOS_DRV_DLK     --insert a blank disk in drive 'A'

>md MSLANMAN.DOS          --make a directory called MSLANMAN.DOS

>cd MSLANMAN.DOS

>md DRIVERS               --make a directory called DRIVERS

>cd DRIVERS

>md ETHERNET              --make a directory called ETHERNET

>md NIF                   --make a directory called NIF

>cd ETHERNET

>md DE500                 --make a directory called DE500

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|                  |

ETHERNET            NIF

|

-------------

|           |

DE500

2) Insert the DE-500 Driver Program in drive 'A' and insert the

prepared diskette (DOS_DRV_DLK) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:                     --go to drive 'A'

>cd \NDIS\DOS

>copy DE500.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DE500

>copy PROTOCOL.INI  B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DE500

>copy DE500.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for DE-500 card.

-If you are installing LAN Manager for the first time, do the following:

Near the end of Setup program, you will be asked if you want to import

additional network drivers:  Answer "YES".

-If you have already installed LAN Manager, do the following:

Run the Setup program, go to the Configuration menu and select the

"Network Drivers" option.  Afterwards, choose the "Add New Config" option

and then choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_DLK) in drive 'A' and follow the

instructions on the screen.  The SETUP program will copy the necessary

NDIS driver programs from drive 'A' to your \MSLANMAN.DOS directory.

5) After completion of the SETUP, it will create the following files:

\CONFIG.SYS

===========

files=100

buffers=20

stack=9,256

device=c:\mslanman.dos\drivers\protman\protman.dos /i:c:\mslanman.dos

device=c:\mslanman.dos\drivers\ethernet\de500\de500.dos

\AUTOEXEC.BAT

=============

set path=c:\mslanman.dos\basic;%path%

c:\mslanman.dos\drivers\protocol\netbeui\netbeui.exe

netbind

net start workstation john

\MSLANMAN.DOS\PROTOCOL.INI

=====================

[PROTMAN]

Drivername = protman$

Dynamic = yes

[NETBEUI_XIF]

Drivername = Netbeui$

Bindings = "DE500_NIF"

[DE500_NIF]

Drivername = DE500$

Ioaddress = 0x300

Interrupt = 5

***************************************************

*  How to install DE500 on WIN/TCP PathWay Access *

***************************************************

1) Copy the DE500.DOS & PROTOCOL.INI files to C:\LANMAN directory:

>copy a:\NDIS\DOS\DE500.DOS  c:\LANMAN

>copy a:\NDIS\DOS\PROTOCOL.INI  c:\LANMAN

2) Go to the directory C:\LANMAN, and edit the PROTOCOL.INI as follows:

[DE500]

drivername = DE500$

ioaddress = 0x300

interrupt = 5

3) Add the following statement to your CONFIG.SYS file:

device=c:\lanman\DE500.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis

winbind

wintcp

************************************************

* How to install DE500 on DECnet PathWork v4.0 *

************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install D-Link NDIS

driver program. Please refer to DECnet installation manual for detailed

information. The following instructions assume that you have already installed

the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

from:                             to:

-----------------------------     ----------------------------

device = c:\decnet\ELNKII.SYS     device = c:\decnet\DE500.DOS

2) Modify the PROTOCOL.INI file as follows:

from:                             to:

------------------------------    ----------------------------

[ELNKMACII]                       [DE500]

drivername = ELNKII$              drivername = DE500$

ioaddress = 0x300                 ioaddress = 0x300

interrupt = 3                     interrupt = 5

xmitbufs = 2

[IPX4MAC]                          [IPX4MAC]

:                                  :

bindings = ELNKMACII               bindings = DE500

[DATALINK]                         [DATALINK]

:                                  :

bindings = ELNKMACII               bindings = DE500

****************************************

* How to install DE500 on 3Com 3+ Open *

****************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy DE500.DOS to  C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device = c:\3open\doswksta\lanman\drivers\DE500.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = DE500$

ioaddress = 0x300

interrupt = 5

:

*************************************************

* How to install DE500 on Banyan VINES v4.00(2) *

*************************************************

NOTE : D-Link NDIS driver program will support Banyan VINES v4.00(2) or above.

1. Create a bootable diskette (DOS 3.1 or above) which can be used as a boot

disk for Banyan VINES workstation.  Or, create a sub-directory on your

hard disk.  Afterwards, copy the necessary Banyan VINES workstation

files to the boot disk (or your hard disk).  There are two different

methods in doing this:

Method 1

--------

Execute the 'INSTALL' command from the diskette labeled "VINES MASTER DISK".

Follow its instructions and copy the necessary files to the boot disk (or

your hard disk).

Method 2

--------

Execute the 'BAN' command and Login to the VINES system.

Afterwards, execute the 'PCCOPY' command...Select the 'NDIS Ethernet' item

and follow its instructions and copy the necessary files to the boot disk

(or your hard disk).

2. Insert the DE-500 driver program diskette in drive 'A' and insert the

prepared boot disk in drive 'B'. Afterwards, copy the DE500.DOS file

from drive 'A' to 'B' (or your hard disk).

3. Create a CONFIG.SYS file which contains the following statements:

DEVICE=PROTMAN.DOS /I:A:\

DEVICE=DE500.DOS

NOTE:  The path "A:\" may be modified depending on the location

of the Banyan VINES workstation files.

4. Create a DE500.NIF file and include the following statements:

Model = D-Link DE-500 Ethernet Card

Path = A:\

DeviceDriver = DE500.DOS

DeviceName = DE500$

Type = NDIS

NOTE:  The path "A:\" may be modified depending on the location

of the Banyan VINES workstation files.

5. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "DE500"

[DE500]

drivername = DE500$

ioaddress = 0x300

interrupt = 5

6. Execute 'PCCONFIG' command

1) The "CONFIGURING A PC FOR THE NETWORK" is displayed on the screen

then select item '1 - Network Card Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

The "NDIS Ethernet" is displayed on the screen

then set your choice for D-LINK NIC

Ex. Interrupt Level (2-15) : 5

PROTOCOL.INI bindings = DE500

|

2) Return to Screen "CONFIGURING A PC FOR THE NETWORK"

then select item '2 - Login Environment Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

then 'PCCONFIG.DB' file will be generated.

7. Reboot your computer.

8. Login into VINES by executing the 'BAN' command.

**********************************************************

* How to install DE500 on IBM LAN Support Program v1.2.1 *

**********************************************************

Supposing that you have installed the IBM LAN Support Program in your computer,

you can use DE500.DOS to replace the installed driver program. Follow the

instructions below to change the contents of CONFIG.SYS and PROTOCOL.INI files.

1) Copy DE500.DOS to the directory where your IBM LAN Support program resides.

2) Modify the original NDIS device driver statement in CONFIG.SYS as follows:

DEVICE = [driver]:\path\DE500.DOS

3) Modify the PROTOCOL.INI file as follows:

BINDINGS = DE500

:

:

[DE500]

DRIVERNAME = DE500$

ioaddress = 0x300

interrupt = 5

***************************************************

* How to install DE-500.OS2 on IBM Lan Server 3.0 *

***************************************************

1.  Prepare an empty disk and insert it into drive 'A'.  Afterwards,

insert the D-Link Drive disk in drive 'B' and do the following:

For an OS2 machine, you need to use the OS2 NDIS driver.

> copy B:\NDIS\OS2\DE500.OS2     A:\

> copy B:\NDIS\OS2\DE500IBM.NIF  A:\DE500.NIF

OR, for a DOS machine, you need to use the DOS NDIS driver.

> copy B:\NDIS\DOS\DE500.DOS          A:\

> copy B:\NDIS\DOS\DE500IBM.NIF  A:\DE500.NIF

** The DE500IBM.NIF file must be renamed to DE500.NIF

2.  You must first install IBM OS/2 in your system:

Bootup with OS/2 Install Disk in drive 'A' and install OS/2

You must install the LAPs program before installing the LAN Server

or Requester program. Insert the "NTS/2 Adapter & Protocol Support Disk"

in drive B and run the LAPs program:

>LAPS  --choose "Install" and specify the target drive

choose "Install" again and insert your prepared

Driver disk in drive 'A'

choose "Configure"

click "D-Link DE-500 Ethernet Card" --> Add

click "IBM OS/2 NETBIOS"            --> Add

click "IEEE 802.2"                  --> Add

click OK  --> Exit  -->  Continue

Reboot your computer

Your CONFIG.SYS and PROTOCOL.INI files should reflect your card setup:

\CONFIG.SYS

===========

:

Device=d:\IBMCOM\MACS\DE500.DOS  or DE500.OS2

:

\IBMCOM\PROTOCOL.INI

====================

:

[DE500_NIF]

Drivername = de500$

3.  You can now install the LAN Server or Requester program.  For more

instruction, please refer to your IBM LAN Server "Planning and

Installation" manual.

==============================================================================

D-Link NDIS PROTOCOL.INI User Guide

This guide lists the available PROTOCOL.INI keywords for all D-Link NDIS

device drivers.  Most of the keywords are optional and have default values.

IOADDRESS - This keyword specifies the base I/O port address used by the

D-Link Network Interface Card. This value must match the

I/O address setting of your Network card.  The default value

for this keyword is 300h.

INTERRUPT - This keyword specifies the interrupt line used by the D-Link

Network Card. This value must match the IRQ jumper setting

on your NIC.  The default value for this keyword is 5.

DRIVERNAME- This keyword specifies the Network Card's Driver name.

The driver name follows the Network Card type:

i.e. DE100, DE200, DE220, DE500...etc.

PRINTPORT - This keyword specifies the printer port used by your

D-Link Pocket LAN Adapter.

** This keyword applies only to DE-600, DE-620

BASEMEMORY- This keyword specifies the mapped memory address used

by the D-Link Network Card.  This value must match the base

memory jumper setting on your NIC.  Base memory will

only be in effect if the remote boot ROM jumper is enabled.

ADAPTERID - This keyword specifies the adapter ID used by D-Link MCA bus

card. This value must match the card ID jumper setting on your

MCA adapter.

** This keyword applies only to DE-300, DE-320

NETADDRESS- This keyword specifies the node address (hex) used by your

network interface card.  This option is used only

if you want to override the node address burned in your NIC.

In this case, the specified address will be used instead of

the NIC's actual node address.

Following is an example of a PROTOCOL.INI file for DE-500 cards:

Drivername = DE500$

ioaddress = 0x300

interrupt = 5