How to Install NDIS 2.0.1 Drivers

===================================

NDIS (Network Driver Interface Specification) 2.0.1 is a driver

specification developed by Microsoft Corporation to offer a standard

driver interface for all adapter cards.  A standard driver interface

provides flexibility to support not only multiple network adapter

cards but also multiple protocol stacks.  As a result, many operating

systems have incorporated the NDIS interface to make configuration

and installation of network adapter cards easy and independent of

network adapter card.

Included in this manual are instructions to install the D-Link PCN family

NDIS 2.0.1 driver for the following environments:

Microsoft LAN Manager 2.x

Microsoft Windows for Workgroups 3.1

Microsoft Windows for Workgroups 3.11

Banyan VINES

IBM LAN Server 2.0, 3.0

Sun Microsystems PC-NFS 5.0

How to Installing the Windows for Workgroups 3.1 Driver

===========================================================

The PCN NDIS driver also supports Windows for Workgroups 3.1 environment.

The installation for the NDIS 2.0.1 driver is menu driven through the

Windows interface. The PCN driver and other relevant files are located

in the WFW31 subdirectory.

The directory structure provided on the D-Link driver diskette has been setup

for automatic OEM installation. The OEM Installation section in this

manual provides a brief explanation regarding OEM driver installation.

Follow the guidelines in the Microsoft Windows for Workgroups User's

Guide for more details to install the PCN family driver from the D-Link

driver diskettes.

Note:Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

To install the family Family NDIS 2.0.1 driver, follow the instructions

given below:

1. Insert the D-Link driver diskette in the system's floppy drive.

2. From the Main group in Windows, choose the Control Panel icon.

3. In the Network Settings dialog box, choose the Adapters button

and the Network Adapters dialog box should appear.

4. Choose the Add button to install the PCN driver from the D-Link

diskette.

5. In the list of network adapters, select the "Unlisted or Updated

Network Adapter" option.

6. Enter the drive letter the WFW31 path to load the PCN family

NDIS driver. For example, A:\WFW31.

7. Select the D-Link PCN family Driver and choose OK.

8. If the network adapter driver requires additional settings, a setup

dialog box appears displaying the default settings.

For DE-240CT, DE-510CAT, and DE-520CT\CAT adapter cards, no settings

need to be specified for one installed adapter.

9. Then, in the Network Adapters dialog box, choose the Close button.

Then choose the OK button in the Network Settings dialog box.

10.Follow the remaining Window instructions, to restart the computer

to have the driver to take effect.