How to Install NDIS 2.0.1 Drivers

===================================

NDIS (Network Driver Interface Specification) 2.0.1 is a driver

specification developed by Microsoft Corporation to offer a standard

driver interface for all adapter cards.  A standard driver interface

provides flexibility to support not only multiple network adapter

cards but also multiple protocol stacks.  As a result, many operating

systems have incorporated the NDIS interface to make configuration

and installation of network adapter cards easy and independent of

network adapter card.

Included in this manual are instructions to install the D-Link PCN family

NDIS 2.0.1 driver for the following environments:

Microsoft LAN Manager 2.x

Microsoft Windows for Workgroups 3.1

Microsoft Windows for Workgroups 3.11

Banyan VINES

IBM LAN Server 2.0, 3.0

Sun Microsystems PC-NFS 5.0

How to install the Microsoft LAN Manager NDIS 2.0.1 Driver

=============================================================

The hardware for the Microsoft LAN Manager environment may be configured

to run as an OS/2 server, OS/2 workstation, enhanced MS-DOS workstation,

or a basic MS-DOS workstation. Please refer to the Microsoft documentation

for hardware and software requirements.

The directory structure provided on the D-link driver diskette has been setup

for automatic OEM installation.  The OEM Installation section in this manual

provides a brief explanation regarding OEM driver installation.Follow the

guidelines in the Microsoft LAN Manager Installation Guide for more details

to install the PCN family driver from the D-Link driver diskettes.

Note: Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

LAN Manager has a SETUP program that functions to import and configure

drivers. The SETUP utility for LAN Manager will install the PCN NDIS

driver that is provided in the D-Link Driver Diskette. Setup requires that

two information files be present in order to install the drivers:NIF,

and PROTOCOL.INI.  These files are located in the MSLANMAN.DOS and

MSLANMAN.OS2 directories of the Driver Diskette.

The DEPCN.NIF file provides the information that the setup utility need to

coordinate and install the various drivers that are available. Information

that is contained in the two files include driver names, paths on the

diskette where the driver is stored, and the type of driver.

The PROTOCOL.INI file is used by the LANMAN Protocol manager and netbind

program at startup time. This file contains the NDIS and Transport specific

parameters for the network. Both the NDIS (PCN driver) and Transport

(NETBEUI) will have their own PROTOCOL.INI, and when setup installs the

NDIS and Transport, the two PROTOCOL.INI files will be merged into one

file.

To install the PCN family NDIS driver, follow the instructions given below:

1. Insert the D-Link driver diskette into the floppy drive.

2. From the LAN Manager subdirectory on your hard disk, run SETUP

to begin installation.

3. Use the ALT key on your keyboard and the highlighted letter to

pull down the Configuration menu (ALT + C).

4. From the Network Adapter Drivers dialog box, select the D-Link PCN

family driver from the menu, if shown.  Otherwise, select Other Driver

to import the PCN driver from the D-Link disk.

5. Then, enter the disk drive letter for the D-Link diskette. key in

path MSLANMAN.DOS subdirectory and choose OK.

6. Select the PCN family Ethernet adapter and choose OK.

7. Select the desired protocol from the Network Protocol dialog box

and choose OK.

8. In the Workstation Configuration dialog box, select the PCN

family Ethernet adapter and choose OK.

9. Follow the remaining instructions from LAN Manager to complete

installation.

How to Install the Banyan VINES Client Driver

==============================================

The PCN driver may configured to run in the Banyan VINES environment

for client based systems only.The steps to load the driver are as

follows:

1. Compile all the files necessary (see enclosed list) for VINES.

For ease of use, all of these files were placed in one

subdirectory - C:\NDIS

2. Create PROTOCOL.INI, CONFIG.SYS, and AUTOEXEC.BAT

(see below as an example)

a. Modify the PROTOCOL.INI to reflect the changes necessary

for the appropriate LAN card section.

b. Modify the AUTOEXEC.BAT and CONFIG.SYS to reflect changes

in the DIRECTORY of where the files are to be placed.

3. Run the VINES PCCONFIG to configure the NDIS Ethernet Driver

4. Reboot the machine and run the AUTOEXEC.BAT

5. Run BAN

;***************

;Files required:

;***************

BAN      EXE 32835 02-12-92 6:07p

BAN      MSB 2058  02-12-92 6:07p

REDIR    MSB 982   02-12-92 6:07p

NEWREV   EXE 47933 02-12-92 6:07p

NEWREV   MSB 5139  02-12-92 6:07p

PCCONFIG DB  19656 02-12-92 6:07p

REDIRALL EXE 51546 02-12-92 6:07p

PCCONFIG EXE 61155 02-12-92 6:07p

PCCONFIG MSB 5785  02-12-92 6:07p

NDISBAN  COM 35356 02-12-92 6:07p

NETBIND  EXE 15639 02-12-92 6:07p

NDISBAN  DOC 3072  02-12-92 6:07p

NDISBAN  OVL 15639 02-12-92 6:07p

PROTMAN  DOS 10649 02-12-92 6:07p

;*****************

;User Must Create:

;*****************

PROTOCOL.INI

CONFIG.SYS

AUTOEXEC.BAT

;***************

;User Must Find:

;***************

DEPCN.DOS

The following path should be used to access the driver from the D-Link diskette:

A:\NDIS\DOS\DEPCN.DOS

The modifications to CONFIG.SYS, AUTOEXEC.BAT, and PROTOCOL.INI are

shown below as an example for PCN family adapter cards:

;**********

;CONFIG.SYS

;**********

FILES=20

BUFFERS=20

DEVICE=C:\NDIS\PROTMAN.DOS  /I:C\NDIS

DEVICE=C:\NDIS\DEPCN.DOS

;************

;AUTOEXEC.BAT

;************

PATH=C:=DOS

PROMPT $P$G

C:

CD\NDIS

BAN

;************

;PROTOCOL.INI

;************

[PROTOCOL.INI]

DRIVENAME= PROTMAN$

;       PROTOCOL.INI section for the D-Link PCN family Card

[DEPCN_NIF]

DRIVENAME = DEPCN$

IOBASE = 0x300

INTERRUPT = 3

DMACHANNEL = 5

[VINES_XIF]

DRIVENAME = NDISBAN.DOS$

BINDINGS = DEPCN_NIF

How to Installing the Windows for Workgroups 3.1 Driver

===========================================================

The PCN NDIS driver also supports Windows for Workgroups 3.1 environment.

The installation for the NDIS 2.0.1 driver is menu driven through the

Windows interface. The PCN driver and other relevant files are located

in the WFW31 subdirectory.

The directory structure provided on the D-Link driver diskette has been setup

for automatic OEM installation. The OEM Installation section in this

manual provides a brief explanation regarding OEM driver installation.

Follow the guidelines in the Microsoft Windows for Workgroups User's

Guide for more details to install the PCN family driver from the D-Link

driver diskettes.

Note:Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

To install the family Family NDIS 2.0.1 driver, follow the instructions

given below:

1. Insert the D-Link driver diskette in the system's floppy drive.

2. From the Main group in Windows, choose the Control Panel icon.

3. In the Network Settings dialog box, choose the Adapters button

and the Network Adapters dialog box should appear.

4. Choose the Add button to install the PCN driver from the D-Link

diskette.

5. In the list of network adapters, select the "Unlisted or Updated

Network Adapter" option.

6. Enter the drive letter the WFW31 path to load the PCN family

NDIS driver. For example, A:\WFW31.

7. Select the D-Link PCN family Driver and choose OK.

8. If the network adapter driver requires additional settings, a setup

dialog box appears displaying the default settings.

For DE-240, DE-510, and DE-520 adapter cards, no settings

need to be specified for one installed adapter.

9. Then, in the Network Adapters dialog box, choose the Close button.

Then choose the OK button in the Network Settings dialog box.

10.Follow the remaining Window instructions, to restart the computer

to have the driver to take effect.

How to Installing the Windows for Workgroups 3.11 Driver

============================================================

Please refer to the Installing the Windows for Workgroups 3.11 Driver under

the Installing NDIS 3.0 Driver section in this manual.The procedure to load

NDIS 2.0.1 is exactly the same as for NDIS 3.0.In fact, Windows for

Workgroups 3.11 will automatically load NDIS 2.01, NDIS 3.0, both drivers

together.

Microsoft's Windows for Workgroups 3.11 supports both NDIS 2.0.1 and

NDIS 3.0 drivers.D-Link provides both NDIS drivers to run under

the Windows for Workgroups 3.11 environment.  Please note the

format and procedure to install the NDIS drivers for Windows for

Workgroups 3.11 is different from that of Windows for Workgroups 3.1.

The directory structure provided on the D-Link driver diskette

has been setup for automatic OEM installation. The OEM

Installation section in this manual provides a brief explanation

regarding OEM driver installation.  Follow the guidelines in the

Microsoft Windows for Workgroups 3.11 User's Guide for more details

to install the PCN family driver from the D-Link driver diskettes.

Note:Some of the custom installation procedures provided here

are either an abbreviated or a duplication of the OEM installation

guidelines.

To install the PCN NDIS 2.0.1 and/or NDIS 3.0 drivers, simply follow

the instructions given below:

1. Insert the D-Link driver diskette into the system's floppy drive.

2. In the Windows for Workgroups 3.11 Program Manager, double click

on the Windows Setup icon from the MAIN window.

3. While in the Windows Setup dialog box, select the Change Network

Settings from the Options menu to install PCN drivers from the

D-Link driver diskette.

4. From the Network Setup dialog box, select the "Drivers...

" button to install the PCN NDIS 2.0.1 and NDIS 3.0 drivers.

5. Then, in the Network Drivers dialog box, select Add Adapter

button to install the PCN device drivers.

6. Now, in the Add Network Adapter window, choose the "Unlisted

or Updated Network Adapter" from the menu and then select OK.

7. The Install driver dialog box will appear.  Enter the floppy

drive with the D-Link driver diskette and specify the WFW311 path

to install the PCN drivers.  For example,

A:\WFW311

Then select OK.

8. The Advanced Micro Devices PCN family selection will appear

under the Network Adapters list. Simply select OK to continue.

9. Then, the user will be asked a family of questions for the correct

Interrupt channel, DMA channel, Twisted Pair keyword option, and the

four LED keyword options.  Refer to the Software Keywords section in

this manual for more details.

For the other PCN family adapter boards (DE-240, DE-510, DE-520),

no values need to be specified for the Interrupt channel and

DMA channel since these PCN devices are software configurable.

10.Then, select OK to exit.  Windows for Workgroups 3.11 will

ask the user to reboot the machine once installation is completed.

The installation for Windows for Workgroups is now completed.