How to Install the Packet Driver

======================================

The D-Link Packet Driver for the PCN architecture (backwards compatible

to the Am2100/Am1500T) is based on the packet driver from Clarkson

University.  The Driver Diskette contains the executable code for

the DOS version of the Packet Driver.

Copy the D-Link Packet Driver, DEPCNPD.COM, in a directory. The packet

driver can be loaded from the DOS prompt or from the AUTOEXEC.BAT

file.The command line format for Packet Driver is:

DEPCNPD [-n] [-d] [-w] <INT=packet_int_no> [IRQ=int_no]                                [IOADDR=io_addr] [DMA=dma_no]

If the [IRQ=int_no], [IOADDR=io_addr], and [DMA=dma_no] are not

specified for the PCN adapter card, the default values are

assumed by the Packet Driver.

Additional common keywords may be specified.  Please refer to the

Software Keyword section for the available keywords.