How to Install NetWare Server 3.12 Driver

===========================================

For a NetWare 3.12 server configuration, the D-Link Server 3.12 driver

is included on the D-Link driver diskette. The driver is located under

the 3.12 subdirectory in the NETWARE\SRV directory of the structure D-Link

disk.

Use the LOAD and BIND commands to load the NetWare 3.12 Open Datalink

Interface (ODI) server driver as described below. For more information

on these commands, refer to the NetWare 3.12 LAN driver installation

documentation.	Located in the Driver Disk is the version 3.12 server

driver labeled NWDEPCN.LAN.  Two methods for loading this driver are

to use the server command line prompted by ":" or to edit AUTOEXEC.NCF.

If you use the server command line, you will need to load the driver

each time you edit AUTOEXEC, it will automatically load the driver each

time your system is booted.

To install the NetWare Server 3.12, follow the instructions given below:

1. Install NetWare 3.12 on your system.

2. Copy the ODI server driver, NWDEPCN.LAN, from the D-Link Driver

Diskette to the hard disk directory where the NetWare 3.12 system

files are located. The path on the D-Link Diskette should be as follows:

A:\NETWARE\SRV\3.12

3. Boot NetWare by executing SERVER.EXE. Refer to NetWare documentation

on loading disk drivers and mounting volumes.

4. To load the driver manually, simply follow the instructions given below.

For DE-240, DE-510, and DE-520 adapter boards, type the

following command at the file server console:

LOAD NWDEPCN   <enter>

( You will need to specify DOS path if the driver is located in a

DOS partition).

Do not use Port, INT, and DMA keywords. It is recommended that the

user use IOP, IRQ, and DRQ keywords instead. However, for the

DE-240CT, DE-510CAT, and DE-520CT/CAT adapter cards, no keywords need

to be specified for interrupt.

Make sure that MSM and ETHERTSM files are present in the same

directory as NWDEPCN. The NWDEPCN driver will autoload these files.

These files should be copied from the D-Link disk from the 3.12

subdirectory under the Netware directory.

Then, bind the driver to the IPX protocol stack by typing:

BIND IPX TO NWDEPCN NET=xxxx  <enter>

(Bind to ipx or any other protocol stack).

If two more adapter to install:

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

5.To load the driver using AUTOEXEC, you must edit AUTOEXEC.NCF. First,

type "LOAD INSTALL" at the server prompt ":"  to access the install

utility. Open the "SYSTEM OPTIONS" menu to access "EDIT AUTOEXEC".

Add the following command lines in AUTOEXEC.NCF file, for an example

on a PCnet-ISA card:

LOAD  NWDEPCN IRQ=3 IOP=320 FRAME=Ethernet_xx

( You will need to specify DOS path if the  driver is located in a

DOS partition).

Do not use Port, INT, and DMA keywords. It is recommended that the user

use IOP, IRQ, and DRQ keywords instead. However, for the DE-240CT,

DE-510CAT, and DE-520CT/CAT adapter cards, no keywords need to be specified

for interrupt channel, DMA channel, and I/O address.

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).