How to Install the NetWare DOS ODI Client Driver

===================================================

The PCN DOS ODI driver supports the client configuration for the

NetWare environment. The DOS ODI driver may be loaded using the

PCN family Configuration and Installation Utility.

To install the PCN family driver, follow the instructions given below:

You can load the NetWare PCN ODI driver through the DOS command line or

through a batch file such as AUTOEXEC.BAT.

1.Insert the D-Link Driver diskette into the system's floppy drive.

2.Create a subdirectory in your client's main boot drive with the

following files:

LSL.COM- Link Support Layer

DEPCNODI.COM- The PCnet DOS ODI Driver

The IPX protocol stack file (IPXODI.COM, for instance)

NETX.EXE- NetWare Shell (Used with conventional memory workstations)

NET.CFG- Optional; for configuring options different from

the default settings

NETX.EXE, LSL.COM, and IPXODI.COM can be obtained from the

Workstation Driver Diskette in the Novell Client Software Package.

Copy the client file DEPCNODI.COM from the DOS ODI directory of the

D-Link Driver Diskette to the subdirectory.  To do this, copy the

files through the path:

A:\NETWARE\WKS\ODI.DOS

3.To manually load these files from the DOS prompt each time the system

is restarted, type the following commands in given order to initialize

DOS ODI driver support and the IPX protocol stack:

LSL.COM <enter>

DEPCNODI.COM <enter>

IPXODI.COM <enter>

NETX.EXE  <enter>

NETX.EXE is compatible with the versions of DOS 3.x, 4.x, 5.x, and 6.x.

4.To automatically load these files,use a DOS text editor to create

and/or modify an AUTOEXEC.BAT file,then save it to the disk.

Load the files in the following order: link support layer,LAN Driver,

protocol stacks,then the shell.The AUTOEXEC.BAT file should look

similar to the following:

\<subdirectory>\LSL

\<subdirectory>\DEPCNODI

\<subdirectory>\IPXODI

\<subdirectory>\NETX

5.Reboot the workstation. After NETX is executed, and if a server is present,

it will attach to the server and display the server name on the monitor.

Logging in to the network ensures that the AUTOEXEC.BAT and CONFIG.SYS

files you created will connect you to the network. Logging in to the

server is as follows:

type  "f:"<enter> (assumes the network drive begins on the F drive)

at  "f:" prompt, type  "login"