How to Install NDIS 2.0.1 Drivers

===================================

NDIS (Network Driver Interface Specification) 2.0.1 is a driver

specification developed by Microsoft Corporation to offer a standard

driver interface for all adapter cards.  A standard driver interface

provides flexibility to support not only multiple network adapter

cards but also multiple protocol stacks.  As a result, many operating

systems have incorporated the NDIS interface to make configuration

and installation of network adapter cards easy and independent of

network adapter card.

Included in this manual are instructions to install the D-Link PCN family

NDIS 2.0.1 driver for the following environments:

Microsoft LAN Manager 2.x

Microsoft Windows for Workgroups 3.1

Microsoft Windows for Workgroups 3.11

Banyan VINES

IBM LAN Server 2.0, 3.0

Sun Microsystems PC-NFS 5.0

How to install the Microsoft LAN Manager NDIS 2.0.1 Driver

=============================================================

The hardware for the Microsoft LAN Manager environment may be configured

to run as an OS/2 server, OS/2 workstation, enhanced MS-DOS workstation,

or a basic MS-DOS workstation. Please refer to the Microsoft documentation

for hardware and software requirements.

The directory structure provided on the D-link driver diskette has been setup

for automatic OEM installation.  The OEM Installation section in this manual

provides a brief explanation regarding OEM driver installation.Follow the

guidelines in the Microsoft LAN Manager Installation Guide for more details

to install the PCN family driver from the D-Link driver diskettes.

Note: Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

LAN Manager has a SETUP program that functions to import and configure

drivers. The SETUP utility for LAN Manager will install the PCN NDIS

driver that is provided in the D-Link Driver Diskette. Setup requires that

two information files be present in order to install the drivers:NIF,

and PROTOCOL.INI.  These files are located in the MSLANMAN.DOS and

MSLANMAN.OS2 directories of the Driver Diskette.

The DEPCN.NIF file provides the information that the setup utility need to

coordinate and install the various drivers that are available. Information

that is contained in the two files include driver names, paths on the

diskette where the driver is stored, and the type of driver.

The PROTOCOL.INI file is used by the LANMAN Protocol manager and netbind

program at startup time. This file contains the NDIS and Transport specific

parameters for the network. Both the NDIS (PCN driver) and Transport

(NETBEUI) will have their own PROTOCOL.INI, and when setup installs the

NDIS and Transport, the two PROTOCOL.INI files will be merged into one

file.

To install the PCN family NDIS driver, follow the instructions given below:

1. Insert the D-Link driver diskette into the floppy drive.

2. From the LAN Manager subdirectory on your hard disk, run SETUP

to begin installation.

3. Use the ALT key on your keyboard and the highlighted letter to

pull down the Configuration menu (ALT + C).

4. From the Network Adapter Drivers dialog box, select the D-Link PCN

family driver from the menu, if shown.  Otherwise, select Other Driver

to import the PCN driver from the D-Link disk.

5. Then, enter the disk drive letter for the D-Link diskette. key in

path MSLANMAN.OS2 subdirectory and choose OK.

6. Select the PCN family Ethernet adapter and choose OK.

7. Select the desired protocol from the Network Protocol dialog box

and choose OK.

8. In the Workstation Configuration dialog box, select the PCN

family Ethernet adapter and choose OK.

9. Follow the remaining instructions from LAN Manager to complete

installation.

How to install IBM LAN Server Driver

======================================

The PCN driver may also be configured to run under the IBM LAN Server

environment. Although the IBM LAN Server software uses the same NDIS

driver as the Microsoft LAN Manager network operating system, the IBM

LAN Server software requires a unique NIF file is included in the D-Link

Driver diskette.

The directory structure provided on the D-Link driver diskette has been

setup for automatic OEM installation.The OEM Installation section

in this manual provides a brief explanation regarding OEM driver

installation.Follow the guidelines in the IBM LAN Adapter and Protocol

support Configuration Guide for more details to install the PCN

family driver from the D-Link driver diskettes.

Note: Some of the custom installation procedures provided here

are either an abbreviated or a duplication of the OEM installation

guidelines.

To install the PCN NDIS driver for IBM LAN Server, complete

the following steps:

1. Insert the floppy with D-Link files into the floppy drive.

2. In IBM LAN Server, select or run the LAPS (LAN Adapter and

Protocol Support) program.  Please refer to the IBM LAN Adapter

and Protocol Support Configuration Guide to access this utility and

for more details on how to install additional adapters and

device drivers.

3. From the IBM LAPS dialog box, choose the INSTALL button to

install a PCN Network adapter card for the IBM LAN Server

environment.If the PCN Adapter card has already been

INSTALLed, skip to step 6.

4. Then specify the drive and path (NDIS\OS2) to copy the appropriate

files from the D-Link driver diskette.  For example, please enter

A:\LANSRVR.

5. After successfully installing or copying the appropriate NIF files

from the D-Link disk.The IBM LAPS dialog box will appear again.

6. Now from the IBM LAPS dialog box, select the Configure option to

complete the installation of the NDIS driver for the PCN family

adapter card.

7. In the Configuration dialog box, choose the "Configure LAN

transports" (default) option and select Continue.

8. When the Configure Workstation dialog box is displayed, remove

the current configurations from the Current Configuration list box.

9. Choose the D-Link PCN family Ethernet Adapter from the Network

Adapters list box, by highlighting the entry. Then choose Add to add

the D-Link PCN Adapter card to the Current Configuration list box.

10. Highlight the appropriate protocols in the Protocols list box.

Then choose Add to add them to the Current Configuration list box.

11. Choose the D-Link PCN adapter at the Current Configuration list

box. Then choose Edit. The Parameters for D-Link dialog box is displayed.

12. View the default configuration. If the default configuration is

appropriate, choose OK.

If you need to change D-Link configuration, change the options in this

dialog box. Choose Range to display the available settings for each

option. Press [F1] if you need help deciding on a setting.

For DE-240, DE-510, and DE-520 adapter cards, no settings

need to be specified since they are software configurable.

You have finished installing the D-Link driver for IBM LAN Server 2.0.

To install the driver for IBM Extended Services 1.0, follow these steps:

1. Insert the floppy with D-Link files into the floppy drive.

2. Display the Communication Manager main menu and choose Advanced.

3. Choose Configuration.

4. Choose LAN Adapter and Protocol Support (LAPS).

5. At the Options dialog box, choose Copy additional network

adapter drivers. Then choose OK.

6. At the Copy Additional Network Adapter Drivers dialog box,

type the path (a:). Then choose OK.

7. When the Copy Diskette dialog box is displayed, choose OK.

8. When the Options dialog box is displayed, choose Configure Workstation. Then choose OK.

9. When the Configure Workstation dialog box is displayed, remove

the current configurations from the Current Configuration list box.

10. Choose the D-Link PCN family Ethernet Adapter from the Network

Adapters list box, by highlighting the entry. Then choose Add to

add the D-Link adapter to Current Configuration list box.

11. Highlight the appropriate protocols in the Protocols list

box. Then choose Add to add them to the Current Configuration list

box.

12. Choose the D-Link at the Current Configuration list box. Then

choose Edit. The Parameters for D-Link dialog box is displayed.

13. View the default configuration. If the default configuration

is appropriate, choose OK.

If you need to change D-Link's configuration, change the options in this

dialog box. Choose Range to display the available settings for each

option. Press [F1] if you need help deciding on a setting.

For DE-240, DE-510, and DE-520 adapter cards, no settings

need to be specified since they are software configurable.

You have finished installing the driver for IBM Extended Services 1.0.