How to Install NetWare Server 3.11 Driver

==========================================

For a NetWare 3.11 server configuration, the D-Link Server 3.11 driver is

included on the D-Link driver diskette. The driver is located under the 3.11

subdirectory in the NETWARE\SRV directory of the D-Link disk.

Use the LOAD and BIND commands to load the NetWare 3.11 open datalink

interface (ODI) server driver as described below.  For more information

on these commands, refer to the NetWare 3.11 LAN driver installation

documentation.	Located in the Driver Disk is the version 3.11 server driver

labeled NWDEPCN.LAN. Two methods for loading this driver are to use the

server command line prompted by ":" or to edit AUTOEXEC.NCF.

If you use the server command line, you will need to load the driver each time

the system is booted. If you edit AUTOEXEC.NCF, it will automatically load the

driver each time your system is booted.

To install the NetWare Server 3.11, follow the instructions given below:

1. Install NetWare 3.11 on your system.

2. Copy all the files from 3.11 server directory of the D-Link diskette

to the hard disk directory where NetWare 3.11 system files are located.

The following path should be used to access the files from the D-Link

diskette:

A:\NETWARE\SRV\3.11

3. Boot NetWare by executing SERVER.EXE. Refer to NetWare documentation

on loading disk drivers and mounting volumes.

4. To load the driver manually, simply follow the instructions given

below.

For DE-240CT, DE-510CAT, and DE-520CT/CAT adapter boards, type the

following command at the file server console:

( All files below needed for NetWare 3.11 and must use versions

included on D-Link disks)

LOAD LSLENH.NLM

LOAD MSM31X.NLM

LOAD ETHERTSM.NLM

LOAD NWDEPCN

LOAD MONITOR

<enter>

( You will need to specify DOS path if the  driver is located in

a DOS partition).

Do not use Port, INT, and DMA keywords.It is recommended that

the user use IOP, IRQ, and DRQ keywords instead.However, for the

DE-240CT, DE-510CAT, and DE-520CT/CAT adapter cards, no keywords need

to be specified for inter

Make sure that LSLENH, MSM31X, PATCHMAN, and ETHERTSM files are

present in the  same directory as NWDEPCN.These files should be copied

from the D-Link disk from the 3.11 subdirectory under the Netware

directory.

Then, bind the driver to the IPX protocol stack by typing:

BIND IPX TO NWDEPCN NET=xxxx  <enter>

(Bind to ipx or any other protocol stack).

NOTE: Please make sure that you use the monitor program which

is distributed  on the D-Link driver diskette.The original

monitor shipped with NetWare  3.11 has a bug and will

crash the system.

If two more adapter to install:

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

5. To load the driver using AUTOEXEC, you must edit AUTOEXEC.NCF. First,

type "LOAD INSTALL" at the server prompt ":"  to access the install

utility.  Open	the "SYSTEM OPTIONS" menu  to access "EDIT AUTOEXEC".

Add the following command lines

LOAD  NWDEPCN IRQ=3 IOP=320 FRAME=Ethernet_xx

( You will need to specify DOS path if the  driver is located in a

DOS partition).

Do not use Port, INT, and DMA keywords. It is recommended that

the user use IOP, IRQ, and DRQ keywords instead.However, for the

DE-240CT, DE-510CAT, and DE-520CT/CAT adapter cards,no keywords need

to be specified for inter

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).