How to Install NetWare Server 4.x Driver

==============================================

For a NetWare 4.x server configuration, the D-Link Server 4.x driver is

included on the D-Link driver diskette. The driver is located under the

4.x subdirectory in the NETWARE\SRV directory of the D-Link disk.

The directory structure provided on the D-Link driver diskette has been

setup for automatic OEM installation. The OEM Installation section in

this manual provides a brief explanation regarding OEM driver installation.

Follow the guidelines in the Novell Netware documentation for more details

to install the PCN family driver from the D-Link driver diskettes.

Note: Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

Use the LOAD and BIND commands to load the NetWare 4.x open datalink

interface (ODI) server driver as described below. For more information

on these commands, refer to the NetWare 4.x LAN driver installation

documentation.	Located in the Driver Disk is the version 4x server

driver labeled NWDEPCN.LAN.

To install the NetWare Server 4.x, follow the instructions given below:

1. Install NetWare 4.x on your system.

2. Copy the ODI server driver, NWDEPCN.LAN, from the D-Link Driver

Diskette to the hard disk directory where the NetWare 4.x

system files are located. The path on the D-Link Diskette should

be as follows:

A:\NETWARE\SRV\4.X

3. Boot NetWare by executing SERVER.EXE. Refer to NetWare documentation

on loading disk drivers and mounting volumes.

4. To load the driver manually, simply follow the instructions given

below.

For DE-240CT, DE-510CAT, and DE-520CT/CAT adapter boards, type the

following command at the file server console:

LOAD NWDEPCN	 <enter>

( You will need to specify DOS path if the driver is located in

a DOS partition).

Do not use Port, INT, and DMA keywords. It is recommended that

the user use IOP, IRQ, and DRQ keywords instead. However, for the

DE-240CT, DE-510CAT, and DE-520CT/CAT adapter cards, no keywords

need to be specified for interrupt.

Make sure that MSM and ETHERTSM files are present in the same

directory as NWDEPCN. The NWDEPCN driver will autoload these files.

These files should be copied from the D-Link disk from the

4.x subdirectory under the Netware directory

Then, bind the driver to the IPX protocol stack by typing:

BIND IPX TO NWDEPCN NET=xxxx	<enter>

(Bind to ipx or any other protocol stack).

If two more adapter to install:

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

5.   To load the driver using AUTOEXEC, you must edit AUTOEXEC.NCF.

First, type "LOAD INSTALL" at the server prompt ":"  to access

the install utility. Open the MAINTENANCE/SELECTIVE INSTALL to

select the "NCF File Options" from the menu to access the

"EDIT AUTOEXEC". Add the following command lines in AUTOEXEC.NCF

file, for example on a PCN family adapter card:

LOAD	NWDEPCN IOP=xxx IRQ=xxx FRAME=Ethernet_xx

( You will need to specify DOS path if the  driver is located

in a DOS partition).

Do not use Keyboard Port, INT, and DMA keywords.  It is recommended

that the user use IOP, IRQ, AND DRQ keywords instead.  However,

for the DE-240CT, DE-510CAT, and DE-520CT/CAT adapter cards, no

keywords need to be specified for interrupt channel, DMA channel,

and I/O address.

BIND IPX TO NWDEPCN NET=xxxx

(Bind to ipx or any other protocol stack).