NDIS Driver for DE-530CT card			 Jan. 1994

=============================

The NDIS driver in this diskette is organized as follows:

Directory			    Contents

-------------------------	    -----------------------------

\NDIS\DOS\DC21040.DOS		    NDIS 2.01 driver for DOS

\NDIS\OS2\DC21040.OS2		    NDIS 2.01 driver for DOS

\NDIS\IBMLAN.DOS\DC21040.DOS	    NDIS 2.01 driver for IBM LAN server v3.0

\NDIS\IBMLAN.OS2\DC21040.OS2	    NDIS 2.01 driver for IBM LAN server v3.0

D-Link NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

D-Link NDIS driver program for DOS support the following NOS:

MS Windows for Workgroups v3.1

MS Windows for Workgroups v3.11

MS LAN Manager v2.1

DECnet PathWork v4.0 or 4.1

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.00(2)

IBM LAN Support Program v1.2.1

Copyright 1994 Digital Equipment Corporation.  All Rights Reserved.

DEC, DECnet, and PATHWORKS are trademarks of Digital Equipment Corporation.

IBM and OS/2 are registered trademarks of IBM Corporation.

Microsoft, MS, and MS-DOS are registered trademarks, and Windows is a

trademark of Microsoft Corporation.

***************************************************************

*   How to Install DE-530CT on MS Windows for Workgroups 3.1  *

***************************************************************

If you are installing Windows for Workgroups for the first time, we advise

you to run the SETUP program with /I option (ignore hardware):	>SETUP /I

If you don't use the /I option, the computer may hang up during installation.

When this happens, reboot the machine, and run the SETUP program again.

If you have already installed WFW, you can go to the "Main => Control Panel =>

Network => Adapter" menu and answer the following questions.

1) When asked for the name of network card, select "Unlisted Network Card"

2) Insert the DE-530CT Driver Disk in drive 'A'

3) When asked for the path of VENDOR-PROVIDED driver file, type A:\NDIS

4) Select the "D-Link DE-530CT PCI Ethernet" adapter

5) You don't need to specify the IRQ and I/O settings, since they are

auto-selected by your PC BIOS.

After WFW finishes the setup for DE-530CT, it will create the following files:

\CONFIG.SYS

===========

files=100

buffers=20

stack=9,256

device=c:\windows\protman.dos /i:c:\windows

device=c:\windows\workgrp.sys

device=c:\windows\dc21040.dos

\AUTOEXEC.BAT

=============

c:\windows\net start

\WINDOWS\PROTOCOL.INI

=====================

[network.setup]

version=0x3100

netcard=dlk$de530,1,dlk$de530

transport=ms$netbeui,MS$NETBEUI

lana0=d$DC21040,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=dlk$de530

LANABASE=0

[dlk$de530]

DriverName=D21040$

SLOT=0

MAXTRANSMITS=4

MAXRECEIVES=4

****************************************************************

*   How to Install DE-530CT on MS Windows for Workgroups 3.11  *

****************************************************************

Inside Windows, please select the following icons:  Program Manager =>

Network  =>  Network Setup  =>	Drivers  =>  Add Adapter  =>

1) When asked for the name of network card, select "Unlisted Network Card"

2) Insert the DE-530CT Driver Disk in drive 'A'

3) When asked for the path of VENDOR-PROVIDED driver file, type A:\NDIS

4) Select the "D-Link DE-530CT PCI Ethernet" adapter

5) You don't need to specify the IRQ and I/O settings, since they are

auto-selected by your PC BIOS.

After WFW finishes the setup for DE-530CT, it will create the following files:

\CONFIG.SYS

===========

:

device=c:\windows\ifshlp.sys

\AUTOEXEC.BAT

=============

c:\windows\net start

\WINDOWS\PROTOCOL.INI

=====================

[network.setup]

version=0x3110

netcard=dlk$de530,1,dlk$de530,1

transport=ms$nwlinknb,NWLINK

transport=ms$ndishlp,MS$NDISHLP

transport=ms$netbeui,NETBEUI

lana0=dlk$de530,1,ms$netbeui

lana1=dlk$de530,1,ms$nwlinknb

lana2=dlk$de530,1,ms$ndishlp

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NDISHLP

[dlk$de530]

DriverName=D21040$

SLOT=0

MAXTRANSMITS=4

MAXRECEIVES=4

[NWLINK]

BINDINGS=dlk$de530

[MS$NDISHLP]

DriverName=ndishlp$

BINDINGS=dlk$de530

[NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=12

BINDINGS=dlk$de530

LANABASE=0

*************************************************

*   How to install DE-530CT on MS LAN MANAGER	*

*************************************************

1)  Prepare a diskette with volume name "DOS_DRV_DLK" and create directories

in the following manner:

>label a: DOS_DRV_DLK	  --insert a blank disk in drive 'A'

>md MSLANMAN.DOS	  --make a directory called MSLANMAN.DOS

>cd MSLANMAN.DOS

>md DRIVERS		  --make a directory called DRIVERS

>cd DRIVERS

>md ETHERNET		  --make a directory called ETHERNET

>md NIF 		  --make a directory called NIF

>cd ETHERNET

>md DC21040		  --make a directory called DC21040

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|		  |

ETHERNET		 NIF

|

-------------

|	     |

DC21040

2) Insert the DE-530 Driver Program in drive 'A' and insert the

prepared diskette (DOS_DRV_DLK) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:			--go to drive 'A'

>cd \NDIS\DOS

>copy DC21040.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DC21040

>copy PROTOCOL.INI    B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DC21040

>copy DC21040.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for DE-530 card.

-If you are installing LAN Manager for the first time, do the following:

Near the end of Setup program, you will be asked if you want to import

additional network drivers:  Answer "YES".

-If you have already installed LAN Manager, do the following:

Run the Setup program, go to the Configuration menu and select the

"Network Drivers" option.  Afterwards, choose the "Add New Config" option

and then choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_DLK) in drive 'A' and follow the

instructions on the screen.	The SETUP program will copy the necessary

NDIS driver programs from drive 'A' to your \MSLANMAN.DOS directory.

5) After completion of the SETUP, it will create the following files:

\CONFIG.SYS

===========

files=100

buffers=20

stack=9,256

device=c:\mslanman.dos\drivers\protman\protman.dos /i:c:\mslanman.dos

device=c:\mslanman.dos\drivers\ethernet\DC21040\DC21040.dos

\AUTOEXEC.BAT

=============

set path=c:\mslanman.dos\basic;%path%

c:\mslanman.dos\drivers\protocol\netbeui\netbeui.exe

netbind

net start workstation john

\MSLANMAN.DOS\PROTOCOL.INI

=====================

[PROTMAN]

Drivername = protman$

Dynamic = yes

[NETBEUI_XIF]

Drivername = Netbeui$

Bindings = "DC21040_NIF"

[DC21040_NIF]

Drivername = D21040$

******************************************************

*  How to install DE-530CT on WIN/TCP PathWay Access *

******************************************************

1) Copy the DC21040.DOS & PROTOCOL.INI files to C:\LANMAN directory:

>copy a:\NDIS\DOS\DC21040.DOS  c:\LANMAN

>copy a:\NDIS\DOS\PROTOCOL.INI	c:\LANMAN

2) Go to the directory C:\LANMAN, and edit the PROTOCOL.INI as follows:

[DC21040]

drivername = D21040$

3) Add the following statement to your CONFIG.SYS file:

device=c:\lanman\DC21040.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis

winbind

wintcp

**********************************************************

* How to install DE-530CT on DECnet PathWork v4.0 or 4.1 *

**********************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install D-Link NDIS

driver program. Please refer to DECnet installation manual for detailed

information. The following instructions assume that you have already installed

the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

from:				  to:

-----------------------------	  ------------------------------

device = c:\decnet\ELNKII.SYS	  device = c:\decnet\DC21040.DOS

2) Modify the PROTOCOL.INI file as follows:

from:				  to:

------------------------------	  ----------------------------

[ELNKMACII]			  [DC21040]

drivername = ELNKII$		  drivername = D21040$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]			   [IPX4MAC]

:				      :

bindings = ELNKMACII		   bindings = DC21040

[DATALINK]			   [DATALINK]

:				      :

bindings = ELNKMACII		   bindings = DC21040

*******************************************

* How to install DE-530CT on 3Com 3+ Open *

*******************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy DC21040.DOS to	C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device = c:\3open\doswksta\lanman\drivers\DC21040.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = D21040$

:

****************************************************

* How to install DE-530CT on Banyan VINES v4.00(2) *

****************************************************

NOTE : D-Link NDIS driver program will support Banyan VINES v4.00(2) or above.

1. Create a bootable diskette (DOS 3.1 or above) which can be used as a boot

disk for Banyan VINES workstation.  Or, create a sub-directory on your

hard disk.  Afterwards, copy the necessary Banyan VINES workstation

files to the boot disk (or your hard disk).	There are two different

methods in doing this:

Method 1

--------

Execute the 'INSTALL' command from the diskette labeled "VINES MASTER DISK".

Follow its instructions and copy the necessary files to the boot disk (or

your hard disk).

Method 2

--------

Execute the 'BAN' command and Login to the VINES system.

Afterwards, execute the 'PCCOPY' command...Select the 'NDIS Ethernet' item

and follow its instructions and copy the necessary files to the boot disk

(or your hard disk).

2. Insert the DE-530 driver program diskette in drive 'A' and insert the

prepared boot disk in drive 'B'. Afterwards, copy the DC21040.DOS file

from drive 'A' to 'B' (or your hard disk).

3. Create a CONFIG.SYS file which contains the following statements:

DEVICE=PROTMAN.DOS /I:A:\

DEVICE=DC21040.DOS

NOTE:  The path "A:\" may be modified depending on the location

of the Banyan VINES workstation files.

4. Create a DC21040.NIF file and include the following statements:

Model = D-Link DE-530 PCI Ethernet Card

Path = A:\

DeviceDriver = DC21040.DOS

DeviceName = D21040$

Type = NDIS

NOTE:  The path "A:\" may be modified depending on the location

of the Banyan VINES workstation files.

5. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "DC21040"

[DC21040]

drivername = D21040$

6. Execute 'PCCONFIG' command

1) The "CONFIGURING A PC FOR THE NETWORK" is displayed on the screen

then select item '1 - Network Card Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

The "NDIS Ethernet" is displayed on the screen

then set your choice for D-LINK NIC

Ex. Interrupt Level (2-15) : 5

PROTOCOL.INI bindings = DC21040

|

2) Return to Screen "CONFIGURING A PC FOR THE NETWORK"

then select item '2 - Login Environment Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

then 'PCCONFIG.DB' file will be generated.

7. Reboot your computer.

8. Login into VINES by executing the 'BAN' command.

*************************************************************

* How to install DE-530CT on IBM LAN Support Program v1.2.1 *

*************************************************************

Supposing that you have installed the IBM LAN Support Program in your computer,

you can use DC21040.DOS to replace the installed driver program. Follow the

instructions below to change the contents of CONFIG.SYS and PROTOCOL.INI files.

1)Copy DC21040.DOS to the directory where your IBM LAN Support program resides.

2)Modify the original NDIS device driver statement in CONFIG.SYS as follows:

DEVICE = [driver]:\path\DC21040.DOS

3) Modify the PROTOCOL.INI file as follows:

BINDINGS = DC21040

:

:

[DC21040]

DRIVERNAME = D21040$

********************************************************

* How to install DE-530CT on IBM OS/2 2.0 Lan Server *

********************************************************

1. Assuming your OS/2 2.0 is properly installed and working, proceed to

install LAN SERVER.	At this point in time, no D-Link network adapters

must be installed in the computer.

2. Install LAN SERVER according to IBM installation procedures.  At the

point where LAN SERVER asks for LAN Adapter, select any of the adapters

supported by LAN SERVER.  For example, we select 'Western Digital

EtherCard PLUS ...'.  After LAN Adapter selection, continue installing

LAN SERVER.

3. After LAN SERVER is successfully installed, shut down OS/2 for system

econfiguration.  Do not insert any D-Link network adapters.	The OS/2

system will boot up bringing up LAN SERVER services. You will encounter

errors with LAN SERVER because the network adapter you selected is not

installed.  Ignore the errors and continue with OS/2 booting.

4. Assuming you are using D-Link DE-530CT card, insert the DE-530CT driver disk

in drive A: and copy the following files,

COPY A:\NDIS\OS2\DC21040.OS2 C:\IBMCOM\MACS

COPY A:\NDIS\OS2\DC21040.NIF C:\IBMCOM\MACS

At the OS/2 Desktop, run OS/2 system editor.  You have to edit two files

namely,

C:\CONFIG.SYS

C:\IBMCOM\PROTOCOL.INI

Editing CONFIG.SYS

------------------

(1) Look for the following line

DEVICE=C:\IBMCOM\MACS\MACWD.OS2

and change it to,

DEVICE=C:\IBMCOM\MACS\DC21040.OS2

Editing PROTOCOL.INI

--------------------

(1) In the section [IBMLXCFG], add the following line

DE220_nif = DC21040.nif

(2) In the section [NETBEUI_nif], look for the line

Bindings = MACWDMC_nif

and change it to,

Bindings = DC21040_nif

(3) In the MAC SECTION definitions, add the following section

[DC21040_nif]

DriverName = D21040$

5. After modifying the two files above, shutdown your OS/2 system and

turn the computer off.  Install the D-Link DE-530CT card using the same

INTERRUPT and IOADDRESS specified in the PROTOCOL.INI file.	Turn on

your computer to start OS/2.

Upon OS/2 startup, you should notice that LAN SERVER services will now

successfully start without any errors.

**************************************************

* How to install DE-530CT  on IBM LAN SERVER 3.0 *

**************************************************

1. Prepare an empty disk and insert it into drive 'A'.  Afterwards,

insert the D-Link Drive disk in drive 'B' and do the following:

For an OS2 machine, you need to use the OS2 NDIS driver.

> copy B:\NDIS\IBMLAN.OS2\DC21040.OS2	 A:\

> copy B:\NDIS\IBMLAN.OS2\DC21040I.NIF  A:\DC21040.NIF

OR, for a DOS machine, you need to use the DOS NDIS driver.

> copy B:\NDIS\IBMLAN.DOS\DC21040.DOS	      A:\

> copy B:\NDIS\IBMLAN.DOS\DC21040I.NIF  A:\DC21040.NIF

** The DC21040I.NIF file must be renamed to DC21040.NIF

2. You must first install IBM OS/2 in your system:

Bootup with OS/2 Install Disk in drive 'A' and install OS/2

You must install the LAPs program before installing the LAN Server

or Requester program. Insert the "NTS/2 Adapter & Protocol Support Disk"

in drive B and run the LAPs program:

>LAPS  --choose "Install" and specify the target drive

choose "Install" again and insert your prepared

Driver disk in drive 'A'

choose "Configure"

click "D-Link DE-530CT PCI Ethernet adapter" --> Add

click "IBM OS/2 NETBIOS"            --> Add

click "IEEE 802.2"                  --> Add

click OK  --> Exit  -->  Continue

Reboot your computer

Your CONFIG.SYS and PROTOCOL.INI files should reflect your card setup:

\CONFIG.SYS

===========

:

Device=d:\IBMCOM\MACS\Dc21040.DOS  or DC21040.OS2

:

\IBMCOM\PROTOCOL.INI

====================

:

[DC21040_NIF]

Drivername = d21040$

3. You can now install the LAN Server or Requester program.  For more

instruction, please refer to your IBM LAN Server "Planning and

Installation" manual.

***************************************************************

* How to install DE-530CT  on IBM Communication Manager v 1.0 *

***************************************************************

Supposing that you have installed the IBM Communication Manager in your

computer.

1. Copy both DC21040.OS2 and DC21040.NIF from the \NDIS\OS2 subdirectory of

the shipping driver diskette to C:\IBMCOM\MACS directory.

2. Execute CM's install program, select D-Link DE-530CTadapter and select

protocols.

3. Make sure "DEVICE = C:\IBMCOM\MACS\DC21040.OS2" statement is in the

CONFIG.SYS file.

4. Shutdown your OS/2 system, then reboot your PC.

==============================================================================

D-Link NDIS PROTOCOL.INI User Guide

This guide lists the available PROTOCOL.INI keywords for all D-Link NDIS

device drivers.  Most of the keywords are optional and have default values.

IOADDRESS - This keyword specifies the base I/O port address used by the

D-Link Network Interface Card.	Since DE530 can auto-detect

this address, you do not need to specify it.

IRQ       - This keyword specifies the interrupt line used by the D-Link

Network Card (range:5,9,10,11,0). Since DE530 can auto-detect

this address, you do not need to specify it.

DRIVERNAME- This keyword specifies the Network Card's Driver (8 char max).

The driver name follows the Network Card type:

i.e. DE100, DE200, DE220, DE500, DC21040$

NETADDRESS- This keyword specifies the node address (hex) used by your

network interface card.  This option is used only

if you want to override the node address burned in your NIC.

In this case, the specified address will be used instead of

the NIC's actual node address.

MaxTransmits-Specifies the number of Transmit requests that the Driver

guarantees to accept without giving the OUT_OF_RESOURCE

response (ranges from 1 thru 20).  Default is set to 4.

MaxReceives-Specifies the maximum number of full frames that could be

acquired and stored in the DC21040 receive list (ranges from 1

thru 20).  Default is set to 4.

Slot      - Specifies where the DE-530CT adapter is placed.

o A number from 16 to 26 specifies the relative position of an

adapter on the PCI bus. 16 specifies the first DE-530CT

adapter on the PCI bus, 17 the second such adapter, & so on.

Range: 0 thru 26.  Default is set to 0.

Sia_Mode  - Defines the Network Cable Type.

o AUI selects the AUI connector for external transceiver or

the BNC connector for the on-board transceiver. The selection

between the two connectors is done via a set of hardware

jumpers on the adapter board.

o TP selects the RJ45 connector for the Twisted Pair (10BaseT)

on-board transceiver, for connection to a Twisted Pair hub.

o TP_FULL_DUPLEX specifies Full Duplex operation via the

Twisted Pair transceiver. The adapter should be connected to

another Ethernet station with a cable suitable for

Full Duplex mode.

Following is an example of a PROTOCOL.INI file for DE-530 cards:

Drivername = D21040$