NDIS 2 MAC DRIVER for DE-530/DFE-500  V2.00

****************************************************

Note that this installation kit includes both the DOS

and OS/2 NDIS2 driver and this document describes

installation under both environments.  This revision

supersedes the DOS and OS/2 NDIS2 driver version 1.03 .

NOTE: The generic term "DC21X4" relates to the DECchip

21040, DECchip 21041 and DECchip 21140 Ethernet and

Fast Ethernet LAN controllers.

.  Installing the NDIS2 DOS driver.

. Installation with Microsoft LAN Manager 2.x

.  Automatic installation using the LAN Manager Setup Program:

1. Start the LMSETUP program.

2. Select <Configuration> from the Main Menu.

3. Select <Network Driver> from the Configuration Menu.

4. Remove the old adapter using <Remove Configuration>.

5. Select <Add Configuration>.

6. Select <Other Drivers> and follow the directions.

.  Manual Installation for DOS LAN Manager

If LAN Manager is already installed and configured and you do

not wish to use the installation program, you have to perform

the following steps manually:

1. Create DC21X4 Driver directory

<DRIVE>:\LANMAN.DOS\DRIVERS\ETHERNET\DC21X4

2. Copy the files

\MSLANMAN.DOS\DRIVERS\ETHERNET\DC21X4\DC21X4.DOS

and

\MSLANMAN.DOS\DRIVERS\ETHERNET\DC21X4\PROTOCOL.INI

to the new directory.

3. Copy the file

\MSLANMAN.DOS\DRIVERS\ETHERNET\DC21X4\DC21X4.NIF

to the directory

<DRIVE>:\LANMAN.DOS\DRIVERS\NIF

4. Add the following line to your CONFIG.SYS to load the

driver:

DEVICE=<DRIVE>:\LANMAN.DOS\DRIVERS\ETHERNET\DC21X4\DC21X4.DOS

5. Modify your PROTOCOL.INI file using any DOS Editor by

appending the PROTOCOL.INI stub (included with this kit) to

the end of your old PROTOCOL.INI file.  The stub contains

the following information:

[DC21X4_NIF]

DRIVERNAME=DC21X4$

6. Modify TCPIP_XIF and NETBEUI_XIF modules in the PROTOCOL.INI

file by changing the value of the BINDINGS keyword

from  BINDINGS = driverlabel

to    BINDINGS = DC21X4_NIF