Windows NT Driver Program Installation            Jan. 1994

======================================

Copyright Notice:  First Edition, April 1994

Copyright Digital Equipment Corporation 1994.

All rights reserved.

Do not redistribute the driver files.

Intel is a trademark of Intel Corporation.

Windows, Windows NT, and NT are trademarks of Microsoft

Corporation.

Microsoft, MS, MS-DOS are registered trademarks of Microsoft

Corporation.

Alpha AXP, AXP, DEC, DECchip, DECnet, Digital, PATHWORKS,

ThinWire, and the DIGITAL logo, are  trademarks of Digital

Equipment Corporation.

All other trademarks and registered trademarks are the property

of their respective holders.

1. Run Windows NT and go to Main -->  Control Panel -->  Network  -->  Adapter

2. Select the [Add_Adapter]  option

3. Select the last choice from the Add Network Adapter box:

<Other>  Requires disk from the manufacturer

4. Insert the DE-530CT Driver Disk in drive 'A' and set the path to 'A:\NT'.

5. Select your option from the Select OEM Option box:

[D-Link DE-530CT PCI Ethernet Controller]

Select [OK] and then select [CONTINUE].

6. A window will appear allowing you to specify your network connection type.

Choose the AUTOSENSE option.

AutoSense           = Auto detects UTP, BNC or AUI connectors

UTP                 = Twisted Pair.

UTP - Full Duplex   = Twisted Pair and Full Duplex.

UTP - Link Disabled = Twisted Pair and Link Integrity test disabled

AUI BNC             = AUI 10Base2

AUI 15_pin          = AUI 10Base5

Since this driver initializes using the base I/O address and interrupt

vector set by the PCI BIOS, you do not need to specify the Platform type,

Slot number, or Interrupt number (IRQ).

7. Select [OK] and your NT driver files will be copied to your HD.

Upon completion,

[ [n]D-Link DE-530CT PCI Ethernet Controller]

will appear in the Installed Adapter Cards list of the

Network Setting box (n is the adapter number).

9. Click the [OK] button to complete the Network Setting initialization.

10. Invoke the Shutdown command and reboot your system